Response Letter for Manuscript (JOLT-D-24-01236) submitted to Optics and Laser Technology

Design and On-Orbit Performance Evaluation of Ethiopian Earth Observation Satellite Multispectral Optical Imaging Payload

The authors are thankful to the Reviewers and Editors for feedback and consideration for the manuscript.

Suggestions have been incorporated in the revised manuscript, where changes are highlighted in blue text

Please find our responses to all comments below.

Comments and responses

| Reviewer #1: |  |
| --- | --- |
| Reviewer general comments | * The authors revised the manuscript in accordance with the reviewer comments. Generally, responses to comments are fulfilling but during the revision process some issues emerged, which are listed below as comments. Other than these the manuscript is good state. |
| Reviewer comments | * A statement should be added to end of the ground resolution formula, as this formula is valid according to a design criterion. |
| Response | On page 4, “For a satellite at altitude, 𝐑, the linear resolution or ground resolution, Gr (diameter on the ground corresponding to point spread function, 𝐚, on the focus plane) at nadir is Gr = 2.44( Rλ/D). The ground resolution (Gr) of an earth observation satellite’s optical imaging payload system is often correlated with the diffraction-limited resolution of an optical imaging payload, which is the lowest discernible detail on the Earth’s surface that the system can capture. It is determined by the system’s optical design, which includes focal length, aperture size, detector pixel size, and satellite altitude. Further, it is proportional to the spacecraft's orbital altitude and spectral wavelength, and inversely proportional to the system's aperture diameter.” |
| Reviewer comments | SNR and MTF were greater than 100% -> SNR and MTF were greater than 100 |
| Response | On page 2, “The SNR and MTF of the payload push-broom scanner type flying at 700 km altitude exceeded 100 and 40, respectively, according to their findings.” |
| Reviewer comments | which are significant largely as a manufacturing -> which are significantly large as manufacturing |
| Response | On page 3, “These requirements include brightness sensitivity, brightness range, and calibrated sensor accuracy, which are significantly large as a manufacturing and testing constraints, but the first two are what drive the initial design or selection of an existing sensor for an application.” |
| Reviewer comments | An optical imaging payload performance evaluation categories -> Optical imaging payload performance evaluation categories |
| Response | On page 3, “Optical imaging payload performance evaluation categories include physical performance constraints such as spatial, spectral, radiometric, and temporal resolution, and operational constraints such as sensor duty cycle limits, tasking and scheduling limits [5].” |
| Reviewer comments | aberrations, modulation transfer faction -> aberrations, modulation transfer function |
| Response | On page 3, “Furthermore, the performance of an optical imaging payload system was determined by a variety of optical characteristics, including aperture size, field of view, optical aberrations, modulation transfer function, and others, all of which influence the quality and accuracy of the acquired images.” |
| Reviewer comments | - Table 2: 26.7dB -> 26.7 - Table 2: 48.5dB -> 48.5 |
| Response | The unit indicating SNR in Table #2 has been removed, and the manuscript has been modified. |