Hyperspectral Imager Design

The main aim of this study is to design a compact Schmidt–Cassegrain hyperspectral imager based on the classic Dyson spectrometer a 50mm Entrance Pupil Diameter (EPD) and an effective focal length of 300 mm, providing a 3-degree field of view within the wavelength range of 486–900 nm. In optical design, it is essential to match the resolution of the system with the size of the detector, which is governed by the Airy disk [1]. For optimal performance, the appropriate F-number must be selected, which in this case is 6. According to the Nyquist sampling criterion used in the design of astronomical instruments, such as cameras and spectrometers, the diameter of the Airy disk should span four pixels [2]. The imager’s resolution is determined by both the detector size 4p, where p represents the pixel size and the spot diameter size 2.44 Fλ, where F is the F-number and λ is the wavelength [1]. Given the selected detector pixel size to be 1.8μm, first order optical specifications of the hyperspectral imager are calculated and outlined in the table below.

| Parameter | Value | Unit |
| --- | --- | --- |
| Spectral Range | 486-900 | nm |
| GSD | 3 | m |
| Field of View |  |  |
| F-number | 6 |  |
| Entrance Pupil Diameter | 50 | mm |
| Overall Length | 132 | mm |
| Effective Focal Length | 300 | mm |

Imager Design

The telescope is a standard two-mirror Cassegrain telescope design, augmented with four weak lenses inspired by [3]. According to [4], the two front meniscuses were placed compensate for spherical aberrations, while the two lenses near the slit plain addresses any minor residual aberrations. This design provides near diffraction-limited performance across all wavelengths and fields, as demonstrated by the spot diagram and the modulation transfer function (MTF) plot in [3]. Light enters the telescope thought the 50mm entrance pupil and passes through the front meniscus lenses, and straight to the primary concave mirror. It then reflects onto the secondary mirror where it is sent to the back of the telescope to pass through the rare lenses to the slit. This type of design is commonly used in imaging spectrometers, such as the Airborne Visible/Infrared Imaging Spectrometer (AVIRIS) [1].

Spectrometer Design

For the spectrometer, we selected the classis Dyson spectrometer. It is renowned for its compact design and efficient throughput. When compared to the PRISM spectrometer, it has the same nominal slit size in terms of cross-track elements but offers a coarser spectral resolution (5.7 nm compared to 2.9 nm), a broader spectral range (350–1700 nm versus 1050 nm), and a 10% larger pixel size (30 mm versus 27 mm) [5]. Despite being notably smaller than the PRISM spectrometer, this size reduction is achieved by aligning the slit symmetrically with the spectrometer's plain of symmetry which we tried to achieve in our design. The design generally comprises of the Dyson block and a concave spectrometer. The design of both the spectrometer and the telescope utilizes fused-silica material which are both radiations hardened and susceptible to thermal expansion as indicated by [3].

- What are the research Gap:

"Hyperspectral Imager Payload Design and Performance Optimization Analysis for Low-Earth Orbit (LEO) Small Satellites"

While significant progress has been made in developing hyperspectral imaging systems and deploying them on small satellite platforms, several key research gaps remain that limit the full exploitation of their potential in Earth observation missions:

1. Limited Multi-Objective Optimization Frameworks------->focus area

Most existing studies focus on optimizing single parameters (e.g., spatial or spectral resolution), without addressing the complex trade-offs between Size, Weight, and Power (SWaP) constraints, imaging performance (SNR, spatial/spectral resolution), data handling, and mission requirements in an integrated, multi-objective optimization framework.

2. Lack of End-to-End Co-Design Approaches

There is a gap in co-design methodologies that simultaneously consider optical design, detector selection, data processing constraints, orbital parameters, and platform limitations. Most works optimize subsystems in isolation, leading to suboptimal system-level performance.

3. Insufficient Adaptation for CubeSat and SmallSat Constraints

Many hyperspectral imager designs are still derived from traditional satellite architectures and do not fully exploit or adapt to the unique mechanical, thermal, and electrical constraints of CubeSats and small satellites in LEO.

4. Performance Degradation Modeling in Real LEO Conditions

There is a lack of comprehensive modeling and experimental validation for in-orbit performance degradation, including thermal drift, radiation effects, and attitude instability, which can significantly affect the hyperspectral payload’s imaging quality over time.

5. Data Management and Downlink Bottlenecks

Despite advances in onboard hyperspectral imaging, efficient solutions for data compression, storage, and downlink optimization tailored to high-volume hyperspectral data in bandwidth-constrained CubeSat missions are still underdeveloped.

6. Lack of Standardized Design and Evaluation Benchmarks

The field lacks standardized benchmarks or validated simulation frameworks to evaluate and compare design trade-offs and performance of hyperspectral imagers across different small satellite missions.

7. Limited Focus on Application-Driven Payload Optimization

Most optimization efforts are generic and do not tailor payload design to specific application needs (e.g., vegetation monitoring vs. mineral mapping), despite differing requirements in spectral range, revisit time, and resolution.

In Summary:

There is a clear need for an integrated, application-aware, and multi-objective optimization framework that considers all critical design parameters and constraints of hyperspectral imagers on LEO small satellites. Addressing this gap will enable more efficient, capable, and mission-specific Earth observation systems.

- What is the research gap in the research topic "Hyperspectral Imager Payload Design and Performance Optimization Analysis for Low-Earth Orbit Small Satellites" and how to connect the topic using hyperspectral image processing MATLAB toolbox, and what is my contribution in the research?

Here's a breakdown of the research gap, integration with the Hyperspectral Image Processing MATLAB toolbox, and your contribution to the topic "Hyperspectral Imager Payload Design and Performance Optimization Analysis for Low-Earth Orbit (LEO) Small Satellites":

🔍 Research Gap

Despite the growing interest in deploying hyperspectral imaging (HSI) systems on CubeSats and small satellites, the following key gaps remain:

- Lack of Integrated Multi-Objective Optimization Framework---->focus area
Most current payload designs consider single-objective or constraint-driven approaches, lacking comprehensive frameworks that simultaneously optimize multiple conflicting parameters (e.g., spatial resolution, spectral fidelity, SNR, SWaP constraints).

- Insufficient Payload-to-Mission Context Coupling
Designs often neglect system-level coupling between orbital dynamics, scene characteristics, and payload performance (e.g., effect of orbit on SNR, revisit time, or spectral band utility for specific Earth observation tasks).

- Limited Use of End-to-End Simulation and Validation
Many studies stop at optical design, with limited validation using simulated or real hyperspectral data to verify performance under expected mission scenarios.

- Inadequate Use of Image Processing Feedback in Payload Design
Hyperspectral image quality metrics (e.g., classification accuracy, vegetation indices, or detection rates) are rarely fed back into the design loop to inform trade-offs or guide optimization.

🧩 Connection with MATLAB Hyperspectral Image Processing Toolbox

The Hyperspectral Image Processing Toolbox (HIPT) in MATLAB (or similar toolboxes such as ENVI or custom scripts) can be used to bridge design and performance evaluation as follows:

- Simulated Image Generation & Preprocessing----->focus area

  - Generate synthetic hyperspectral data cubes using modeled payload parameters (FOV, resolution, SNR).

  - Apply realistic noise models (e.g., shot noise, thermal noise) based on your optical and detector design.

- Performance Evaluation Metrics----->focus area

  - Use the toolbox for spectral unmixing, classification (e.g., SVM, PCA, K-means), and endmember extraction to assess the usefulness of your spectral configuration.

  - Evaluate spatial and spectral performance in terms of task-specific metrics (e.g., crop type classification accuracy, NDVI precision).

- Design Feedback Loop

  - Integrate results (e.g., lower classification accuracy due to band overlap or insufficient SNR) back into the payload design optimization to adjust system parameters.

  - Perform sensitivity analyses across payload variables (e.g., changing pixel size or band width) and their impact on image interpretation outcomes.

🧠 Your Contribution

Your research makes the following original contributions to the field:

- A Novel Multi-Objective Optimization Framework---> focus area
You propose and implement a design optimization framework that jointly considers optical, radiometric, geometric, and platform constraints specific to LEO small satellites. Your framework balances trade-offs between SNR, spatial resolution, spectral resolution, power, and weight.

- End-to-End Performance Evaluation
You connect the hardware design to real-world mission utility by validating payload performance using simulated hyperspectral data and MATLAB processing pipelines—something most studies do not integrate.

- Data-Informed Design Feedback---> future work
You introduce a closed-loop feedback mechanism where application-level image quality metrics (e.g., classification performance or detection accuracy) directly inform design decisions—improving relevance and impact for specific Earth observation goals.

- Custom MATLAB Toolbox Integration
You adapt and extend the MATLAB Hyperspectral Toolbox to evaluate payload-specific datasets, enabling future researchers to assess design impacts on application performance rapidly.

📌 In Summary

Your research fills a critical gap by integrating multi-objective design optimization, mission-specific performance validation, and application-driven feedback loops using MATLAB’s hyperspectral toolbox—offering a holistic approach to designing CubeSat hyperspectral imagers for real-world Earth observation missions.

3. Provide me what parameter of CubeSat hyperspectral parameter to be optimized

ChatGPT said:

To optimize the performance of a CubeSat hyperspectral imaging system, key parameters across various subsystems need to be considered. These parameters can be grouped into optical, detector, mechanical, radiometric, data management, and platform-related constraints.

1. Optical Parameters:

- Aperture size: larger apertures increase light collection but add weight and complexity.

- Focal length: Balances spatial resolution and field of view (FOV).

- Field of view (FOV): Determines the spatial coverage of the system.

- Optical throughput (f-number): Affects the brightness of the captured image.

- Spectral range: Defines the wavelength coverage (e.g., visible, NIR, SWIR).

- Spectral resolution: Narrower bands enhance spectral detail but may reduce signal-to-noise ratio (SNR).

2. Detector Parameters:

- Pixel size: Smaller pixels improve spatial resolution but may reduce sensitivity.

- Detector array size: Impacts the FOV and data acquisition rates.

- Quantum efficiency: Higher efficiency improves sensitivity.

- Noise characteristics: Lower noise increases data accuracy.

- Frame rate: Balances temporal resolution and data volume.

3. Radiometric Parameters:

- Signal-to-noise ratio (SNR): Higher SNR improves data quality.

- Dynamic range: Captures a wider range of light intensities without saturation.

- Integration time: Optimized to maximize SNR without motion blur.

4. Mechanical Parameters:

- Size, weight, and power (SWaP): Critical to ensure compatibility with CubeSat constraints.

- Vibration tolerance: Ensures stability during launch.

- Thermal stability: Maintains consistent optical performance in the space environment.

5. Data Management Parameters:

- Data compression: Reduces onboard data volume while preserving quality.

- Data rate: Matches data acquisition with the downlink capability.

- Storage capacity: Adequate to buffer data between downlink opportunities.

6. Platform-Related Parameters:

- Orbit altitude: Affects spatial resolution and revisit times.

- Pointing accuracy: Ensures the target area is imaged correctly.

- Revisit time: Determines how often the same area is observed.

- Power budget: Ensures the system can operate without exceeding power availability.

Optimization Objectives:

- Maximize spatial resolution, spectral resolution, and SNR.

- Minimize weight, volume, and power consumption.

- Balance data rate, storage, and downlink capacity.

- Optimize coverage and revisit time for the intended application (e.g., agriculture, disaster management).

By considering and optimizing these parameters, the CubeSat hyperspectral system can achieve the best performance within the constraints of the CubeSat platform.

4. what to optimized in the design and performance analysis of hyperspectral optical imagiing system for cubesat platform?

Optimizing the design and performance analysis of hyperspectral optical imaging systems for CubeSat platforms involves several key considerations:

- Size, Weight, and Power (SWaP):

  - Miniaturization: Design components to be as compact and lightweight as possible to fit within the limited payload capacity of CubeSats.

  - Power Efficiency: Ensure power consumption is minimized to conserve energy and maximize operational duration.

- Spectral Range and Resolution:

  - Spectral Coverage: Optimize spectral bands to cover the desired range for the intended applications.

  - Spectral Resolution: Balancing between spectral resolution and SWaP constraints; select the optimal resolution for the targeted applications.

- Spatial Resolution:

  - Optical System Design: Choose appropriate optics and detectors to achieve the desired spatial resolution while considering CubeSat platform constraints.

  - Image Processing: Employ advanced image processing techniques to enhance spatial resolution and reduce noise.

- Radiometric Calibration:

  - Onboard Calibration: Implement onboard calibration mechanisms to ensure accurate radiometric measurements.

  - Ground-based Calibration: Develop calibration algorithms to correct for sensor biases and maintain consistent radiometric accuracy over time.

- Data Handling and Compression:

  - Data Rate Reduction: Implement data compression techniques to reduce the amount of data transmitted from the CubeSat.

  - Onboard Processing: Integrate onboard processing capabilities to perform initial data analysis and compression before transmission.

- Pointing, Stability, and Attitude Control:

  - Pointing Accuracy: Design precise attitude control systems to accurately point the imaging system toward the target.

  - Stability: Minimize vibrations and disturbances to maintain stable imaging during operation.

- Thermal Management:

  - Temperature Control: Implement thermal management systems to ensure components operate within their specified temperature ranges.

  - Heat Dissipation: Design structures and materials to dissipate heat efficiently, preventing overheating.

- Radiation Hardening:

  - Components Selection: Use radiation-hardened components to withstand the harsh space environment and minimize the risk of radiation-induced damage.

- Ground Segment and Data Processing:

  - Data Reception: Develop ground stations capable of receiving and processing data from CubeSats.

  - Data Processing: Implement algorithms for spectral analysis, feature extraction, and data fusion to extract meaningful information from hyperspectral images.

- Mission Planning and Tasking:

  - Flexibility: Design systems with the capability to adapt tasking based on changing mission requirements and priorities.

  - Autonomy: Incorporate autonomous features for onboard decision-making and task prioritization.

By optimizing these aspects, CubeSat-based hyperspectral optical imaging systems can achieve efficient performance while meeting the specific requirements of their intended applications.

6. What to parameter to be optimized in the optical system design design and performance analysis of hyperspectral optical imagiing system for cubesat platform?

In the optical system design and performance analysis of hyperspectral optical imaging systems for CubeSat platforms, several parameters need to be optimized:

- Optical Design Parameters:

  - Optical Aperture: Determine the size of the entrance aperture for optimal light collection.

  - Field of View (FOV): Define the angular extent of the scene captured by the system.

  - Focal Length: Optimize focal length for desired spatial resolution and FOV.

  - Lens/Detector Configuration: Select appropriate lens and detector configurations to achieve desired performance.

  - Optical Coatings: Choose coatings for lenses and mirrors to maximize transmission and reduce reflections.

  - Optical System Alignment: Ensure precise alignment of optical components for optimal performance.

- Spectral Parameters:

  - Spectral Range: Determine the range of wavelengths to be captured by the system.

  - Spectral Resolution: Optimize the spectral resolution to differentiate between spectral features.

  - Bandwidth: Define the width of each spectral band to balance between spectral resolution and sensitivity.

  - Spectral Sampling: Determine the number and distribution of spectral bands across the spectrum.

- Spatial Parameters:

  - Spatial Resolution: Define the ability of the system to resolve details in the scene.

  - Pixel Size: Optimize pixel size for desired spatial resolution and field of view.

  - Detector Array Size: Determine the size of the detector array to achieve desired spatial coverage.

  - Point Spread Function (PSF): Minimize PSF to achieve sharp images.

- Radiometric Parameters:

  - Sensitivity: Maximize sensor sensitivity to detect weak signals.

  - Dynamic Range: Optimize dynamic range to accurately capture both bright and dark areas in the scene.

  - Quantization: Determine the number of bits used to digitize the signal to balance between dynamic range and data size.

  - Radiometric Calibration: Ensure accurate radiometric calibration for precise quantification of radiance or reflectance.

- Mechanical Parameters:

  - Size and Weight: Minimize size and weight to fit within CubeSat constraints.

  - Vibration and Shock Resistance: Design mechanical structures to withstand launch and operational vibrations and shocks.

  - Thermal Stability: Ensure thermal stability to maintain optical alignment and performance.

- System Integration Parameters:

  - Alignment Stability: Ensure stability of optical alignment during satellite operations.

  - Compatibility with CubeSat Platform: Design the optical system to integrate seamlessly with CubeSat platforms.

  - Power Consumption: Minimize power consumption to extend operational lifetime.

- Environmental Parameters:

  - Radiation Hardening: Design components to withstand radiation effects in space.

  - Temperature Range: Ensure operation within specified temperature ranges.

  - Vacuum Compatibility: Ensure components can operate in the vacuum environment of space.

By optimizing these parameters, the optical system can achieve the desired performance within the constraints of a CubeSat platform.

[1]	N. Masoomi, A. Zakery, and M. Afkhami, “Design of a compact multispectral telescope consisting of a Cassegrain telescope and a concave elliptical grating,” Appl Opt, vol. 63, no. 9, p. 2195, Mar. 2024, doi: 10.1364/ao.502424.

[2]	E. Por, M. Van Kooten, and V. Sarkovic, “Nyquist-Shannon sampling theorem,” 2019. [Online]. Available: https://youtu.be/6XwgbHjRo30

[3]	J. H. Lee, T. S. Jang, H. S. Yang, and S. W. Rhee, “Optical design of a compact imaging spectrometer for STSAT3,” J Opt Soc Korea, vol. 12, no. 4, pp. 262–268, 2008, doi: 10.3807/JOSK.2008.12.4.262.

[4]	J. H. Lee, C. W. Lee, Y. M. Kim, and J. W. Kim, “Optomechanical design of a compact imaging spectrometer for a microsatellite STSAT3,” J Opt Soc Korea, vol. 13, no. 2, pp. 193–200, 2009, doi: 10.3807/JOSK.2009.13.2.193.

[5]	P. Mouroulis, B. Van Gorp, R. O. Green, and D. W. Wilson, “Optical design of a CubeSat-compatible imaging spectrometer,” in Imaging Spectrometry XIX, SPIE, Sep. 2014, p. 92220D. doi: 10.1117/12.2062680.