|  |  |  | 编 号 Reference | DS-VRSSS-2JY083 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 密 级 Confidentiality | 内部 |  |
|  |  |  | 阶段标记 Phase | Z |  |
|  |  |  | 页 数 Pages | 170 |  |
| 型号代号 PROJECT; ETRSS-1. 文件名称 TITLE; ETRSS-1 Satellite Telecommand Format. |
|  | 编 写 Written by | 苏晨光20190901 |  |  |  |
|  | 校 对 Checked by | 李国军20190901 |  |  |  |
|  | 审 核 Reviewed by | 李军予20190901 吕大旻20190901 |  |  |  |
|  | 标 审 Form checked by | 蒋轶颍20190902 |  |  |  |
|  | 批 准 Approved by | 谭田20190902 |  |  |  |
| 中国空间技术研究院 China Academy of Space Technology |  |  |  |  |  |

会 签 Confirmed by

| 内容概要 / SUMMARY |  |  |  |
| --- | --- | --- | --- |
| This document gives the Tele-command (TC) list of ETRSS-1 Satellite. |  |  |  |
| 主题词 / KEY WORDS |  |  |  |
| ETRSS-1 Satellite Telecommand Format |  |  |  |
| 更改栏 / CHANGE |  |  |  |
| 更改单号 Serial No. | 更改日期 Date | 更改人 Changed by | 更改办法 Methods |

Content

Tele-command Format for ETRSS-1 Satellite	1

1 FOREWORD	1

2 ABBREVIATIONS	1

3 TC FORMAT DESCRIPTION	3

3.1 TC TYPE	3

ETRSS-1 TC types are shown in Table 1.	3

3.2 TC number	4

4 TC FORMAT	4

4.1 DIRECT TC FORMAT	4

4.2 UPLINK DATA FORMAT	5

4.3 TC Transfer frame	1

4.4 Transfer Frame Data Field	1

4.4.1 Data Field Format for OBC	1

4.4.2 Data Field Format For subcomputer	9

5 Direct TC	9

6 INDIRECT TC	10

6.1 OBC(KSM)	10

6.2 TCU(KRK)	30

6.3 TransponderA(KCA)	40

6.4 TransponderB(KCB)	41

6.5 GPS(KGS)	42

6.6 PCU(KNY)	43

6.7 BMU(KNB)	44

6.8 MC(KHC)	45

7 Data Block for Sub-computer	46

7.1 PCU data block	46

1 Power Parameter block	46

2 Battery Heater block	46

7.2 MC data block	46

1 MC 57byte data block	46

7.3 TCU data Block	47

1 TCU Datablock for Thermal Control	47

# Tele-command Format for ETRSS-1 Satellite

FOREWORD

This document gives the Tele-command (TC) list of ETRSS-1 Satellite.

Tele-command can be divided to two types by commander:

- Direct command: direct control, only sent by GS, decoded by Transponder hardware directly, simple and reliable without OBC and software, mainly control PCU, OBC and so on.

- Indirect command: decoded by OBC, can be sent by GS directly or by OBC through Automatic Safety Mode manage, Time Tagged Command or Relative Time Tagged Command.

And indirect command: can be divided to two types by executing time

- Immediate indirect command, sent by GS directly and excuted by OBC immediately.

- Time tagged Command (Include Relative Time Tagged Command), all the indirect TC can be arranged as Time Tagged Command.

- Data block is is used to modify program and work module parameter

ABBREVIATIONS

| A/D(AD) | Analogue-Digital |
| --- | --- |
| Ah | Ampere-hour |
| AIT | Assembly, Integration And Test |
| AGC | Automatic Gain Control |
| AN | Analogue |
| AOCC | Attitude and Orbit Control Computer |
| AOCS | Attitude and Orbit Control Subsystem |
| AOS | Advanced Orbiting System |
| ASS | Analogue Sun Sensor |
| BCR | Battery Charge Regulators |
| BDR | Battery Discharge Regulators |
| BL | Bi-level |
| BMU | Battery Manage Unit |
| CAN | Control Area Network |
| CCD | Charge-Coupled Device |
| CCSDS | Consultative Committee for Space Data Systems |
| CCU | Centre Control Unit |
| CDR | Critical Design Review |
| CRC | Cyclic Redundancy Check |
| D/A(DA) | Digital-Analogue |
| dB | Decibels |
| DC | Direct Current |
| DOD | Depth of Discharge |
| DPSK | Differential Phase Shift Keying |
| DS | Digital Signal |
| DSS | Digital Sun Sensor |
| DTS | Data Transmission Subsystem |
| DCU | DTS Control Unit |
| EIRP | Equivalent Isotropic Radiated Power |
| FM | Flight Model |
| FPGA | Field-Programmable Gate Array |
| GPS | Global Positioning System |
| GS | Ground Station |
| G/T | Gain/Temperature |
| GYRO | Gyroscope |
| HR | High Resolution |
| ICD | Interface Control Document |
| IDS | Interface Data Sheet |
| IRES | Infrared Earth Sensor |
| JSLC | JiuQuan Satellite Launch Site |
| LDPC | Low Density Parity Check Code |
| LVDS | Low Voltage Differential Signaling |
| MC | Multi-Spectrum Camera |
| MEA | Main Error Amplifier |
| MS | Multi-Spectrum |
| MT | Magnetic Torquer |
| MTF | Modulation Transfer Function |
| MW | Momentum Wheel |
| NA | Not Applicable/Available |
| NIR | Near Infra-Red |
| NRZ-L | Non-Return to Zero Level |
| NRZ-S | Non-Return to Zero Space |
| OBC | On-board Computer |
| OBDH | On-board Data Handling Subsystem |
| OC | Open-Collector |
| PAA | Phased Array Antenna |
| PAN | Panchromatic |
| PCM | Pulse Code Modulation |
| PCU | Power Control Unit |
| PLD | Payload |
| PM | Phase Modulation |
| PN | Pseudo-Noise |
| PROM | Programmable Read-Only Memory |
| PSK | Phase Shift Keying |
| PSS | Power Supply Subsystem |
| QPSK | Quadrature Phase Shift Keying |
| RFU | Radio Frequency Unit |
| RMS | Root Mean Square |
| RTTTC | Relative Time Tagged Telecommand |
| SA | Solar Array |
| SCC | Satellite Control Center |
| SDC | Subsystem Distribution Controller |
| SLTU | Sensor Local Terminal Unit |
| SNR | Signal to Noise Ratio |
| SR | Shunt Regulator |
| SRAM | Static Random Access Memory |
| STS | Star Sensor |
| SS-H | Star Sensor Head |
| SS-E | Star Sensor Electro-box |
| SSO | Sun- Synchronous Orbit |
| USB | Unified S-band |
| TC | Tele-Command |
| TCS | Thermal Control Subsystem |
| TDI | Time Delay Integration |
| TDM | TC Unit Data Module |
| TM | Telemetr |
| TMP | Temperature |
| TH | Thermistor |
| TT&C TTDB | Telemetry Tracking and Command Subsystem Time Tagged Data Block |
| TTTC | Time Tagged Telecommand |
| TU | TC Unit |
| TWTA | Travel Wave Tube Amplifier |
| UTC | Coordinated Universal Time |
| VCO | Voltage Controlled Oscillator |

TC FORMAT DESCRIPTION

TC TYPE

- ETRSS-1 TC types are shown in Table 1.

Table 1 TC type

| TYPES | DESCRIPTIONS |
| --- | --- |
| Direct TC | Only sent by GS, decoded by Transponder hardware directly. |
| Indirect TC | Decoded by OBC, can be sent by GS directly or by OBC through Automatic Safety Mode manage, Time Tagged Command or Relative Time Tagged Command. |
| Time-Tagged Command | Composed of Indirect TC with time tagged and stored in the SRAM waiting to execute. When the on board time is equal to the time tagged of Indirect TC, the command will be executed |
| Relative Time- Tagged Command | It is generally used for payload command sequence and automatic safety control sequence with relative time compared to start time. The command is stored in EEPROM by way of data block. Relative-time tagged Command is started by way of sending indirect TC or time tagged command. |
| Data Block | It is used to modify program and work module parameter. |

TC number

TC number is shown in Table 2.

Table 2 TC number

| No | tc NUMBER | units/subsystem |
| --- | --- | --- |
|  | OBC | KSM |
|  | TCU | KRK |
|  | TransponderA | KCA |
|  | TransponderB | KCB |
|  | GPS | KGS |
|  | PCU | KNY |
|  | BMU | KNB |
|  | AOCC | KZK |
|  | DCU | KYA |
|  | MC | KHC |

TC FORMAT

TC format is shown as Table 3。

Table 3 TC format

| PSK Preambe | TC code |
| --- | --- |

DIRECT TC FORMAT

The format of direct TC is shown as Table 4.

Table 4 Format of Direct TC

Satellite address synchronization word: 7718H

Mode word: CCH

UPLINK DATA FORMAT

Uplink data includes: indirect TC, TTC, TTD, RTTC and Data Block.

Table 5 Format of Uplink Data

Satellite address synchronization word: 41CAH

Mode word:

| 5AH | The length of executing code is 128bits(16bypes) |
| --- | --- |
| 69H | The length of executing code is 512bits(64bypes) |
| 99H | The length of executing code is 1024bits(128bypes) |
| A5H | The length of executing code is 2048bits(256bypes) |

The calculation of CRC: Y(X)=X16＋X12＋X5＋1

TC Transfer frame

The TC transfer frame is shown as following table.

Table 6 TC Transfer frame

| Transfer Frame Primary Header (40 bits) |  |  |  |  |  |  |  | Transfer Frame Data Field | Frame Error Control Field |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transfer Frame Version Number | RSVD. SPARE | RSVD. SPARE | RSVD. SPARE | Spacecraft ID | Virtual Channel ID | Frame Length | Frame Sequence Number |  |  |
| 2 bits | 1 bit | 1 bit | 2 bits | 10 bits | 6 bits | 10 bits | 8 bits |  | 16 bits |
| 00 | 0 | 0 | 00 | 00 0001 0001 |  | 0x0F, 0X3F, 0X7F, 0XFF |  | 9,57,121,249 | XOR |

Virtual Channel ID is shown as Table7：

Table7  Virtual Channel ID

| number | Sub computer | Channel identification |
| --- | --- | --- |
|  | OBC | 00H |
|  | Transponder A | 02H |
|  | PCU | 03H |
|  | AOCC | 04H |
|  | Transponder B | 05H |
|  | TCU | 07H |
|  | GPS | 0AH |
|  | DCU | 0BH |
|  | MSC | 0CH |
|  | BMU | 08H |

Transfer Frame Data Field

Data Field Format for OBC

Table8 OBC Data Field Format

| IDENTIFICATION | Data block |
| --- | --- |
| 1 byte | 8 bytes, 56 bytes, 120 bytes, 248 bytes |

Table9 IDENTIFICATION

| 20H： | Indirect TC |
| --- | --- |
| 40H： | Data block |
| 80H： | TTC |
| C0H： | TTD |
| A0H： | RTTC |

Indirect TC

Table10 Indirect TC Format

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | 20H | IDENTIFICATION |
| 1 | m | Lenghth m=4n+2 |
| 2 | 00H | reserved |
| 3 | n | Command counte |
| 4 | ##H | Command 1 |
| 5 | ##H |  |
| 6 | ##H |  |
| 7 | ##H |  |
|  | ##H | Command 2 |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  |  | … |
|  |  | … |
|  | ##H |  |
|  | ##H | Command n |
|  | ##H |  |
| m+1 | ##H |  |
| m+2 | ##H |  |
|  | AAH | reserved |
|  | … |  |
|  | AAH |  |
| L-1 | AAH |  |

Data block For OBC

One time adjustment

The time difference between the ground and satellite should be calculated by ground station and be transmitted through the uplink channel. OBC performs immediately the operation of addition between the on board time and the time difference in order to aim at the ground time after acceptance of time calibration data block. The time adjustment shall be performed once the time difference goes beyond ±5 milliseconds.。

Table11 One time adjustment data block format

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | 40H | IDENTIFICATION |
| 1 | 11H | ASSISTANT SYMBOL |
| 2 | 01H | TYPE |
| 3 | ##H | ADJUST SECOND Complementary code, HIGH BYTE FIRST |
| 4 | ##H |  |
| 5 | ##H |  |
| 6 | ##H |  |
| 7 | ##H | ADJUST MILLSECOND TRUE FORM, HIGH BYTE FIRST |
| 8 | ##H |  |

Periodic Time Correction

The periodic time correction is to add or subtract periodically one millisecond of on board time to eliminate the predicted time error. The interval to perform the uniform time correction is the range from 1 to 0xFFFFFF seconds.

Table12 uniform time correction Format

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | 40H | IDENTIFICATION |
| 1 | 11H | ASSISTANT SYMBOL |
| 2 | 02H | TYPE |
| 3 | ##H | PERIOD TIME(Sec, >=1) TRUE FORM, HIGH BYTE FIRST |
| 4 | ##H |  |
| 5 | ##H |  |
| 6 | ##H | PERIOD TIME(ms, <1000) TRUE FORM, HIGH BYTE FIRST |
| 7 | ##H |  |
| 8 | ##H | 00H: STOP; 66H: +1ms 99H: -1ms |

Ground time setting

Ground time setting means setting the on board time directly according to forward data block .

Table13 Ground time setting Format

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | 40H | IDENTIFICATION |
| 1 | 11H | ASSISTANT SYMBOL |
| 2 | 03H | TYPE |
| 3 | ##H | ADJUST SECOND TRUE FORM, HIGH BYTE FIRST |
| 4 | ##H |  |
| 5 | ##H |  |
| 6 | ##H |  |
| 7 | ##H | ADJUST MILLSECOND TRUE FORM, HIGH BYTE FIRST |
| 8 | ##H |  |

Memory download

Table14 Memory download Format

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | 40H | IDENTIFICATION |
| 1 | 12H | ASSISTANT SYMBOL |
| 2 | 00H | reserved |
| 3 | ##H | Address |
| 4 | ##H |  |
| 5 | ##H |  |
| 6 | ##H |  |
| 7 | ##H | LENGTH(TOTAL BYTES/64) |
| 8 | ##H |  |

TTC Format

Each TTTC is composed with 10 bytes. The first 2 bytes is Task ID, the next 4 bytes is UTC time, and the last 4 bytes is execution code (Indirect TC CODE). The time sequence is from smallest to largest. OBC TTC data block format is described in the following table.

Table15 TTC data block format

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | 80H | IDENTIFICATION |
| 1 | m | m=10n+2 |
| 2 | 00H | RESERVE WORD |
| 3 | n | COMMAND COUNT |
| 4 | ##H | TASK ID1 |
| 5 | ##H |  |
| 6 | ##H | UTC TIME1 |
| 7 | ##H |  |
| 8 | ##H |  |
| 9 | ##H |  |
| 10 | ##H | EXECUTION CODE1 |
| 11 | ##H |  |
| 12 | ##H |  |
| 13 | ##H |  |
| 14 | ##H | TASK ID2 |
| 15 | ##H |  |
| 16 | ##H | UTC TIME2 |
| 17 | ##H |  |
| 18 | ##H |  |
| 19 | ##H |  |
| 20 | ##H | EXECUTION CODE2 |
| 21 | ##H |  |
| 22 | ##H |  |
| 23 | ##H |  |
| … | … | … |
| … | … | … |
| … | … | … |
|  | ##H | TASK IDn |
|  | ##H |  |
|  | ##H | UTC TIMEn |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H | EXECUTION CODEn |
|  | ##H |  |
| m | ##H |  |
| m+1 | ##H |  |
|  | AAH | RESERVED |
|  | … |  |
|  | AAH |  |
| L-1 | AAH |  |

Table16 TTC data block insert rule

| number | New TTC Data block |  | TTC in buffer | Executing status |
| --- | --- | --- | --- | --- |
|  | Max time | < | Min time | New TTC block insert in front of Min time command |
|  | Min time | > | Max time | New TTC block insert in behind of Max time command |
|  | Min time | > | Commend n time | New TTC block insert between command n and command n+1 |
|  | Max time | < | Commend n+1 time |  |
|  | Other condition |  |  | TTC error |

Remark: n is index of TTC.

TTD format

Each TTD is composed with 64 bytes. The first 2 bytes is Task ID，the next 4 bytes is UTC time, the next is virtual channel ID and the last 57 bytes is TC data. the time sequence is from smallest to largest. OBC TTD format is described in the following table.

Table17程控数据块格式

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | C0 | IDENTIFICATION |
| 1 | m | Length m=14n+2 |
| 2 | 00 | reserved |
| 3 | n | Command counter（1≤n≤3） |
| 4 | ##H | Task ID1 |
| 5 | ##H |  |
| 6 | ##H | Data block time 1 (s) |
| 7 | ##H |  |
| 8 | ##H |  |
| 9 | ##H |  |
| 10 | ##H | Data block channel identification 1 |
| 11 | ##H | Data block code 1 |
| 12 | ##H |  |
| 13 | ##H |  |
|  | …… |  |
| 63 | ##H |  |
| 64 | ##H |  |
| 65 | ##H |  |
| 66 | ##H |  |
| 67 | ##H |  |
|  | ##H | Task ID2 |
|  | ##H |  |
|  | ##H | Data block time 2(s) |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H | Data block channel identification 2 |
|  | ##H | Data block code 2 |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  |  | … |
|  |  | … |
|  | ##H | Task IDn |
|  | ##H |  |
|  | ##H | Data block time n (s) |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H | Data block channel identification n |
|  | ##H | Data block code n |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
| m | ##H |  |
| m+1 | ##H |  |
|  | AAH | reserved |
|  | … |  |
|  | AAH |  |
| L-1 | AAH |  |

Table18 TTD Insert rule

| Number | New TTD block |  | TTD in buffer | Executing status |
| --- | --- | --- | --- | --- |
|  | Max time | < | Min time | New TTD block insert in front of Min time command |
|  | Min time | > | Max time | New TTD block insert behind of Max time command |
|  | Min time | > | Commend n time | New TTD block insert between command n and command n+1 |
|  | Max time | < | Commend n+1 time |  |
|  | Other condition |  |  | TTD block error |

Remark: n is index of TTD.

RTTC format

Each RTTTC is composed with 6 bytes. The first 2 bytes is the time relative to start time, and the last 4 bytes is TC code (Indirect TC CODE). The time sequence is from smallest to largest. OBC RTTC data block format is described in the following table.

Table19 RTTC data block format

| NO | DATA | LENGTH | DESCRIPTION |
| --- | --- | --- | --- |
| 0 | A0H | 1 | IDENTIFICATION |
| 1 | ##H | 1 | group number |
| 2 | ##H | 1 | Command counter |
| 3 | ##H | 8 | reserved |
| 4 | ##H | 2 | High 4bit, segment id Low 12bit, TC ∆time |
|  | ##H | 4 | TC code 1 |
|  | ##H | 2 | High 4bit, segment id Low 12bit, TC ∆time |
|  | ##H | 4 | TC code 2 |
| … | … | … | … |
|  | ##H | 2 | High 4bit, segment id Low 12bit, TC ∆time |
| L-1 | ##H | 4 | TC code n |

OBC Program Data block

Table20 Program data block format

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | E0H | IDENTIFICATION |
| 1 | m | Lengh , m= Code length +6 |
| 2 | ##H | Program Type：00H |
| 3 | n | Code length |
| 4 | ##H | Address |
| 5 | ##H |  |
| 6 | ##H |  |
| 7 | ##H |  |
|  | ##H | Code data |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | ##H |  |
|  | AAH | reserved |
|  | … |  |
|  | AAH |  |
| L-1 | AAH |  |

Table21 Program data check block format

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | E0H | IDENTIFICATION |
| 1 | m | m = 5 |
| 2 | 0AH | Check Block |
| 3 | ##H | Program ID 00H~03H |
| 4 | ##H | Code length |
| 5 | ##H |  |
| 6 | ##H | Code Check Sum |
|  | AAH | reserved |
|  | … |  |
|  | AAH |  |
| L-1 | AAH |  |

Safety Mode AN threshold Setting Block

Table22 Safety Mode AN threshold Setting Block

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | 40H | IDENTIFICATION |
| 1 | 14H | Safety Mode Criterion Table Setting Block |
| 2 | 00H | Set AN threshold |
| 3 | ##H | Safety Mode ID： 07- Energy protect Safety Mode(triggered by TCU 08- CCD temperature Safety Mode 09- Phased array antenna temperature Safety Mode 0A-Phased array antenna working timeout Safety Mode |
| 4 | ##H | AN Threshold |
| 5 | ##H |  |
| 6 | ##H |  |
| 8 | ##H |  |

Safety Mode Trigger Count Setting Block

Table23 Safety Mode Trigger Count Setting Block

| NO | DATA | DESCRIPTION |
| --- | --- | --- |
| 0 | 40H | IDENTIFICATION |
| 1 | 14H | Safety Mode Criterion Table Setting Block |
| 2 | 01H | Set Trigger Count |
| 3 | ##H | Safety Mode ID： 0A- Phased array_antenna working timeout Safety Mode |
| 4 | ##H | Trigger Count |
| 5 | ##H |  |
| 6 | ##H |  |
| 8 | ##H |  |

Data Field Format For subcomputer

The data field for subcomputer is defined by subcomputer, refer to section 7.

Direct TC

| 序 号 | TC Number | Name | 备份间接指令 | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | K01 | Discharge_switch_on |  | Dischargeswitchon |  |
| 3 | K03 | OBC(A)_ON/(B)_OFF |  | OBC(A)_ON_and_OBC(B)_OFF |  |
| 4 | K04 | OBC(B)_ON/(A)_OFF |  | OBC(B)_ON_and_OBC(A)_OFF |  |
| 5 | K05 | PCU_Subcomputer(A)_ON/(B)_OFF |  | PCU_Subcomputer(A)_ON/(B)_OFF |  |
| 6 | K06 | PCU_Subcomputer(B)_ON/(A)_OFF |  | PCU_Subcomputer(B)_ON/(A)_OFF |  |
| 7 | K07 | Reserved |  | Reserved |  |
| 9 | K09 | telecommand_channel_self-test |  | telecommand_channel_self-test |  |
| 10 | K10 | Transmitter_of_USB_A_on |  | Transmitter_of_USB_A_on |  |
| 11 | K11 | Transmitter_of_USB_B_on |  | Transmitter_of_USB_B_on |  |
| 12 | K12 | Transmitter_of_USB_A_off |  | Transmitter_of_USB_A_off |  |
| 13 | K13 | Transmitter_of_USB_B_off |  | Transmitter_of_USB_B_off |  |
| 14 | K14 | Transponder_A_and_B_coherentin_work |  | Transponder_A_and_B_coherentin_work |  |
| 15 | K15 | Transponder_A_and_B_incoherentin_work |  | Transponder_A_and_B_incoherentin_work |  |
| 16 | K16 | Transponder_A_and_B_ranging_on |  | Transponder_A_and_B_ranging_on |  |
| 17 | K17 | Transponder_A_and_B_ranging_off |  | Transponder_A_and_B_ranging_off |  |
| 18 | K18 | Approving_state_controlled_by_OBDH |  | Approving_state_controlled_by_OBDH |  |
| 19 | K19 | Forbidding_state_controlled_by_OBDH |  | Forbidding_state_controlled_by_OBDH |  |

INDIRECT TC

OBC(KSM)

Table 23OBC(KSM)Table

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KSM001 | RTTC_START | 0001XXXXH | XXXX:B15~B11:Group number,from 00H to1FH. B10~B0:Segment time(△t).When the KSM37 is enabled,the command is executed. |  |
| 2 | KSM01-0 | RTTC_0_START | 00010000H | △t=0 |  |
| 3 | KSM01-1 | RTTC_1_START | 00010800H | △t=0 |  |
| 4 | KSM01-2 | RTTC_2_START | 00011000H | △t=0 |  |
| 5 | KSM01-3 | RTTC_3_START | 00011800H | △t=0 |  |
| 6 | KSM01-4 | RTTC_4_START | 00012000H | △t=0 |  |
| 7 | KSM01-5 | RTTC_5_START | 00012800H | △t=0 |  |
| 8 | KSM01-6 | RTTC_6_START | 00013000H | △t=0 |  |
| 9 | KSM01-7 | RTTC_7_START | 00013800H | △t=0 |  |
| 10 | KSM01-8 | RTTC_8_START | 00014000H | △t=0 |  |
| 11 | KSM01-9 | RTTC_9_START | 00014800H | △t=0 |  |
| 12 | KSM01-10 | RTTC_10_START | 00015000H | △t=0 |  |
| 13 | KSM01-11 | RTTC_11_START | 00015800H | △t=0 |  |
| 14 | KSM01-12 | RTTC_12_START | 00016000H | △t=0 |  |
| 15 | KSM01-13 | RTTC_13_START | 00016800H | △t=0 |  |
| 16 | KSM01-14 | RTTC_14_START | 00017000H | △t=0 |  |
| 17 | KSM01-15 | RTTC_15_START | 00017800H | △t=0 |  |
| 18 | KSM01-16 | RTTC_16_START | 00018000H | △t=0 |  |
| 19 | KSM01-17 | RTTC_17_START | 00018800H | △t=0 |  |
| 20 | KSM01-18 | RTTC_18_START | 00019000H | △t=0 |  |
| 21 | KSM01-19 | RTTC_19_START | 00019800H | △t=0 |  |
| 22 | KSM01-20 | RTTC_20_START | 0001A000H | △t=0 |  |
| 23 | KSM01-21 | RTTC_21_START | 0001A800H | △t=0 |  |
| 24 | KSM01-22 | RTTC_22_START | 0001B000H | △t=0 |  |
| 25 | KSM01-23 | RTTC_23_START | 0001B800H | △t=0 |  |
| 26 | KSM01-24 | RTTC_24_START | 0001C000H | △t=0 |  |
| 27 | KSM01-25 | RTTC_25_START | 0001C800H | △t=0 |  |
| 28 | KSM01-26 | RTTC_26_START | 0001D000H | △t=0 |  |
| 29 | KSM01-27 | RTTC_27_START | 0001D800H | △t=0 |  |
| 30 | KSM01-28 | RTTC_28_START | 0001E000H | △t=0 |  |
| 31 | KSM01-29 | RTTC_29_START | 0001E800H | △t=0 |  |
| 32 | KSM01-30 | RTTC_30_START | 0001F000H | △t=0 |  |
| 33 | KSM01-31 | RTTC_31_START | 0001F800H | △t=0 |  |
| 34 | KSM02 | RTTC_XX~YY_STOP | 0002XXYYH | Stop XX~YY groups RTTTC. XX<=YY<32 |  |
| 35 | KSM02-00 | RTTC_0_STOP | 00020000H |  |  |
| 36 | KSM02-01 | RTTC_1_STOP | 00020101H |  |  |
| 37 | KSM02-02 | RTTC_2_STOP | 00020202H |  |  |
| 38 | KSM02-03 | RTTC_3_STOP | 00020303H |  |  |
| 39 | KSM02-04 | RTTC_4_STOP | 00020404H |  |  |
| 40 | KSM02-05 | RTTC_5_STOP | 00020505H |  |  |
| 41 | KSM02-06 | RTTC_6_STOP | 00020606H |  |  |
| 42 | KSM02-07 | RTTC_7_STOP | 00020707H |  |  |
| 43 | KSM02-08 | RTTC_8_STOP | 00020808H |  |  |
| 44 | KSM02-09 | RTTC_9_STOP | 00020909H |  |  |
| 45 | KSM02-10 | RTTC_10_STOP | 00020A0AH |  |  |
| 46 | KSM02-11 | RTTC_11_STOP | 00020B0BH |  |  |
| 47 | KSM02-12 | RTTC_12_STOP | 00020C0CH |  |  |
| 48 | KSM02-13 | RTTC_13_STOP | 00020D0DH |  |  |
| 49 | KSM02-14 | RTTC_14_STOP | 00020E0EH |  |  |
| 50 | KSM02-15 | RTTC_15_STOP | 00020F0FH |  |  |
| 51 | KSM02-16 | RTTC_16_STOP | 00021010H |  |  |
| 52 | KSM02-17 | RTTC_17_STOP | 00021111H |  |  |
| 53 | KSM02-18 | RTTC_18_STOP | 00021212H |  |  |
| 54 | KSM02-19 | RTTC_19_STOP | 00021313H |  |  |
| 55 | KSM02-20 | RTTC_20_STOP | 00021414H |  |  |
| 56 | KSM02-21 | RTTC_21_STOP | 00021515H |  |  |
| 57 | KSM02-22 | RTTC_22_STOP | 00021616H |  |  |
| 58 | KSM02-23 | RTTC_23_STOP | 00021717H |  |  |
| 59 | KSM02-24 | RTTC_24_STOP | 00021818H |  |  |
| 60 | KSM02-25 | RTTC_25_STOP | 00021919H |  |  |
| 61 | KSM02-26 | RTTC_26_STOP | 00021A1AH |  |  |
| 62 | KSM02-27 | RTTC_27_STOP | 00021B1BH |  |  |
| 63 | KSM02-28 | RTTC_28_STOP | 00021C1CH |  |  |
| 64 | KSM02-29 | RTTC_29_STOP | 00021D1DH |  |  |
| 65 | KSM02-30 | RTTC_30_STOP | 00021E1EH |  |  |
| 66 | KSM02-31 | RTTC_31_STOP | 00021F1FH |  |  |
| 67 | KSM03 | RTTC_XX~YY_CLEAR | 0003XXXXH | Clear XX~YY RTTTC groups. XX<=YY<32 |  |
| 68 | KSM03-00 | RTTC_0_CLEAR | 00030000H |  |  |
| 69 | KSM03-01 | RTTC_1_CLEAR | 00030101H |  |  |
| 70 | KSM03-02 | RTTC_2_CLEAR | 00030202H |  |  |
| 71 | KSM03-03 | RTTC_3_CLEAR | 00030303H |  |  |
| 72 | KSM03-04 | RTTC_4_CLEAR | 00030404H |  |  |
| 73 | KSM03-05 | RTTC_5_CLEAR | 00030505H |  |  |
| 74 | KSM03-06 | RTTC_6_CLEAR | 00030606H |  |  |
| 75 | KSM03-07 | RTTC_7_CLEAR | 00030707H |  |  |
| 76 | KSM03-08 | RTTC_8_CLEAR | 00030808H |  |  |
| 77 | KSM03-09 | RTTC_9_CLEAR | 00030909H |  |  |
| 78 | KSM03-10 | RTTC_10_CLEAR | 00030A0AH |  |  |
| 79 | KSM03-11 | RTTC_11_CLEAR | 00030B0BH |  |  |
| 80 | KSM03-12 | RTTC_12_CLEAR | 00030C0CH |  |  |
| 81 | KSM03-13 | RTTC_13_CLEAR | 00030D0DH |  |  |
| 82 | KSM03-14 | RTTC_14_CLEAR | 00030E0EH |  |  |
| 83 | KSM03-15 | RTTC_15_CLEAR | 00030F0FH |  |  |
| 84 | KSM03-16 | RTTC_16_CLEAR | 00031010H |  |  |
| 85 | KSM03-17 | RTTC_17_CLEAR | 00031111H |  |  |
| 86 | KSM03-18 | RTTC_18_CLEAR | 00031212H |  |  |
| 87 | KSM03-19 | RTTC_19_CLEAR | 00031313H |  |  |
| 88 | KSM03-20 | RTTC_20_CLEAR | 00031414H |  |  |
| 89 | KSM03-21 | RTTC_21_CLEAR | 00031515H |  |  |
| 90 | KSM03-22 | RTTC_22_CLEAR | 00031616H |  |  |
| 91 | KSM03-23 | RTTC_23_CLEAR | 00031717H |  |  |
| 92 | KSM03-24 | RTTC_24_CLEAR | 00031818H |  |  |
| 93 | KSM03-25 | RTTC_25_CLEAR | 00031919H |  |  |
| 94 | KSM03-26 | RTTC_26_CLEAR | 00031A1AH |  |  |
| 95 | KSM03-27 | RTTC_27_CLEAR | 00031B1BH |  |  |
| 96 | KSM03-28 | RTTC_28_CLEAR | 00031C1CH |  |  |
| 97 | KSM03-29 | RTTC_29_CLEAR | 00031D1DH |  |  |
| 98 | KSM03-30 | RTTC_30_CLEAR | 00031E1EH |  |  |
| 99 | KSM03-31 | RTTC_31_CLEAR | 00031F1FH |  |  |
| 100 | KSM04-02 | Transponder_A_TM_online_SET | 000402AAH |  |  |
| 101 | KSM04-03 | PCU_TM_online_SET | 000403AAH |  |  |
| 102 | KSM04-04 | AOCC_TM_online_SET | 000404AAH |  |  |
| 103 | KSM04-05 | Transponder_TM_online_SET | 000405AAH |  |  |
| 104 | KSM04-07 | TCU_TM_online_SET | 000407AAH |  |  |
| 105 | KSM04-08 | BMU_TM_online_SET | 000408AAH |  |  |
| 106 | KSM04-0A | GPS_TM_online_SET | 00040AAAH |  |  |
| 107 | KSM04-0B | DCU_TM_online_SET | 00040BAAH |  |  |
| 108 | KSM04-0C | MC_TM_online_SET | 00040CAAH |  |  |
| 109 | KSM05-02 | Transponder_A_TM_offline_SET | 000502AAH |  |  |
| 110 | KSM05-03 | PCU_TM_offline_SET | 000503AAH |  |  |
| 111 | KSM05-04 | AOCC_TM_offline_SET | 000504AAH |  |  |
| 112 | KSM05-05 | Transponder_TM_offline_SET | 000505AAH |  |  |
| 113 | KSM05-07 | TCU_TM_offline_SET | 000507AAH |  |  |
| 114 | KSM05-08 | BMU_TM_offline_SET | 000508AAH |  |  |
| 115 | KSM05-0A | GPS_TM_offline_SET | 00050AAAH |  |  |
| 116 | KSM05-0B | DCU_TM_offline_SET | 00050BAAH |  |  |
| 117 | KSM05-0C | MC_TM_offline_SET | 00050CAAH |  |  |
| 118 | KSM06 | All_Subcomputers_online_SET | 0006AAAAH |  |  |
| 119 | KSM07 | All_Subcomputers_offline_SET | 0007AAAAH |  |  |
| 120 | KSM08-02 | TransponderA_CANbusA_working_SET | 000802AAH |  |  |
| 121 | KSM08-03 | PCU_CANbusA_working_SET | 000803AAH |  |  |
| 122 | KSM08-04 | AOCC_CANbusA_working_SET | 000804AAH |  |  |
| 123 | KSM08-05 | TransponderB_CANbusA_working_SET | 000805AAH |  |  |
| 124 | KSM08-07 | TCU_CANbusA_working_SET | 000807AAH |  |  |
| 125 | KSM08-08 | BMU_CANbusA_working_SET | 000808AAH |  |  |
| 126 | KSM08-0A | GPS_CANbusA_working_SET | 00080AAAH |  |  |
| 127 | KSM08-0B | DCU_CANbusA_working_SET | 00080BAAH |  |  |
| 128 | KSM08-0C | MC_CANbusA_working_SET | 00080CAAH |  |  |
| 129 | KSM09-02 | TransponderA_CANbusB_working_SET | 000902AAH |  |  |
| 130 | KSM09-03 | PCU_CANbusB_working_SET | 000903AAH |  |  |
| 131 | KSM09-04 | AOCC_CANbusB_working_SET | 000904AAH |  |  |
| 132 | KSM09-05 | TransponderB_CANbusB_working_SET | 000905AAH |  |  |
| 133 | KSM09-07 | TCU_CANbusB_working_SET | 000907AAH |  |  |
| 134 | KSM09-08 | BMU_CANbusB_working_SET | 000908AAH |  |  |
| 135 | KSM09-0A | GPS_CANbusB_working_SET | 00090AAAH |  |  |
| 136 | KSM09-0B | DCU_CANbusB_working_SET | 00090BAAH |  |  |
| 137 | KSM09-0C | MC_CANbusB_working_SET | 00090CAAH |  |  |
| 138 | KSM10 | All_Subcomputers_CANbusA_working_SET | 000AAAAAH |  |  |
| 139 | KSM11 | All_Subcomputers_CANbusB_working_SET | 000BAAAAH |  |  |
| 140 | KSM12 | TTC_and_TTD_ENABLE | 000CAAAAH |  |  |
| 141 | KSM13 | TTC_and_TTD_DISABLE | 000DAAAAH |  |  |
| 142 | KSM14 | TTC_and_TTD_memory_CLEAR | 000EAAAAH |  |  |
| 143 | KSM15-55 | RS422_interface_test_mod1 | 000F55AAH |  |  |
| 144 | KSM15-66 | RS422_interface_test_mod2 | 000F66AAH |  |  |
| 145 | KSM15-99 | RS422_interface_DISABLE | 000F99AAH |  |  |
| 146 | KSM16 | Time_calibration_with_GPS_ENABLE | 0010AAAAH |  |  |
| 147 | KSM17 | Time_calibration_with_GPS_DISABLE | 0011AAAAH |  |  |
| 148 | KSM18 | Delay_TM_sampling_rate_SET | 0012XXAAH | XX:sampling rate(01H~FFH) |  |
| 149 | KSM19 | TM_Download_DISABLE | 0013AAAAH |  |  |
| 150 | KSM20 | TM_Download_ENABLE | 0014AAAAH |  |  |
| 151 | KSM21-66 | Safety_Mode_Task_ENABLE | 001566AAH |  |  |
| 152 | KSM21-99 | Safety_Mode_Task_DISABLE | 001599AAH |  |  |
| 153 | KSM22 | safety_mode_SET | 0016XXYY | XX：safety mode type；YY：66H-enable，99H-disable |  |
| 154 | KSM22-00-66 | TTC_check_safety_mode_ENABLE | 00160066H |  |  |
| 155 | KSM22-01-66 | TTD_check_safety_mode_ENABLE | 00160166H |  |  |
| 156 | KSM22-02-66 | Energy_protect_Safety_Mode(_triggered_by_PCU)_ENABLE | 00160266H |  |  |
| 157 | KSM22-03-66 | AOCS_Global_Attitude_Acquisition_Safety_Mode_ENABLE | 00160366H |  |  |
| 158 | KSM22-04-66 | AOCS_Stop_Control_Safety_Mode_ENABLE | 00160466H |  |  |
| 159 | KSM22-05-66 | Energy_warning_Safety_Mode_1_ENABLE | 00160566H |  |  |
| 160 | KSM22-06-66 | Energy_warning_Safety_Mode_2_ENABLE | 00160666H |  |  |
| 161 | KSM22-07-66 | Energy_protect_Safety_Mode(_triggered_by_TCU)_ENABLE | 00160766H |  |  |
| 162 | KSM22-08-66 | CCD_temperature_Safety_Mode_ENABLE | 00160866H |  |  |
| 163 | KSM22-09-66 | Phased_array_antenna_temperature_Safety_Mode_ENABLE | 00160966H |  |  |
| 164 | KSM22-0A-66 | Phased_array_antenna_working_timeout_Safety_Mode_ENABLE | 00160A66H |  |  |
| 165 | KSM22-0B-66 | S-L_Separation_Switch_Timeout_Safety_Mode_ENABLE | 00160B66H |  |  |
| 166 | KSM22-0C-66 | TC_monitor_Timeout_Safety_Mode_ENABLE | 00160C66H |  |  |
| 167 | KSM22-00-99 | TTC_check_safety_mode_DISABLE | 00160099H |  |  |
| 168 | KSM22-01-99 | TTD_check_safety_mode_DISABLE | 00160199H |  |  |
| 169 | KSM22-02-99 | Energy_protect_Safety_Mode(_triggered_by_PCU)_DISABLE | 00160299H |  |  |
| 170 | KSM22-03-99 | AOCS_Global_Attitude_Acquisition_Safety_Mode_DISABLE | 00160399H |  |  |
| 171 | KSM22-04-99 | AOCS_Stop_Control_Safety_Mode_DISABLE | 00160499H |  |  |
| 172 | KSM22-05-99 | Energy_warning_Safety_Mode_1_DISABLE | 00160599H |  |  |
| 173 | KSM22-06-99 | Energy_warning_Safety_Mode_2_DISABLE | 00160699H |  |  |
| 174 | KSM22-07-99 | Energy_protect_Safety_Mode(_triggered_by_TCU)_DISABLE | 00160799H |  |  |
| 175 | KSM22-08-99 | CCD_temperature_Safety_Mode_DISABLE | 00160899H |  |  |
| 176 | KSM22-09-99 | Phased_array_antenna_temperature_Safety_Mode_DISABLE | 00160999H |  |  |
| 177 | KSM22-0A-99 | Phased_array_antenna_working_timeout_Safety_Mode_DISABLE | 00160A99H |  |  |
| 178 | KSM22-0B-99 | S-L_Separation_Switch_Timeout_Safety_Mode_DISABLE | 00160B99H |  |  |
| 179 | KSM22-0C-99 | TC_monitor_Timeout_Safety_Mode_DISABLE | 00160C99H |  |  |
| 180 | KSM23 | Packets_on/offline_SET | 0017XXYY | XX：Packet；YY：66H-online，99H-offline |  |
| 181 | KSM24 | TTC_online_SET | 0018XXXXH | XXXX:TTC number, include 6 commands For example: 0000H,No.1~No.6,0001H,No.2~No.7. |  |
| 182 | KSM25 | TTD_online_SET | 0019XXXXH | XXXX:TTD number, include1datablock |  |
| 183 | KSM26-00 | RTTC_1_online | 001A00AAH |  |  |
| 184 | KSM26-01 | RTTC_2_online | 001A01AAH |  |  |
| 185 | KSM26-02 | RTTC_3_online | 001A02AAH |  |  |
| 186 | KSM26-03 | RTTC_4_online | 001A03AAH |  |  |
| 187 | KSM26-04 | RTTC_5_online | 001A04AAH |  |  |
| 188 | KSM26-05 | RTTC_6_online | 001A05AAH |  |  |
| 189 | KSM26-06 | RTTC_7_online | 001A06AAH |  |  |
| 190 | KSM26-07 | RTTC_8_online | 001A07AAH |  |  |
| 191 | KSM26-08 | RTTC_9_online | 001A08AAH |  |  |
| 192 | KSM26-09 | RTTC_10_online | 001A09AAH |  |  |
| 193 | KSM26-10 | RTTC_11_online | 001A0AAAH |  |  |
| 194 | KSM26-11 | RTTC_12_online | 001A0BAAH |  |  |
| 195 | KSM26-12 | RTTC_13_online | 001A0CAAH |  |  |
| 196 | KSM26-13 | RTTC_14_online | 001A0DAAH |  |  |
| 197 | KSM26-14 | RTTC_15_online | 001A0EAAH |  |  |
| 198 | KSM26-15 | RTTC_16_online | 001A0FAAH |  |  |
| 199 | KSM26-16 | RTTC_17_online | 001A10AAH |  |  |
| 200 | KSM26-17 | RTTC_18_online | 001A11AAH |  |  |
| 201 | KSM26-18 | RTTC_19_online | 001A12AAH |  |  |
| 202 | KSM26-19 | RTTC_20_online | 001A13AAH |  |  |
| 203 | KSM26-20 | RTTC_21_online | 001A14AAH |  |  |
| 204 | KSM26-21 | RTTC_22_online | 001A15AAH |  |  |
| 205 | KSM26-22 | RTTC_23_online | 001A16AAH |  |  |
| 206 | KSM26-23 | RTTC_24_online | 001A17AAH |  |  |
| 207 | KSM26-24 | RTTC_25_online | 001A18AAH |  |  |
| 208 | KSM26-25 | RTTC_26_online | 001A19AAH |  |  |
| 209 | KSM26-26 | RTTC_27_online | 001A1AAAH |  |  |
| 210 | KSM26-27 | RTTC_28_online | 001A1BAAH |  |  |
| 211 | KSM26-28 | RTTC_29_online | 001A1CAAH |  |  |
| 212 | KSM26-29 | RTTC_30_online | 001A1DAAH |  |  |
| 213 | KSM26-30 | RTTC_31_online | 001A1EAAH |  |  |
| 214 | KSM26-31 | RTTC_32_online | 001A1FAAH |  |  |
| 215 | KSM27 | packets_phase_set | 001BXXYYH | XX：packet；YY：phase |  |
| 216 | KSM28 | packets_circle_set | 001CXXYY | XX:packet；YY：circle |  |
| 217 | KSM29 | TTD_task_ENABLE | 001DAAAAH |  |  |
| 218 | KSM30 | TTD_task_DISABLE | 001EAAAAH |  |  |
| 219 | KSM31 | TTD_memory_CLAER | 001FAAAAH |  |  |
| 220 | KSM32 | reserved | 0020AAAAH |  |  |
| 221 | KSM33 | reserved | 0021AAAAH |  |  |
| 222 | KSM34 | safety_mode_counter_accelerate | 0022XXYYH | XX：counter type；YY：second_before_timeout |  |
| 223 | KSM34-00-60 | S-L_Separation_Switch_Timeoutsafety_mode_counter_accelerate | 0022003CH |  |  |
| 224 | KSM34-01-60 | TC_monitor_Timeout_Safety_Mode_counter_accelerate | 0022013CH |  |  |
| 225 | KSM35 | safety_mode_counter_reset | 0023XXAAH | XX：counter type |  |
| 226 | KSM35-00 | S-L_Separation_Switch_Timeoutsafety_mode_counter_reset | 002300AAH |  |  |
| 227 | KSM35-01 | TC_monitor_Timeout_Safety_Mode_counter_reset | 002301AAH |  |  |
| 228 | KSM36 | communication_safety_mode_SET | 0024XXYY | XX：sub-computer virtual channel；YY：0x66-enable，0x99-disable |  |
| 229 | KSM36-02-66 | TransponderA_communication_safety_mode_ENABLE | 00240266H |  |  |
| 230 | KSM36-03-66 | PCU_communication_safety_mode_ENABLE | 00240366H |  |  |
| 231 | KSM36-05-66 | TransponderB_communication_safety_mode_ENABLE | 00240566H |  |  |
| 232 | KSM36-07-66 | TCU_communication_safety_mode_ENABLE | 00240766H |  |  |
| 233 | KSM36-08-66 | BMU_communication_safety_mode_ENABLE | 00240866H |  |  |
| 234 | KSM36-0A-66 | GPS_communication_safety_mode_ENABLE | 00240A66H |  |  |
| 235 | KSM36-02-99 | TransponderA_communication_safety_mode_ENABLE | 00240299H |  |  |
| 236 | KSM36-03-99 | PCU_communication_safety_mode_DISABLE | 00240399H |  |  |
| 237 | KSM36-05-99 | TransponderB_communication_safety_mode_DISABLE | 00240599H |  |  |
| 238 | KSM36-07-99 | TCU_communication_safety_mode_DISABLE | 00240799H |  |  |
| 239 | KSM36-08-99 | BMU_communication_safety_mode_DISABLE | 00240899H |  |  |
| 240 | KSM36-0A-99 | GPS_communication_safety_mode_DISABLE | 00240A99H |  |  |
| 241 | KSM37 | RTTC_task_ENABLE | 0025AAAAH |  |  |
| 242 | KSM38 | RTTC_task_DISABLE | 0026AAAAH |  |  |
| 243 | KSM39 | GPS_to_OBC_TIME_TRANSFER | 0027AAAAH |  |  |
| 244 | KSM40-00-07 | Downlink_Energy_protect_Safety_Mode(triggered_by_TCU)_AN_threshold | 00280007H |  |  |
| 245 | KSM40-00-08 | Downlink_CCD_temperature_Safety_Mode_AN_threshold | 00280008H |  |  |
| 246 | KSM40-00-09 | Downlink_Phased_array_antenna_temperature_Safety_Mode_AN_threshold | 00280009H |  |  |
| 247 | KSM40-00-0A | Downlink_Phased_array_antenna_working_timeout_Safety_Mode_AN_threshold | 0028000AH |  |  |
| 248 | KSM40-01-0A | Downlink_Phased_array_antenna_working_timeout_Safety_Mode_trigger_count_threshold | 0028010AH |  |  |
| 249 | KSM41-66 | Safety_Mode_Criterion_Table_SAVE_To_FLASH | 002966AAH |  |  |
| 250 | KSM41-88 | Safety_Mode_Criterion_Table_LOAD_From_FLASH | 002988AAH |  |  |
| 251 | KSM41-99 | Safety_Mode_Criterion_Table_ERASE_From_FLASH | 002999AAH |  |  |
| 252 | KSM42-00-66 | UP_TASK_1_START | 002A0066H |  |  |
| 253 | KSM42-01-66 | UP_TASK_2_START | 002A0166H |  |  |
| 254 | KSM42-02-66 | UP_TASK_3_START | 002A0266H |  |  |
| 255 | KSM42-03-66 | UP_TASK_4_START | 002A0366H |  |  |
| 256 | KSM42-00-99 | UP_TASK_1_STOP | 002A0099H |  |  |
| 257 | KSM42-01-99 | UP_TASK_2_STOP | 002A0199H |  |  |
| 258 | KSM42-02-99 | UP_TASK_3_STOP | 002A0299H |  |  |
| 259 | KSM42-03-99 | UP_TASK_4_STOP | 002A0399H |  |  |
| 260 | KSM48-11 | Load_TTC_from_FLASH | 003011AAH |  |  |
| 261 | KSM48-22 | Clear_TTC_in_FLASH | 003022AAH |  |  |
| 262 | KSM49 | Delete_TTC_By_TASK_ID | 0031XXXXH | XXXX：TASK_ID |  |
| 263 | KSM50 | Delete_TTD_By_TASK_ID | 0032XXXXH | XXXX：TASK_ID |  |
| 264 | KSM51 | Delete_TTC_TTD_By_TASK_ID | 0033XXXXH | XXXX：TASK_ID |  |
| 265 | KSM64-66 | reserved | 004066AAH |  |  |
| 266 | KSM64-99 | reserved | 004099AAH |  |  |
| 267 | KSM65-66 | one_wire_power_on | 004166AAH |  |  |
| 268 | KSM65-99 | one_wire_power_off | 004199AAH |  |  |
| 269 | KSM66-66 | on_wire_self_manage_ENABLE | 004266AAH |  |  |
| 270 | KSM66-99 | on_wire_self_manage_DISABLE | 004299AAH |  |  |
| 271 | KSM80-66 | OBC_Data_Recover_ENABLE | 005066AAH |  |  |
| 272 | KSM80-99 | OBC_Data_Recover_DISABLE | 005099AAH |  |  |
| 273 | KSM81-66 | Payload_Task_Protect_Safety_Mode_ENABLE | 005166AAH |  |  |
| 274 | KSM81-99 | Payload_Task_Protect_Safety_Mode_DISABLE | 005199AAH |  |  |
| 275 | KSM82-66 | Payload_Task_Protect_Safety_Mode_Settings_SAVE_to_FLASH | 005266AAH |  |  |
| 276 | KSM82-88 | Payload_Task_Protect_Safety_Mode_Settings_LOAD_from_FLASH | 005288AAH |  |  |
| 277 | KSM82-99 | Payload_Task_Protect_Safety_Mode_Settings_ERASE_from_FLASH | 005299AAH |  |  |
| 278 | KSM112-01 | Transponder_A_POWER_ON | 007001AAH |  |  |
| 280 | KSM112-03 | Transponder_B_POWER_ON | 007003AAH |  |  |
| 282 | KSM112-05 | PCU(A)_ON/(B)_OFF | 007005AAH |  |  |
| 283 | KSM112-06 | PCU(B)_ON/(A)_OFF | 007006AAH |  |  |
| 284 | KSM112-07 | GPS_A_POWER_ON | 007007AAH |  |  |
| 285 | KSM112-08 | GPS_A_POWER_OFF | 007008AAH |  |  |
| 286 | KSM112-09 | TCU_B_POWER_ON | 007009AAH |  |  |
| 287 | KSM112-0A | TCU_B_POWER_OFF | 00700AAAH |  |  |
| 288 | KSM112-0B | TCU_A_POWER_ON | 00700BAAH |  |  |
| 289 | KSM112-0C | TCU_A_POWER_OFF | 00700CAAH |  |  |
| 290 | KSM112-0D | reserved | 00700DAAH |  |  |
| 291 | KSM112-0E | AOCC_B_POWER_ON | 00700EAAH |  |  |
| 292 | KSM112-0F | AOCC_B_POWER_OFF | 00700FAAH |  |  |
| 293 | KSM112-10 | AOCC_RESET | 007010AAH |  |  |
| 294 | KSM112-11 | FT_POWER_ON | 007011AAH |  |  |
| 295 | KSM112-12 | FT_POWER_OFF | 007012AAH |  |  |
| 296 | KSM112-13 | AOCC_A_POWER_ON | 007013AAH |  |  |
| 297 | KSM112-14 | AOCC_A_POWER_OFF | 007014AAH |  |  |
| 298 | KSM112-15 | MC_POWER_ON_STEMP1 | 007015AAH |  |  |
| 299 | KSM112-16 | MC_POWER_ON_STEMP2 | 007016AAH |  |  |
| 300 | KSM112-17 | MC_POWER_OFF_STEMP1 | 007017AAH |  |  |
| 301 | KSM112-18 | MC_POWER_OFF_STEMP2 | 007018AAH |  |  |
| 302 | KSM112-19 | DTS_POWER_ON | 007019AAH |  |  |
| 303 | KSM112-1A | DTS_POWER_OFF | 00701AAAH |  |  |
| 304 | KSM252-FFFF | All_subcomputer_TM_invalid_COUNTER_RESET | 00FCFFFFH |  |  |
| 305 | KSM253-0A | OBC_CAN_A_RESET | 00FD0AAAH |  |  |
| 306 | KSM253-0B | OBC_CAN_B_RESET | 00FD0BAAH |  |  |

TCU(KRK)

Table 24TCU(KRK)Table

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KRK001-66 | TCU_Thermal_Control_Circuits_ENABLE | 070166AAH |  |  |
| 2 | KRK001-99 | TCU_Thermal_Control_Circuits_DISABLE | 070199AAH |  |  |
| 3 | KRK002-01 | HL01_Battery(backup)_Thermal_Control_Mode_SET | 070201XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 4 | KRK002-01-60 | HL01_Battery(backup)_Open_Loop-On | 07020160H |  |  |
| 5 | KRK002-01-A0 | HL01_Battery(backup)_Open_Loop-Off | 070201A0H |  |  |
| 6 | KRK002-01-91 | HL01_Battery(backup)_Close_Loop-Proportional_control-R1 | 07020191H |  |  |
| 7 | KRK002-01-51 | HL01_Battery(backup)_Close_Loop-Threshold_Control-R1 | 07020151H |  |  |
| 8 | KRK002-02 | HL02_phased_array_antenna_(main)_Thermal_Control_Mode_SET | 070202XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 9 | KRK002-02-60 | HL02_phased_array_antenna_(main)_Open_Loop-On | 07020260H |  |  |
| 10 | KRK002-02-A0 | HL02_phased_array_antenna_(main)_Open_Loop-Off | 070202A0H |  |  |
| 11 | KRK002-02-91 | HL02_phased_array_antenna_(main)_Close_Loop-Proportional_control-R1 | 07020291H |  |  |
| 12 | KRK002-02-51 | HL02_phased_array_antenna_(main)_Close_Loop-Threshold_Control-R1 | 07020251H |  |  |
| 13 | KRK002-03 | HL03_phased_array_antenna_(backup)_Thermal_Control_Mode_SET | 070203XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 14 | KRK002-03-60 | HL03_phased_array_antenna_(backup)_Open_Loop-On | 07020360H |  |  |
| 15 | KRK002-03-A0 | HL03_phased_array_antenna_(backup)_Open_Loop-Off | 070203A0H |  |  |
| 16 | KRK002-03-91 | HL03_phased_array_antenna_(backup)_Close_Loop-Proportional_control-R1 | 07020391H |  |  |
| 17 | KRK002-03-51 | HL03_phased_array_antenna_(backup)_Close_Loop-Threshold_Control-R1 | 07020351H |  |  |
| 18 | KRK002-04 | HL04_Thermal_Control_Mode_SET | 070204XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 19 | KRK002-05 | HL05_Thermal_Control_Mode_SET | 070205XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 20 | KRK002-06 | HL06_Thermal_Control_Mode_SET | 070206XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 21 | KRK002-07 | HL07_Thermal_Control_Mode_SET | 070207XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 22 | KRK002-08 | HL08_Thermal_Control_Mode_SET | 070208XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 23 | KRK002-09 | HL09_Thermal_Control_Mode_SET | 070209XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 24 | KRK002-10 | HL10_Thermal_Control_Mode_SET | 07020AXXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 25 | KRK002-11 | HL11_Thermal_Control_Mode_SET | 07020BXXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 26 | KRK002-12 | HL12_Thermal_Control_Mode_SET | 07020CXXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 27 | KRK002-13 | HL13_Sunshield_inside_outer_circumferential_Thermal_Control_Mode_SET | 07020DXXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 28 | KRK002-13-60 | HL13_Sunshield_inside_outer_circumferential_Open_Loop-On | 07020D60H |  |  |
| 29 | KRK002-13-A0 | HL13_Sunshield_inside_outer_circumferential_Open_Loop-Off | 07020DA0H |  |  |
| 30 | KRK002-13-91 | HL13_Sunshield_inside_outer_circumferential_Close_Loop-Proportional_control-R1 | 07020D91H |  |  |
| 31 | KRK002-13-51 | HL13_Sunshield_inside_outer_circumferential_Close_Loop-Threshold_Control-R1 | 07020D51H |  |  |
| 32 | KRK002-14 | HL14_+Y_Skin_of_+Z_Thermal_Control_Mode_SET | 07020EXXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 33 | KRK002-14-60 | HL14_+Y_Skin_of_+Z_Open_Loop-On | 07020E60H |  |  |
| 34 | KRK002-14-A0 | HL14_+Y_Skin_of_+Z_Open_Loop-Off | 07020EA0H |  |  |
| 35 | KRK002-14-91 | HL14_+Y_Skin_of_+Z_Close_Loop-Proportional_control-R1 | 07020E91H |  |  |
| 36 | KRK002-14-51 | HL14_+Y_Skin_of_+Z_Close_Loop-Threshold_Control-R1 | 07020E51H |  |  |
| 37 | KRK002-15 | HL15_+Y_Skin_of_-Z_Thermal_Control_Mode_SET | 07020FXXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 38 | KRK002-15-60 | HL15_+Y_Skin_of_-Z_Open_Loop-On | 07020F60H |  |  |
| 39 | KRK002-15-A0 | HL15_+Y_Skin_of_-Z_Open_Loop-Off | 07020FA0H |  |  |
| 40 | KRK002-15-91 | HL15_+Y_Skin_of_-Z_Close_Loop-Proportional_control-R1 | 07020F91H |  |  |
| 41 | KRK002-15-51 | HL15_+Y_Skin_of_-Z_Close_Loop-Threshold_Control-R1 | 07020F51H |  |  |
| 42 | KRK002-16 | HL16_-Y_Skin_of_-Z_Thermal_Control_Mode_SET | 070210XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 43 | KRK002-16-60 | HL16_-Y_Skin_of_-Z_Open_Loop-On | 07021060H |  |  |
| 44 | KRK002-16-A0 | HL16_-Y_Skin_of_-Z_Open_Loop-Off | 070210A0H |  |  |
| 45 | KRK002-16-91 | HL16_-Y_Skin_of_-Z_Close_Loop-Proportional_control-R1 | 07021091H |  |  |
| 46 | KRK002-16-51 | HL16_-Y_Skin_of_-Z_Close_Loop-Threshold_Control-R1 | 07021051H |  |  |
| 47 | KRK002-17 | HL17_-Y_Skin_of_+Z_Thermal_Control_Mode_SET | 070211XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 48 | KRK002-17-60 | HL17_-Y_Skin_of_+Z_Open_Loop-On | 07021160H |  |  |
| 49 | KRK002-17-A0 | HL17_-Y_Skin_of_+Z_Open_Loop-Off | 070211A0H |  |  |
| 50 | KRK002-17-91 | HL17_-Y_Skin_of_+Z_Close_Loop-Proportional_control-R1 | 07021191H |  |  |
| 51 | KRK002-17-51 | HL17_-Y_Skin_of_+Z_Close_Loop-Threshold_Control-R1 | 07021151H |  |  |
| 52 | KRK002-18 | HL18_Top_of_+Z_frame_Thermal_Control_Mode_SET | 070212XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 53 | KRK002-18-60 | HL18_Top_of_+Z_frame_Open_Loop-On | 07021260H |  |  |
| 54 | KRK002-18-A0 | HL18_Top_of_+Z_frame_Open_Loop-Off | 070212A0H |  |  |
| 55 | KRK002-18-91 | HL18_Top_of_+Z_frame_Close_Loop-Proportional_control-R1 | 07021291H |  |  |
| 56 | KRK002-18-51 | HL18_Top_of_+Z_frame_Close_Loop-Threshold_Control-R1 | 07021251H |  |  |
| 57 | KRK002-19 | HL19_Bottom_of_+Z_frame_Thermal_Control_Mode_SET | 070213XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 58 | KRK002-19-60 | HL19_Bottom_of_+Z_frame_Open_Loop-On | 07021360H |  |  |
| 59 | KRK002-19-A0 | HL19_Bottom_of_+Z_frame_Open_Loop-Off | 070213A0H |  |  |
| 60 | KRK002-19-91 | HL19_Bottom_of_+Z_frame_Close_Loop-Proportional_control-R1 | 07021391H |  |  |
| 61 | KRK002-19-51 | HL19_Bottom_of_+Z_frame_Close_Loop-Threshold_Control-R1 | 07021351H |  |  |
| 62 | KRK002-20 | HL20_Back_Skin_Thermal_Control_Mode_SET | 070214XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 63 | KRK002-20-60 | HL20_Back_Skin_Open_Loop-On | 07021460H |  |  |
| 64 | KRK002-20-A0 | HL20_Back_Skin_Open_Loop-Off | 070214A0H |  |  |
| 65 | KRK002-20-91 | HL20_Back_Skin_Close_Loop-Proportional_control-R1 | 07021491H |  |  |
| 66 | KRK002-20-51 | HL20_Back_Skin_Close_Loop-Threshold_Control-R1 | 07021451H |  |  |
| 67 | KRK002-21 | HL21_Upper_of_Back_Skin_Thermal_Control_Mode_SET | 070215XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 68 | KRK002-21-60 | HL21_Upper_of_Back_Skin_Open_Loop-On | 07021560H |  |  |
| 69 | KRK002-21-A0 | HL21_Upper_of_Back_Skin_Open_Loop-Off | 070215A0H |  |  |
| 70 | KRK002-21-91 | HL21_Upper_of_Back_Skin_Close_Loop-Proportional_control-R1 | 07021591H |  |  |
| 71 | KRK002-21-51 | HL21_Upper_of_Back_Skin_Close_Loop-Threshold_Control-R1 | 07021551H |  |  |
| 72 | KRK002-22 | HL22_Bottom_of_Back_Skin_Thermal_Control_Mode_SET | 070216XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 73 | KRK002-22-60 | HL22_Bottom_of_Back_Skin_Open_Loop-On | 07021660H |  |  |
| 74 | KRK002-22-A0 | HL22_Bottom_of_Back_Skin_Open_Loop-Off | 070216A0H |  |  |
| 75 | KRK002-22-91 | HL22_Bottom_of_Back_Skin_Close_Loop-Proportional_control-R1 | 07021691H |  |  |
| 76 | KRK002-22-51 | HL22_Bottom_of_Back_Skin_Close_Loop-Threshold_Control-R1 | 07021651H |  |  |
| 77 | KRK002-23 | HL23_Focal_plane_splice_lens_frame_Thermal_Control_Mode_SET | 070217XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 78 | KRK002-23-60 | HL23_Focal_plane_splice_lens_frame_Open_Loop-On | 07021760H |  |  |
| 79 | KRK002-23-A0 | HL23_Focal_plane_splice_lens_frame_Open_Loop-Off | 070217A0H |  |  |
| 80 | KRK002-23-91 | HL23_Focal_plane_splice_lens_frame_Close_Loop-Proportional_control-R1 | 07021791H |  |  |
| 81 | KRK002-23-51 | HL23_Focal_plane_splice_lens_frame_Close_Loop-Threshold_Control-R1 | 07021751H |  |  |
| 82 | KRK002-24 | HL24_Focal_circuit_Inside_frame_Thermal_Control_Mode_SET | 070218XXH | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional-Min;0x91:Close_Loop-Proportional-R1;0x92:Close_Loop-Proportional-R2;0x50:Close_Loop-Threshold-Min;0x51:Close_Loop-Threshold-R1;0x52:Close_Loop-Threshold-R2 |  |
| 83 | KRK002-24-60 | HL24_Focal_circuit_Inside_frame_Open_Loop-On | 07021860H |  |  |
| 84 | KRK002-24-A0 | HL24_Focal_circuit_Inside_frame_Open_Loop-Off | 070218A0H |  |  |
| 85 | KRK002-24-91 | HL24_Focal_circuit_Inside_frame_Close_Loop-Proportional_control-R1 | 07021891H |  |  |
| 86 | KRK002-24-51 | HL24_Focal_circuit_Inside_frame_Close_Loop-Threshold_Control-R1 | 07021851H |  |  |
| 87 | KRK005-66 | TCU_Settings_SAVE_to_FLASH | 070566AAH |  |  |
| 88 | KRK005-99 | TCU_Settings_ERASE_from_FLASH | 070599AAH |  |  |
| 89 | KRK006 | reserved | 0706XXYYH |  |  |
| 90 | KRK007 | reserved | 0707XXYYH |  |  |
| 91 | KRK008 | reserved | 0708XXYYH |  |  |
| 92 | KRK015 | reserved | 070FXXAAH |  |  |
| 93 | KRK016 | reserved | 0710AAAA |  |  |
| 94 | KRK253-0A | TCU_CAN_A_RESET | 07FD0AAAH |  |  |
| 95 | KRK253-0B | TCU_CAN_B_RESET | 07FD0BAAH |  |  |

TransponderA(KCA)

Table 25TransponderA(KCA)Table

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KCA001 | Discharge switch on | 0201AAAAH | Discharge switch on |  |
| 3 | KCA003 | OBDH main on and standby off | 0203AAAAH | OBDH main on and standby off |  |
| 4 | KCA004 | OBDH standby on and main off | 0204AAAAH | OBDH standby on and main off |  |
| 5 | KCA005 | Power MCU main on and standby off | 0205AAAAH | Power MCU main on and standby off |  |
| 6 | KCA006 | Power MCU standby on and main off | 0206AAAAH | Power MCU standby on and main off |  |
| 7 | KCA007 | Reserved | 0207AAAAH | Reserved |  |
| 9 | KCA009 | Transmitter of USB A on | 0209AAAAH | Transmitter of USB A on |  |
| 10 | KCA010 | Transmitter of USB A off | 020AAAAAH | Transmitter of USB A off |  |
| 11 | KCA011 | Transponder A coherent in work | 020BAAAAH | Transponder A coherent in work |  |
| 12 | KCA012 | Transponder A incoherent in work | 020CAAAAH | Transponder A incoherent in work |  |
| 13 | KCA013 | Transponder A ranging on | 020DAAAAH | Transponder A ranging on |  |
| 14 | KCA014 | Transponder A ranging off | 020EAAAAH | Transponder A ranging off |  |
| 15 | KCA015 | CAN A bus of USB A resetting | 020FAAAAH | CAN A bus resetting counter increase 1 |  |
| 16 | KCA016 | CAN B bus of USB A resetting | 0210AAAAH | CAN B bus resetting counter increase 1 |  |
| 17 | KCA017 | Reserved | 0211AAAAH | / |  |

TransponderB(KCB)

Table 26TransponderB(KCB)Table

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KCB001 | Discharge switch on | 0501AAAAH | Discharge switch on |  |
| 3 | KCB003 | OBDH main on and standby off | 0503AAAAH | OBDH main on and standby off |  |
| 4 | KCB004 | OBDH standby on and main off | 0504AAAAH | OBDH standby on and main off |  |
| 5 | KCB005 | Power MCU main on and standby off | 0505AAAAH | Power MCU main on and standby off |  |
| 6 | KCB006 | Power MCU standby on and main off | 0506AAAAH | Power MCU standby on and main off |  |
| 7 | KCB007 | Reserved | 0507AAAAH | Reserved |  |
| 9 | KCB009 | Transmitter of USB B on | 0509AAAAH | Transmitter of USB B on |  |
| 10 | KCB010 | Transmitter of USB B off | 050AAAAAH | Transmitter of USB B off |  |
| 11 | KCB011 | Transponder B coherent in work | 050BAAAAH | Transponder B coherent in work |  |
| 12 | KCB012 | Transponder B incoherent in work | 050CAAAAH | Transponder B incoherent in work |  |
| 13 | KCB013 | Transponder B ranging on | 050DAAAAH | Transponder B ranging on |  |
| 14 | KCB014 | Transponder B ranging off | 050EAAAAH | Transponder B ranging off |  |
| 15 | KCB015 | CAN A bus of USB B resetting | 050FAAAAH | CAN A bus resetting counter increase 1 |  |
| 16 | KCB016 | CAN B bus of USB B resetting | 0510AAAAH | CAN B bus resetting counter increase 1 |  |
| 17 | KCB017 | Reserved | 0511AAAAH | / |  |

GPS(KGS)

Table 27GPS(KGS)Table

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KGS001 | GPS receiver resetting | 0A00AAAA | The system initialization |  |
| 2 | KGS002 | Navigation orientation software resetting | 0A01AAAA | Slave core resetting counter increase 1 |  |
| 3 | KGS003 | Navigation orientation software upload permission | 0A02AAAA | Navigation orientation software upload permission |  |
| 4 | KGS004 | Navigation orientation software upload ending | 0A03AAAA | Navigation orientation software upload ending |  |
| 5 | KGS005 | Navigation orientation software upload startup | 0A04AAAA | Navigation orientation software upload startup |  |
| 6 | KGS006 | Orbit determination software upload permission | 0A05AAAA | Orbit determination software upload permission |  |
| 7 | KGS007 | Orbit determination software upload ending | 0A06AAAA | Orbit determination software upload ending |  |
| 8 | KGS008 | Orbit determination software upload startup | 0A07AAAA | Orbit determination software upload startup |  |
| 9 | KGS009 | Allowing 1pps output | 0A08AAAA | Allowing 1pps output |  |
| 10 | KGS010 | Forbidding 1pps output | 0A09AAAA | Forbidding 1pps output |  |
| 11 | KGS011 | Time broadcast at CAN bus A | 0A0AAAAA | The CAN bus used currently：“ CAN bus A ” |  |
| 12 | KGS012 | Time broadcast at CAN bus B | 0A0BAAAA | The CAN bus used currently：“ CAN bus B ” |  |
| 13 | KGS013 | Orbit determination data output permission | 0A0CAAAA | orbit determination data output marking：“permission” |  |
| 14 | KGS014 | Orbit determination data output forbidden | 0A0DAAAA | orbit determination data output marking：“forbidden” |  |
| 15 | KGS015 | CAN A bus resetting | 0A0EAAAA | Bus A reset counter increase 1 |  |
| 16 | KGS016 | CAN B bus resetting | 0A0FAAAA | Bus B reset counter increase 1 |  |

PCU(KNY)

Table 28PCU(KNY)Table

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KNY03001 | Transponder 1 Power on | 0301AAAA |  |  |
| 3 | KNY03003 | GPS-B power on | 0303AAAA |  |  |
| 4 | KNY03004 | BMU Power on | 0304AAAA |  |  |
| 6 | KNY03006 | Memory alloy Power off | 0306AAAA |  |  |
| 7 | KNY03007 | Solar panel release power off | 0307AAAA |  |  |
| 9 | KNY03009 | BMU Power off | 0309AAAA |  |  |
| 10 | KNY03010 | Reserved | 030BAAAA |  |  |
| 11 | KNY03011 | OBC switch B to A | 030CAAAA |  |  |
| 12 | KNY03012 | OBC switch A to B | 030DAAAA |  |  |
| 13 | KNY03013 | Discharge switch on | 030EAAAA |  |  |
| 15 | KNY03015 | Transponder 2 Power on | 0310AAAA |  |  |
| 17 | KNY03017 | Overdischarge protection enable | 0312AAAA |  |  |
| 18 | KNY03018 | Overdischarge protection disenable | 0313AAAA |  |  |
| 19 | KNY03019 | Battery reconnection enable | 0314AAAA |  |  |
| 20 | KNY03020 | Battery reconnection disenable | 0315AAAA |  |  |
| 21 | KNY03021 | FLASH erasion disenable | 0319AAAA |  |  |
| 22 | KNY03022 | FLASH erasion enable | 031AAAAA |  |  |
| 23 | KNY03023 | FLASH erase | 031BAAAA |  |  |
| 24 | KNY03024 | Key data storage | 031CAAAA |  |  |
| 25 | KNY03025 | Uploaded task on | 031D66AA |  |  |
| 26 | KNY03026 | Uploaded power task on | 031E66AA |  |  |
| 27 | KNY03027 | Uploaded CAN task on | 031F66AA |  |  |
| 28 | KNY03028 | Uploaded OC task on | 032066AA |  |  |
| 29 | KNY03029 | OBC reset enable | 0321AAAA |  |  |
| 30 | KNY03030 | OBC reset disenable | 0322AAAA |  |  |
| 31 | KNY03031 | OBC reset status clear | 0323AAAA |  |  |
| 32 | KNY03032 | Safe mode send unlock disenable | 0326AAAA |  |  |
| 33 | KNY03033 | Safe mode send unlock enable | 0327AAAA |  |  |
| 34 | KNY03034 | CAN bus reset | 032AAAAA |  |  |

BMU(KNB)

Table 29BMU(KNB)Table

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KNB07001 | Balance control 1 on | 0801AAAA |  |  |
| 2 | KNB07002 | Balance control 2 on | 0802AAAA |  |  |
| 3 | KNB07003 | Balance control 3 on | 0803AAAA |  |  |
| 4 | KNB07004 | Balance control 4 on | 0804AAAA |  |  |
| 5 | KNB07005 | Balance control 5 on | 0805AAAA |  |  |
| 6 | KNB07006 | Balance control 6 on | 0806AAAA |  |  |
| 7 | KNB07007 | Balance control 7 on | 0807AAAA |  |  |
| 8 | KNB07008 | Self-balance control enable | 0811AAAA |  |  |
| 9 | KNB07009 | Self-balance control disenable | 0812AAAA |  |  |
| 10 | KNB07010 | FLASH erasion disenable | 0813AAAA |  |  |
| 11 | KNB07011 | FLASH erasion enable | 0814AAAA |  |  |
| 12 | KNB07012 | Uploaded task on | 0815AAAA |  |  |
| 13 | KNB07013 | Uploaded power task on | 0816AAAA |  |  |
| 14 | KNB07014 | Uploaded CAN task on | 0817AAAA |  |  |
| 15 | KNB07015 | Uploaded OC task on | 0818AAAA |  |  |
| 16 | KNB07016 | CANA bus reset | 0819AAAA |  |  |
| 17 | KNB07017 | FLASH erase | 081AAAAA |  |  |
| 18 | KNB07018 | CANB bus reset | 081BAAAA |  |  |

MC(KHC)

Table 30MC(KHC)Table

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KHC001 | MC B1 gain setting | 0C01XXXXH | B1 Gain,10bit | 0H～1FFH |
| 2 | KHC002 | MC B2 gain setting | 0C02XXXXH | B2Gain,10bit | 0H～1FF |
| 3 | KHC003 | MC B3 gain setting | 0C03XXXXH | B3 Gain,10bit | 0H～1FF |
| 4 | KHC004 | MC B4 gain setting | 0C04XXXXH | B4 Gain,10bit | 0H～1FF |
| 5 | KHC005 | MC Integration Time Settings | 0C05XXXXH | Low 6bit,availableln 1.8~2.7ms，step 0.1ms | 01H~0AH |
| 6 | KHC006 | MC B1 Exposure time Settings | 0C06XXXXH | Low 6bit,availableln 1.5~2.5ms，step 0.1ms | 01H~0BH |
| 7 | KHC007 | MC B2Exposure time Settings | 0C07XXXXH | Low 6bit,availableln 1.5~2.5ms，step 0.1ms | 01H~0BH |
| 8 | KHC008 | MC B3 Exposure time Settings | 0C08XXXXH | Low 6bit,availableln 1.5~2.5ms，step 0.1ms | 01H~0BH |
| 9 | KHC009 | MC B4 Exposure time Settings | 0C09XXXXH | Low 6bit,availableln 1.5~2.5ms，step 0.1ms | 01H~0BH |
| 10 | KHC010 | MC Algorithm Settings | 0C0AXXXXH | 01：on，10：off | 01,10 |
| 11 | KHC011 | MC reset CANA | 0C0BAAAAH | reset CANA | CANA |
| 12 | KHC012 | MC reset CANB | 0C0CAAAAH | reset CANA | CANB |

AOCC(KZK)

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KZK001 | Run the Global Attitude Acquisition Mode(AAM) | 0401AAAAH |  |  |
| 2 | KZK002 | Run the Sun Pointing Mode(SPM) | 0402AAAAH |  |  |
| 3 | KZK003 | Run the Earth Pointing Mode(EPM) | 0403AAAAH |  |  |
| 4 | KZK004 | Run the Stop Control Mode(SCM) | 0404AAAAH |  |  |
| 5 | KZK005 | Run the Capture-earth command | 0405AAAAH |  |  |
| 6 | KZK006 | Reserved | 0406AAAAH |  |  |
| 7 | KZK007 | Rate damping using MW damping | 0407AAAAH | Only used in AAM |  |
| 8 | KZK008 | Rate damping is damped using a magnetic torque | 0408AAAAH | Only used in AAM |  |
| 9 | KZK009 | GPS orbit calculation is introduced | 0409AAAAH |  |  |
| 10 | KZK010 | Remove GPS orbit calculation | 040AAAAAH |  |  |
| 11 | KZK011 | GPS data validation | 040BAAAAH |  |  |
| 12 | KZK012 | Exit GPS data validation | 040CAAAAH |  |  |
| 13 | KZK013 | Mandatory use of GPS orbit | 040DAAAAH |  |  |
| 14 | KZK014 | Mandatory use of GPS orbit is prohibited | 040EAAAAH |  |  |
| 15 | KZK015 | MW unloading during imaging is not allowed | 040FAAAAH |  |  |
| 16 | KZK016 | Allows the MW to unload during imaging | 0410AAAAH |  |  |
| 17 | KZK017 | High current low speed start of MW is not allowed | 0411AAAAH |  |  |
| 18 | KZK018 | Allow the MW to start at low speed with high current | 0412AAAAH |  |  |
| 19 | KZK019 | The integrator clears | 0413AAAAH |  |  |
| 20 | KZK020 | Reserved | 0414AAAAH |  |  |
| 21 | KZK021 | Reserved | 0415AAAAH |  |  |
| 22 | KZK022 | Magnetic damping is measured using a gyroscope | 0416AAAAH |  |  |
| 23 | KZK023 | Magnetic damping is measured using a magnetometer | 0417AAAAH |  |  |
| 24 | KZK024 | Reserved | 0418AAAAH |  |  |
| 25 | KZK025 | Reserved | 0419AAAAH |  |  |
| 26 | KZK026 | Gyro autonomous diagnosis is not allowed | 041AAAAAH |  |  |
| 27 | KZK027 | Allows the Gyro to diagnose autonomously | 041BAAAAH |  |  |
| 28 | KZK028 | Magnetometer diagnosis is not allowed | 041CAAAAH |  |  |
| 29 | KZK029 | Allows the magnetometer to diagnose itself | 041DAAAAH |  |  |
| 30 | KZK030 | The STS is not allowed to diagnose autonomously | 041EAAAAH |  |  |
| 31 | KZK031 | Allow the STS to diagnose autonomously | 041FAAAAH |  |  |
| 32 | KZK032 | MW autonomous diagnosis is not allowed | 0420AAAAH |  |  |
| 33 | KZK033 | Allows the MW to diagnose autonomously | 0421AAAAH |  |  |
| 34 | KZK034 | Allows automatic switching of pose setting mode | 0422AAAAH |  |  |
| 35 | KZK035 | The attitude is reset by using double vector | 0423AAAAH |  |  |
| 36 | KZK036 | The magnetometer is used for quaternion filtering | 0424AAAAH |  |  |
| 37 | KZK037 | Quaternion filtering is performed by using dual vectors | 0425AAAAH |  |  |
| 38 | KZK038 | Automatic switching of pose setting mode is not allowed | 0426AAAAH |  |  |
| 39 | KZK039 | Use star - sensitive gyroscope to determine the attitude | 0427AAAAH |  |  |
| 40 | KZK040 | Use a magnetometer to determine the pose | 0428AAAAH |  |  |
| 41 | KZK041 | Use sun sensor to monitor attitude to the sun | 0429AAAAH |  |  |
| 42 | KZK042 | Do not use sun sensor to monitor attitude to the sun | 042AAAAAH |  |  |
| 43 | KZK043 | Double digit is sun sensor all normal | 042BAAAAH |  |  |
| 44 | KZK044 | Only the number sun sensor 1 is normal | 042CAAAAH |  |  |
| 45 | KZK045 | Only the number sun sensor 2 is normal | 042DAAAAH |  |  |
| 46 | KZK046 | Dual digital sun sensor fault | 042EAAAAH |  |  |
| 47 | KZK047 | All gyros were normal | 042FAAAAH |  |  |
| 48 | KZK048 | Remove the x gyro (use y, z, s) | 0430AAAAH |  |  |
| 49 | KZK049 | Remove y gyro (use x, z, s) | 0431AAAAH |  |  |
| 50 | KZK050 | Remove z gyro (use x, y, s) | 0432AAAAH |  |  |
| 51 | KZK051 | Remove s gyro (use x, y, z) | 0433AAAAH |  |  |
| 52 | KZK052 | All four MWs XYZS are healthy | 0434AAAFH |  |  |
| 53 | KZK053 | X wheel failure other wheels are healthy | 0434AAAEH |  |  |
| 54 | KZK054 | Wheel Y failure other wheels are healthy | 0434AAADH |  |  |
| 55 | KZK055 | Z wheel failure other wheels are healthy | 0434AAABH |  |  |
| 56 | KZK056 | S wheel failure other wheels are healthy | 0434AAA7H |  |  |
| 57 | KZK057 | Four MW operating instructions | 0435AAAAH |  |  |
| 58 | KZK058 | Y,z,s work instructions | 0436AAAAH |  |  |
| 59 | KZK059 | X,z,s work instructions | 0437AAAAH |  |  |
| 60 | KZK060 | X,y,s work instructions | 0438AAAAH |  |  |
| 61 | KZK061 | X,y,z work instructions | 0439AAAAH |  |  |
| 62 | KZK062 | Reserved | 043AAAAAH |  |  |
| 63 | KZK063 | Dual STS operating instructions | 043BAAAAH |  |  |
| 64 | KZK064 | Only micro STSA operating instruction (+X side) | 043CAAAAH |  |  |
| 65 | KZK065 | Only micro STS B operating instructions (-x side) | 043DAAAAH |  |  |
| 66 | KZK066 | Dual STSs are not operating instructions | 043EAAAAH |  |  |
| 67 | KZK067 | Reserved | 043FAAAAH |  |  |
| 68 | KZK068 | Reserved | 0440AAAAH |  |  |
| 69 | KZK069 | Use STS without Gyro to determine the attitude | 0441AAAAH |  |  |
| 70 | KZK070 | Reserved | 0442AAAAH |  |  |
| 71 | KZK071 | Full capture to ground orientation with STS | 0443AAAAH |  |  |
| 72 | KZK072 | Full capture to ground orientation with magnetometer | 0444AAAAH |  |  |
| 73 | KZK073 | Directly adjust the attitude and enter the cruise to sun instruction | 0445AAAAH |  |  |
| 74 | KZK074 | Magnetic unloading using magnetometers | 0446AAAAH |  |  |
| 75 | KZK075 | Magnetic unloading using orbital calculations | 0447AAAAH |  |  |
| 76 | KZK076 | MW unloading during solar orientation is not allowed | 0448AAAAH |  |  |
| 77 | KZK077 | Allows the MW to unload during solar orientation | 0449AAAAH |  |  |
| 78 | KZK078 | full capture adjust attitude to the sun with STS | 044AAAAAH |  |  |
| 79 | KZK079 | full capture adjust attitude to the sun with magnetometer | 044BAAAAH |  |  |
| 80 | KZK080 | Deflection Angle is not introduced in load imaging | 044CAAAAH |  |  |
| 81 | KZK081 | Deflection Angle is introduced in load imaging | 044DAAAAH |  |  |
| 82 | KZK082 | Enable SPM/EPM auto-switch | 044EAAAAH |  |  |
| 83 | KZK083 | Disable SPM/EPM auto-switch | 044FAAAAH |  |  |
| 84 | KZK084 | AOCC is not allowed to correct the aberration of STS A | 0450AAAAH |  |  |
| 85 | KZK085 | AOCC is allowed to correct the aberration of STS A | 0451AAAAH |  |  |
| 86 | KZK086 | AOCC is not allowed to correct the aberration of STS B | 0452AAAAH |  |  |
| 87 | KZK087 | AOCC is allowed to correct the aberration of STS B | 0453AAAAH |  |  |
| 88 | KZK088 | Yaw calibration 90° command | 0454AAAAH |  |  |
| 89 | KZK089 | Yaw calibration -90° command | 0455AAAAH |  |  |
| 90 | KZK090 | Long term in the orientation attitude to the sun | 0456AAAAH |  |  |
| 91 | KZK091 | Long term in the orientation attitude to the ground | 0457AAAAH |  |  |
| 92 | KZK092 | Reserved | 0458AAAAH |  |  |
| 93 | KZK093 | Reserved | 0459AAAAH |  |  |
| 94 | KZK094 | Reserved | 045AAAAAH |  |  |
| 95 | KZK095 | Reserved | 045BAAAAH |  |  |
| 96 | KZK096 | Bias-flight command | 045CAAAAH |  |  |
| 97 | KZK097 | Bias-flight return command | 045DAAAAH |  |  |
| 98 | KZK098 | Reserved | 045EAAAAH |  |  |
| 99 | KZK099 | Reserved | 045FAAAAH |  |  |
| 100 | KZK100 | MW X power up | 0460AAAAH |  |  |
| 101 | KZK101 | MW X power off | 0461AAAAH |  |  |
| 102 | KZK102 | MW Y power up | 0462AAAAH |  |  |
| 103 | KZK103 | MW Y power off | 0463AAAAH |  |  |
| 104 | KZK104 | MW Z power up | 0464AAAAH |  |  |
| 105 | KZK105 | MW Z power off | 0465AAAAH |  |  |
| 106 | KZK106 | MW S power up | 0466AAAAH |  |  |
| 107 | KZK107 | MW S power off | 0467AAAAH |  |  |
| 108 | KZK108 | Magnetic torque X power up | 0468AAAAH |  |  |
| 109 | KZK109 | Magnetic torque X power off | 0469AAAAH |  |  |
| 110 | KZK110 | Magnetic torque Y power up | 046AAAAAH |  |  |
| 111 | KZK111 | Magnetic torque Y power off | 046BAAAAH |  |  |
| 112 | KZK112 | Magnetic torque Z power up | 046CAAAAH |  |  |
| 113 | KZK113 | Magnetic torque Z power off | 046DAAAAH |  |  |
| 114 | KZK114 | Gyro main light source power up | 046EAAAAH |  |  |
| 115 | KZK115 | Gyro main light source power off | 046FAAAAH |  |  |
| 116 | KZK116 | Gyro backup light source power up | 0470AAAAH |  |  |
| 117 | KZK117 | Gyro backup light source power off | 0471AAAAH |  |  |
| 118 | KZK118 | Fiber-optic gyro X power up | 0472AAAAH |  |  |
| 119 | KZK119 | Fiber-optic gyro X power off | 0473AAAAH |  |  |
| 120 | KZK120 | Fiber optic gyro Y power up | 0474AAAAH |  |  |
| 121 | KZK121 | Fiber optic gyro Y power off | 0475AAAAH |  |  |
| 122 | KZK122 | Fiber-optic gyro Z power up | 0476AAAAH |  |  |
| 123 | KZK123 | Fiber-optic gyro Z power off | 0477AAAAH |  |  |
| 124 | KZK124 | Fiber optic gyro S power up | 0478AAAAH |  |  |
| 125 | KZK125 | Fiber optic gyro S power off | 0479AAAAH |  |  |
| 126 | KZK126 | Magnetometer A power up | 047AAAAAH |  |  |
| 127 | KZK127 | Magnetometer A power off | 047BAAAAH |  |  |
| 128 | KZK128 | Magnetometer B power up | 047CAAAAH |  |  |
| 129 | KZK129 | Magnetometer B power off | 047DAAAAH |  |  |
| 130 | KZK130 | STS A power up | 047EAAAAH |  |  |
| 131 | KZK131 | STS A power off | 047FAAAAH |  |  |
| 132 | KZK132 | STS B power up | 0480AAAAH |  |  |
| 133 | KZK133 | STS B power off | 0481AAAAH |  |  |
| 134 | KZK134 | Allow AOCC communication to switch autonomously | 0482AAAAH |  |  |
| 135 | KZK135 | Forbidden AOCC communication to switch autonomously | 0483AAAAH |  |  |
| 136 | KZK136 | Chip's sun sensor power up | 0484AAAAH |  |  |
| 137 | KZK137 | Chip's sun sensor power off | 0485AAAAH |  |  |
| 138 | KZK138 | Reserved | 0486AAAAH |  |  |
| 139 | KZK139 | Reserved | 0487AAAAH |  |  |
| 140 | KZK140 | Reserved | 0488AAAAH |  |  |
| 141 | KZK141 | Allow STS adjust attitude to the sun directly | 0489AAAAH |  |  |
| 142 | KZK142 | Forbidden STS adjust attitude to the sun directly | 048AAAAAH |  |  |
| 143 | KZK143 | Allow update four elements of attitude by STS | 048BAAAAH |  |  |
| 144 | KZK144 | Forbidden update four elements of attitude by STS | 048CAAAAH |  |  |
| 145 | KZK145 | On-chip sun sensor operating mode is set in orbit | 048DXXYYH |  |  |
| 146 | KZK146 | AOCC S-L Separation Time calibration | 048EAAAAH |  |  |
| 147 | KZK147 | AOCC UTC time calibration | 048FAAAAH |  |  |
| 148 | KZK148 | Forbidden time calibration for satellite | 0490AAAAH |  |  |
| 149 | KZK149 | Introduce second pulse for time calibration | 0491AAAAH |  |  |
| 150 | KZK150 | Reserved | 0492AAAAH |  |  |
| 151 | KZK151 | Allow adjust time | 0493AAAAH |  |  |
| 152 | KZK152 | Forbidden adjust time | 0494AAAAH |  |  |
| 153 | KZK153 | Allow calculation satellite time error | 0495AAAAH |  |  |
| 154 | KZK154 | Forbidden calcultion satellite time error | 0496AAAAH |  |  |
| 155 | KZK155 | System detection degradation | 0497AAAAH |  |  |
| 156 | KZK156 | System detection recovery | 0498AAAAH |  |  |
| 157 | KZK157 | Auto switch enable | 0499AAAAH |  |  |
| 158 | KZK158 | Auto switch disable | 049AAAAAH |  |  |
| 159 | KZK159 | Start feature parameter save | 049BAAAAH |  |  |
| 160 | KZK160 | Disable feature parameter save | 049CAAAAH |  |  |
| 161 | KZK161 | Prohibit running any TTC | 049DAAAAH |  |  |
| 162 | KZK162 | FLASH repair enable | 049EAAAAH |  |  |
| 163 | KZK163 | Flash repair disabled | 049FAAAAH |  |  |
| 164 | KZK164 | FLASH self-test enable | 04A0AAAAH |  |  |
| 165 | KZK165 | FLASH self-test disable | 04A1AAAAH |  |  |
| 166 | KZK166 | Gyro signal data from real component | 04A2AAAAH |  |  |
| 167 | KZK167 | Gyro signal data comes from ground inspection serial port | 04A3AAAAH |  |  |
| 168 | KZK168 | Magnetometer signal data from real component | 04A4AAAAH |  |  |
| 169 | KZK169 | Magnetometer signal data from the ground inspection serial port | 04A5AAAAH |  |  |
| 170 | KZK170 | Solar signal data from real component | 04A6AAAAH |  |  |
| 171 | KZK171 | Solar signal data comes from the ground inspection serial port | 04A7AAAAH |  |  |
| 172 | KZK172 | STS signal data from real component | 04A8AAAAH |  |  |
| 173 | KZK173 | STS signal data comes from the ground inspection serial port | 04A9AAAAH |  |  |
| 174 | KZK174 | Allow fast TM 1 download | 04AAAAAAH |  |  |
| 175 | KZK175 | Disable fast TM 1 download | 04ABAAAAH |  |  |
| 176 | KZK176 | Allow fast TM 2 download | 04ACAAAAH |  |  |
| 177 | KZK177 | Disable fast TM 2 download | 04ADAAAAH |  |  |
| 178 | KZK178 | Allow slow TM 1 download | 04AEAAAAH |  |  |
| 179 | KZK179 | Disable slow TM 1 download | 04AFAAAAH |  |  |
| 180 | KZK180 | Allow slow TM 2 download | 04B0AAAAH |  |  |
| 181 | KZK181 | Disable slow TM 2 download | 04B1AAAAH |  |  |
| 182 | KZK182 | Enable kinetic data download | 04B2AAAAH |  |  |
| 183 | KZK183 | Downloading kinetic data is prohibited | 04B3AAAAH |  |  |
| 184 | KZK184 | Set machine abnormality | 04B4AAAAH |  |  |
| 185 | KZK185 | Cancel the machine abnormality | 04B5AAAAH |  |  |
| 186 | KZK186 | Magnetometer A operating mode is set on orbit | 04B6XXYYH |  |  |
| 187 | KZK187 | Magnetometer B operating mode is set on orbit | 04B7XXYYH |  |  |
| 188 | KZK188 | STS A CMD_RESET | 04B8AAAAH |  |  |
| 189 | KZK189 | STS B CMD_RESET | 04B9AAAAH |  |  |
| 190 | KZK190 | STS A extract data forbidden command | 04BAAAAAH |  |  |
| 191 | KZK191 | STS B extract data forbidden command | 04BBAAAAH |  |  |
| 192 | KZK192 | STS A extract data recovery command | 04BCAAAAH |  |  |
| 193 | KZK193 | STS B extract data recovery command | 04BDAAAAH |  |  |
| 194 | KZK194 | STS A goes into standby mode | 04BEAAAAH |  |  |
| 195 | KZK195 | STS B goes into standby mode | 04BFAAAAH |  |  |
| 196 | KZK196 | STS A enters mode B | 04C0AAAAH |  |  |
| 197 | KZK197 | STS B enters mode B | 04C1AAAAH |  |  |
| 198 | KZK198 | STS A synchronous pulse reset | 04C2AAAAH |  |  |
| 199 | KZK199 | STS B synchronous pulse reset | 04C3AAAAH |  |  |
| 200 | KZK200 | STS A CMD_RESTART | 04C4AAAAH |  |  |
| 201 | KZK201 | STS B CMD_RESTART | 04C5AAAAH |  |  |
| 202 | KZK202 | STS A mode control | 04C6AAAAH |  |  |
| 203 | KZK203 | STS B mode control | 04C7AAAAH |  |  |
| 204 | KZK204 | STS A parameter setting | 04C8AAAAH |  |  |
| 205 | KZK205 | STS B parameter setting | 04C9AAAAH |  |  |
| 206 | KZK206 | Reserved | 04CAAAAAH |  |  |
| 207 | KZK207 | Reserved | 04CBAAAAH |  |  |
| 208 | KZK208 | MW X working mode is set in orbit | 04CCXXYYH |  |  |
| 209 | KZK209 | MW Y working mode is set in orbit | 04CDXXYYH |  |  |
| 210 | KZK210 | MW Z working mode is set in orbit | 04CEXXYYH |  |  |
| 211 | KZK211 | MW S working mode is set in orbit | 04CFXXYYH |  |  |
| 212 | KZK212 | Error calculation for STS installation is allowed | 04D0AAAAH |  |  |
| 213 | KZK213 | Error calculation for STS installation is Not allowed | 04D1AAAAH |  |  |
| 214 | KZK214 | Allow STS installation deviation correction | 04D2AAAAH |  |  |
| 215 | KZK215 | Forbidden STS installation deviation correction | 04D3AAAAH |  |  |
| 216 | KZK216 | MW speed data from the real component | 04D4AAAAH |  |  |
| 217 | KZK217 | MW speed data from the ground inspection serial port | 04D5AAAAH |  |  |
| 218 | KZK218 | Checksum calculation after STS program | 04D6AAAAH |  |  |
| 219 | KZK219 | Reserved | 04D7AAAAH |  |  |
| 220 | KZK220 | Allow STS A power on/off automatically | 04D8AAAAH |  |  |
| 221 | KZK221 | Forbidden STS A power on/off automatically | 04D9AAAAH |  |  |
| 222 | KZK222 | Allow STS B power on/off automatically | 04DAAAAAH |  |  |
| 223 | KZK223 | Forbidden STS B power on/off automatically | 04DBAAAAH |  |  |
| 224 | KZK224 | Modification of reversing parameters of magnetic torquer | 04DCAAAAH |  |  |
| 225 | KZK225 | Reserved | 04DDAAAAH |  |  |
| 226 | KZK226 | Allow chip's sun sensor power on/off automatically | 04DEAAAAH |  |  |
| 227 | KZK227 | Forbidden chip's sun sensor power on/off automatically | 04DFAAAAH |  |  |
| 228 | KZK228 | Allow magnetometer A power on/off automatically | 04E0AAAAH |  |  |
| 229 | KZK229 | Forbidden magnetometer A power on/off automatically | 04E1AAAAH |  |  |
| 230 | KZK230 | Allow magnetometer A power on/off automatically | 04E2AAAAH |  |  |
| 231 | KZK231 | Forbidden magnetometer A power on/off automatically | 04E3AAAAH |  |  |
| 232 | KZK232 | Allow Fiber optic gyro X power on/off automatically | 04E4AAAAH |  |  |
| 233 | KZK233 | Forbidden Fiber optic gyro X power on/off automatically | 04E5AAAAH |  |  |
| 234 | KZK234 | Allow Fiber optic gyro Y power on/off automatically | 04E6AAAAH |  |  |
| 235 | KZK235 | Forbidden Fiber optic gyro Y power on/off automatically | 04E7AAAAH |  |  |
| 236 | KZK236 | Allow Fiber optic gyro Z power on/off automatically | 04E8AAAAH |  |  |
| 237 | KZK237 | Forbidden Fiber optic gyro Z power on/off automatically | 04E9AAAAH |  |  |
| 238 | KZK238 | Allow Fiber optic gyro S power on/off automatically | 04EAAAAAH |  |  |
| 239 | KZK239 | Forbidden Fiber optic gyro S power on/off automatically | 04EBAAAAH |  |  |
| 240 | KZK240 | Allow MW X power on/off automatically | 04ECAAAAH |  |  |
| 241 | KZK241 | Forbidden MW X power on/off automatically | 04EDAAAAH |  |  |
| 242 | KZK242 | Allow MW Y power on/off automatically | 04EEAAAAH |  |  |
| 243 | KZK243 | Forbidden MW Y power on/off automatically | 04EFAAAAH |  |  |
| 244 | KZK244 | Allow MW Z power on/off automatically | 04F0AAAAH |  |  |
| 245 | KZK245 | Forbidden MW Z power on/off automatically | 04F1AAAAH |  |  |
| 246 | KZK246 | Allow MW S power on/off automatically | 04F2AAAAH |  |  |
| 247 | KZK247 | Forbidden MW S power on/off automatically | 04F3AAAAH |  |  |
| 248 | KZK248 | Forbidden autonomous diagnosis of gyro resistance to space radiation | 04F4AAAAH |  |  |
| 249 | KZK249 | Allows autonomous diagnosis of gyro resistance to space radiation | 04F5AAAAH |  |  |
| 250 | KZK250 | Forbidden autonomous diagnosis of STS resistance to space radiation | 04F6AAAAH |  |  |
| 251 | KZK251 | Allows autonomous diagnosis of STS resistance to space radiation | 04F7AAAAH |  |  |
| 252 | KZK252 | Forbidden autonomous diagnosis of MW resistance to space radiation | 04F8AAAAH |  |  |
| 253 | KZK253 | Allows autonomous diagnosis of MW resistance to space radiation | 04F9AAAAH |  |  |
| 254 | KZK254 | Forbidden autonomous diagnosis of control computer resistance to space radiation | 04FAAAAAH |  |  |
| 255 | KZK255 | Allows autonomous diagnosis of control computer resistance to space radiation | 04FBAAAAH |  |  |
| 256 | KZK256 | Initialize CAN A | 04FCAAAAH |  |  |
| 257 | KZK257 | Initialize CAN B | 04FDAAAAH |  |  |
| 258 | KZK258 | Magnetometer AB is fault | 04FEAAA0H |  |  |
| 259 | KZK259 | Magnetometer AB is normal | 04FEAAA3H |  |  |
| 260 | KZK260 | Magnetometer A_normal and B_fault | 04FEAAA1H |  |  |
| 261 | KZK261 | Magnetometer B_normal and A_fault | 04FEAAA2H |  |  |
| 262 | KZK262 | Active segment component testing | 04FFAAAAH |  |  |

DTS(KYA)

| NO | TC Number | Name | Code | DESCRIPTION | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | KYA001 | Modulation Mode | 0B01AAAAH |  |  |
| 2 | KYA002 | Single Carrier Mode | 0B02AAAAH |  |  |
| 3 | KYA003 | RF & Antenna TURN ON | 0B03AAAAH |  |  |
| 4 | KYA004 | RF & Antenna TURN OFF | 0B04AAAAH |  |  |
| 5 | KYA005 | Real-time Transmission Start | 0B05AAAAH |  |  |
| 6 | KYA006 | Real-time Transmission Stop | 0B06AAAAH |  |  |
| 7 | KYA007 | Lossless Compress Mode | 0B12AAAAH |  |  |
| 8 | KYA008 | 2:1 Compress Mode | 0B13AAAAH |  |  |
| 9 | KYA009 | SSM Main Work | 0B16AAAAH |  |  |
| 10 | KYA010 | SSM Redn Work | 0B17AAAAH |  |  |
| 11 | KYA011 | SSM Pointer Saving | 0B18AAAAH |  |  |
| 12 | KYA012 | SSM Pointer Recovery | 0B19AAAAH |  |  |
| 13 | KYA013 | SSM Sequence Recording | 0B0AAAAAH |  |  |
| 14 | KYA014 | SSM Address Recording | 0B00AAAAH |  |  |
| 15 | KYA015 | SSM Record Stop | 0B0BAAAAH |  |  |
| 16 | KYA016 | SSM Sequence Playback | 0B0CAAAAH |  |  |
| 17 | KYA017 | SSM Address Playback | 0B11AAAAH |  |  |
| 18 | KYA018 | SSM Playback Stop | 0B0DAAAAH |  |  |
| 19 | KYA019 | SSM Reset | 0B09AAAAH |  |  |
| 20 | KYA020 | SSM Self-Detecting | 0B07AAAAH |  |  |
| 21 | KYA021 | SSM Erase | 0B0EAAAAH |  |  |
| 22 | KYA022 | Autonomic Computing Start | 0B1AAAAAH |  |  |
| 23 | KYA023 | Autonomic Computing Stop | 0B1BAAAAH |  |  |
| 24 | KYA024 | Upload Direction Angle Start | 0B1CAAAAH |  |  |
| 25 | KYA025 | Upload Direction Angle Stop | 0B1DAAAAH |  |  |
| 26 | KYA026 | Ground Station 1 | 0B1FAAAAH |  |  |
| 27 | KYA027 | Ground Station 2 | 0B20AAAAH |  |  |
| 28 | KYA028 | Ground Station 3 | 0B21AAAAH |  |  |
| 29 | KYA029 | Ground Station 4 | 0B22AAAAH |  |  |
| 30 | KYA030 | Ground Station 5 | 0B23AAAAH |  |  |
| 31 | KYA031 | Ground Station 6 | 0B24AAAAH |  |  |
| 32 | KYA032 | Ground Station 7 | 0B25AAAAH |  |  |
| 33 | KYA033 | Ground Station 8 | 0B26AAAAH |  |  |
| 34 | KYA034 | CAN A Reset | 0B27AAAAH |  |  |
| 35 | KYA035 | CAN B Reset | 0B28AAAAH |  |  |

Data Block for Sub-computer

PCU data block

    - Power Parameter block

| Ordinal | Data | Instruction | Remarks |
| --- | --- | --- | --- |
| 1 | W0 | 55H |  |
| 2 | W1 | 01H |  |
| 3 | W2 | Charge/Discharge ratio | V=0.01×N |
| 4 | W3 | Batt_Current_CAPACITY | V=0.1×N |
| 5 | W4 | Battery voltage alarm threshold | V=0.1576N+0.051 |
| 6 | W5 | Battery voltage overdischarge threshold | V=0.1576N+0.051 |
| 7 | W6 | Battery voltage pre-alarm threshold | V=0.1576N+0.051 |
| 8 | W7 | Battery capacity alarm threshold | V=0.256×（N-1） |
| 9 | W8 | AAH |  |

    - Battery Heater block

| Ordinal | Data | Instruction | Remarks |
| --- | --- | --- | --- |
| 1 | W0 | 55H |  |
| 2 | W1 | 02H |  |
| 3 | W2 | Batt_Heaters_Control_SETTINGS | D7D6: 01-closed loop, 10-open loop D5: 1- Heater start when open loop 0- Heater stop when open loop |
| 4 | W3 | Batt_Heaters_Stop_TEMPERATURE | Heater stop temperature |
| 5 | W4 | Batt_Heaters_Start_TEMPERATURE | Heater start temperature |
| 6 | W5 | AAH |  |
| 7 | W6 | AAH |  |
| 8 | W7 | AAH |  |
| 9 | W8 | AAH |  |

MC data block

    - MC 57byte data block

| NO. | Byte | name | instruction | discrption |
| --- | --- | --- | --- | --- |
|  | W0 | B1 Exposure time Settings&gain setting | D15~D12： Exposure time CODE | 01~0BH |
|  | W1 |  | D11~D0： Gain CODE( 10bit) | 0~1FFH |
|  | W2 | B2 Exposure time Settings&gain setting | D15~D12： Exposure time CODE | 01~0BH |
|  | W3 |  | D11~D0： Gain CODE( 10bit) | 0~1FFH |
|  | W4 | B3 Exposure time Settings&gain setting | D15~D12： Exposure time CODE | 01~0BH |
|  | W5 |  | D11~D0： Gain CODE( 10bit) | 0~1FFH |
|  | W6 | B4 Exposure time Settings&gain setting | D15~D12： Exposure time CODE | 01~0BH |
|  | W7 |  | D11~D0： Gain CODE( 10bit) | 0~1FFH |
|  | W8 | Integration Time Settings/Algorithm Settings | D7~D4： Algorithm Settings | on：01H off：02H |
|  |  |  | D3~D0： Integration Time Settings | 01~0AH |
|  | W9~W56 | filling“AA” |  |  |

TCU data Block

    - TCU Datablock for Thermal Control

| NO | Data | Contents | Despcription |
| --- | --- | --- | --- |
| 1 | W0 | 00H |  |
| 2 | W1 | 00H |  |
| 3 | W2 | Heater ID（1-24） | 1-24： 01H: HL01_Battery(backup) 02H: HL02_phased_array_antenna_(main) 03H: HL03_phased_array_antenna_(backup) 13H: HL13_Sunshield_inside_outer_circumferential 14H: HL14_+Y_Skin_of_+Z 15H: HL15_+Y_Skin_of_-Z 16H: HL16_-Y_Skin_of_-Z 17H: HL17_-Y_Skin_of_+Z 18H: HL18_Top_of_+Z_frame 19H: HL19_Bottom_of_+Z_frame 20H: HL20_Back_Skin 21H: HL21_Upper_of_Back_Skin 22H: HL22_Bottom_of_Back_Skin 23H: HL23_Focal_plane_splice_lens_frame 24H: HL24_Focal_circuit_Inside_frame |
| 4 | W3 | Thermal Control Mode | 0x60:Open_Loop-On; 0xA0:Open_Loop-Off; 0x90:Close_Loop-Proportional-Min; 0x91:Close_Loop-Proportional-R1; 0x92:Close_Loop-Proportional-R2; 0x50:Close_Loop-Threshold-Min; 0x51:Close_Loop-Threshold-R1; 0x52:Close_Loop-Threshold-R2; 0xFF:Don’t Change Control Mode |
| 5 | W4 W5 | Temperature High Threshold Source Code(smaller number) |  |
| 6 | W6 W7 | Temperature Low Threshold Source Code(large number) |  |
| 7 | W8 | AAH |  |
| 8 |  |  |  |