|  |  |  | 编 号 Reference |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 密 级 Confidentiality | 内部 |  |
|  |  |  | 阶段标记 Phase | Z |  |
|  |  |  | 页 数 Pages | 438 |  |
| 型号代号 PROJECT; ETRSS-1. 文件名称 TITLE; ETRSS-1 Satellite Telemetry Data Format and Handing Requirement. |
|  | 编 写 Written by | 苏晨光20190901 |  |  |  |
|  | 校 对 Checked by | 李国军20190901 |  |  |  |
|  | 审 核 Reviewed by | 李军予20190901 吕大旻20190901 |  |  |  |
|  | 标 审 Form checked by | 蒋轶颍20190902 |  |  |  |
|  | 批 准 Approved by | 谭田20190902 |  |  |  |
| 中国空间技术研究院 China Academy of Space Technology |  |  |  |  |  |

会 签 Confirmed by

| 内容概要 / SUMMARY |  |  |  |
| --- | --- | --- | --- |
| This document gives telemetry data format and handing requirement for ETRSS-1 Satellite. |  |  |  |
| 主题词 / KEY WORDS |  |  |  |
| ETRSS-1 Satellite Telemetry Data Format and Handing Requirement |  |  |  |
| 更改栏 / CHANGE |  |  |  |
| 更改单号 Serial No. | 更改日期 Date | 更改人 Changed by | 更改办法 Methods |

CONTENT

1 INTRODUCTION	1

2 ACRONYM	1

3 Terms & Definitions	3

4 TELEMETRY CHARACTERISTICS	4

5 TM PACKET FORMAT	4

5.1 Packet Identification	4

5.2 Packet Sequence Control	5

5.3 Packet Data Length	5

5.4 Packet Data Field	5

6 TELEMETRY TRANSFER FRAME	5

7 TELEMETRY PROCESSING	1

7.1 Symbol specificaion	1

7.2 FIXED FORMAT TELEMETRY PROCESSING	2

8 Fixed Format Telemetry	3

9 Packet Telemetry	10

9.1 OBC	14

9.1.1 OBC_fast_TM(20H)	14

9.1.2 OBC_slow_TM(40H)	35

9.1.3 OBC_TTC_TM(90H)	62

9.1.4 CAN_Bus_Communication_TM(A0H)	64

9.1.5 OBC_FORWARD_DATA_TM(B0H)	68

9.1.6 OBC_SRAM_DOWNLOAD_TM(C0H)	69

9.1.7 One_Wire_Temperature_TM(50H)	69

9.1.8 RTTC_send/respond_TM(70H)	73

9.1.9 OBC_RTTC_TM(F0H)	86

9.1.10 OBC_TTD_TM(E0H)	102

9.2 TransponderA	103

9.2.1 Transponder_A_fast_TM(22H)	103

9.3 TransponderB	109

9.3.1 Transponder_B_fast_TM(25H)	109

9.4 PCU	115

9.4.1 PCU_fast_TM(23H)	115

9.4.2 PCU_slow_TM(43H)	118

9.5 AOCC	123

9.5.1 AOCs_fast_TM1(24H)	123

9.5.2 AOCs_fast_TM2(24H)	153

9.5.3 AOCC_GPS_Orbit_Calculation_TM(24H)	192

9.5.4 AOCC_Orbit_Parameter_Injection_ReturnTM(24H)	198

9.5.5 AOCC_Memory_Dump_TM(24H)	206

9.5.6 AOCs_slow_TM1(44H)	212

9.5.7 AOCs_slow_TM2(44H)	239

9.6 BMU	268

9.6.1 BMU_fast_TM(28H)	268

9.7 TCU	271

9.7.1 TCU_Anolog_fast_TM(27H)	271

9.7.2 TCU_Temperature_slow_TM(47H)	273

9.7.3 TCU_Configuration_slow_TM(67H)	302

9.8 DCU	357

9.8.1 DTS_fast_TM(2BH)	357

9.8.2 DTS_slow_TM(4BH)	361

9.9 GPS	365

9.9.1 GPS_fast_TM(2AH)	365

9.9.2 GPS_status_slow_TM(4AH)	368

9.9.3 GPS_Orbit_slow_TM(6AH)	400

9.10 MC	405

9.10.1 MC_fast_TM(2CH)	405

Appendix Formula Description	409

1.1 Direct reading	409

1.2 Additive	409

1.3 Subtraction	409

1.4 Multiplication	409

1.5 Division	409

1.6 Remainder	409

1.7 Linear processing	409

1.8 Temperature Processing	409

1.8.1 Temperature Processing I	409

1.8.2 Temperature Processing II	410

1.9 Custom Expression	410

ETRSS-1 Telemetry data Format and handing requirement

INTRODUCTION

This document gives telemetry data format and handing requirement for ETRSS-1 Satellite.

TM data is collected by OBC through CAN bus and edited into frames under protocol. Then, the frames shall be modulated into DPSK signal, sent to the S band transponder and transmitted to the GS.

ACRONYM

| A/D(AD) | Analogue-Digital |
| --- | --- |
| Ah | Ampere-hour |
| AIT | Assembly, Integration And Test |
| AGC | Automatic Gain Control |
| AN | Analogue |
| AOCC | Attitude and Orbit Control Computer |
| AOCS | Attitude and Orbit Control Subsystem |
| AOS | Advanced Orbiting System |
| ASS | Analogue Sun Sensor |
| BCR | Battery Charge Regulators |
| BDR | Battery Discharge Regulators |
| BL | Bi-level |
| BMU | Battery Manage Unit |
| CAN | Control Area Network |
| CCD | Charge-Coupled Device |
| CCSDS | Consultative Committee for Space Data Systems |
| CCU | Centre Control Unit |
| CDR | Critical Design Review |
| CRC | Cyclic Redundancy Check |
| D/A(DA) | Digital-Analogue |
| dB | Decibels |
| DC | Direct Current |
| DOD | Depth of Discharge |
| DPSK | Differential Phase Shift Keying |
| DS | Digital Signal |
| DSS | Digital Sun Sensor |
| DTS | Data Transmission Subsystem |
| DCU | DTS Control Unit |
| EIRP | Equivalent Isotropic Radiated Power |
| FM | Flight Model |
| FPGA | Field-Programmable Gate Array |
| GPS | Global Positioning System |
| GS | Ground Station |
| G/T | Gain/Temperature |
| GYRO | Gyroscope |
| HR | High Resolution |
| ICD | Interface Control Document |
| IDS | Interface Data Sheet |
| IRES | Infrared Earth Sensor |
| JSLC | JiuQuan Satellite Launch Site |
| LDPC | Low Density Parity Check Code |
| LVDS | Low Voltage Differential Signaling |
| MC | Multi-Spectrum Camera |
| MEA | Main Error Amplifier |
| MS | Multi-Spectrum |
| MT | Magnetic Torquer |
| MTF | Modulation Transfer Function |
| MW | Momentum Wheel |
| NA | Not Applicable/Available |
| NIR | Near Infra-Red |
| NRZ-L | Non-Return to Zero Level |
| NRZ-S | Non-Return to Zero Space |
| OBC | On-board Computer |
| OBDH | On-board Data Handling Subsystem |
| OC | Open-Collector |
| PAA | Phased Array Antenna |
| PAN | Panchromatic |
| PCM | Pulse Code Modulation |
| PCU | Power Control Unit |
| PLD | Payload |
| PM | Phase Modulation |
| PN | Pseudo-Noise |
| PROM | Programmable Read-Only Memory |
| PSK | Phase Shift Keying |
| PSS | Power Supply Subsystem |
| QPSK | Quadrature Phase Shift Keying |
| RFU | Radio Frequency Unit |
| RMS | Root Mean Square |
| RTTTC | Relative Time Tagged Telecommand |
| SA | Solar Array |
| SCC | Satellite Control Center |
| SDC | Subsystem Distribution Controller |
| SLTU | Sensor Local Terminal Unit |
| SNR | Signal to Noise Ratio |
| SR | Shunt Regulator |
| SRAM | Static Random Access Memory |
| STS | Star Sensor |
| SS-H | Star Sensor Head |
| SS-E | Star Sensor Electro-box |
| SSO | Sun- Synchronous Orbit |
| USB | Unified S-band |
| TC | Tele-Command |
| TCS | Thermal Control Subsystem |
| TDI | Time Delay Integration |
| TDM | TC Unit Data Module |
| TM | Telemetr |
| TMP | Temperature |
| TH | Thermistor |
| TT&C TTDB | Telemetry Tracking and Command Subsystem Time Tagged Data Block |
| TTTC | Time Tagged Telecommand |
| TU | TC Unit |
| TWTA | Travel Wave Tube Amplifier |
| UTC | Coordinated Universal Time |
| VCO | Voltage Controlled Oscillator |

Terms & Definitions

- Fixed Format Telemetry:

It uses W8~W13 in each telemetry transfer frame which are generated 4 frames in one second.

- Real-time Telemetry:

The telemetry is collected by OBC and transmitted to GS instantly.

- Delay Telemetry:

The telemetry is sampled as set period by OBC and stored into OBC SRAM. Delay telemetry will be transmitted when real-time telemetry is transmitted over by order. The corresponding sampling time for deley TM refers to the time of 430H packet.

- Fast Telemetry:

The collection period is 1/2/4s.

- Slow Telemetry:

The collection period is 8/16/32s..

TELEMETRY CHARACTERISTICS

| BYTE | 8bits |
| --- | --- |
| FRAME LENGTH: | 128BYTES |
| SYNCHRONIZATION | EB90 (Hex) |
| DOWNLINK BAUD RATE | 4096bps |

TM PACKET FORMAT

TM packet consists of packet primary header and data field, packet primary header size is 6 bytes and consists of Packet Identification, Packet Sequence Control and Packet Data Length.

Table 5-1 TM packet format

| Packet Primary Header (48 bits) |  |  |  |  |  |  | Packet Data Field |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Packet Version Number | Packet Identification |  |  | Packet Sequence Control |  | Packet Data Length | User Data |
|  | Packet Type | Secondary Header Flag | Application Process Identifier | Sequence Flags | Packet Sequence Count |  |  |
| 3 bits | 1 bit | 1 bit | 11 bits | 2 bits | 14 bits | 16 bits | Variable Length |
| "000" | 0 | 0 |  | "11" |  |  |  |

Packet Identification

- Packet Version Number shall be set to “000”.

- Packet Type shall be set to “0” for packet telemetry, used to distinguish with TC.

- Secondary Header Flag shall be “0”, namely a packet secondary header is not present.

- Application Process Identifier: 11 bits, used to identify TM packet, the highest bit: ‘0 ‘means real-time TM packet, while ‘1’ means delay time TM packet. All the bits are ‘1’ means idle packet.

Packet Sequence Control

- Sequence Flags shall be set to “11”, namely the space packet contains unsegmented user data.

- Packet Sequence Count: 14 bits, a counter for a packet.

Packet Data Length

16bits, equal to the size of packet data field minus 1.

Packet Data Field

Variable Length used for stored user TM data.

TELEMETRY TRANSFER FRAME

The length of TM frame is 128 bytes (1024bits) which includes Attached Syn Marker, Transfer Frame Primary Header, Fixed Format Telemetry, Packet Telemetry, Frame Error Control Field and so on。

Table 6-1TM packet Format

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | … | … | 13 | 14 | 15 | 16 | 17 | … | 127 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EB | 90 | AA | AA | AA | AA | ## | ## | ## | … | … | ## | ## | ## | ## | ## | … | ## |
| Attached Syn Marker |  | Reserved |  |  |  | Transfer Frame Primary Header |  | Fixed Format Telemetry |  |  |  | Master Channel Frame count | Virtual Channel Frame count | First Packet Header Pointer | Transfer Frame Data Field |  |  |
| 2 |  | 4 |  |  |  | 2 |  | 6 |  |  |  | 1 | 1 | 1 | 111 |  |  |

TM frame format description：

- Frame Attached Syn Marker：The value is constant: EB90(Hex) , one every 0.25 second.

2  Transfer Frame Primary Header：The value is constant: 0011(Hex).

3  Fixed Format Telemetry: The position relative TM transfer frame is fixed, which contains on board time and the telemetry of Transponder which are uplink and downlink status parameter.

4  Master Channel Frame count& Virtual Channel Frame count: The count range is from 0 to 255 which shows the continuity of TM frame.

5  First Packet Header Pointer: First packet header pointer gives the posion of the first packet in this frame. If the value of first packet header pointer is FFH, that means there is no new packet in this frame.

6  Transfer Frame Data Field: Transfer Frame Data Field is used for packet TM. If there is no enough space for storing a packet, the remnant bytes will be stored in the next frame.

Table 6-2  Continuous Frame Format

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | … | … | … | 13 | 14 | 15 | 16 | 17 | … | 127 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EB | 90 | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | M | FE | ## | ## | ## | ## |
| EB | 90 | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | M+1 | FF | ## | ## | ## | ## |
| EB | 90 | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | M+2 | 00 | ## | ## | ## | ## |
| EB | 90 | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | M+3 | 01 | ## | ## | ## | ## |
| EB | 90 | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | ## | M+4 | 02 | ## | ## | ## | ## |

TELEMETRY PROCESSING

Symbol specificaion

The key symbol used in this document is shown in the following table. Where, 4 TM transfer frame is sent by satellite in every one second, which are marked as F0~F3.

Table 6-1 KEY SYMBOL

| SYMBOL | DESCRIPTION |
| --- | --- |
| F | Frame |
| W | Word |
| B | Bit |
| P | Packet |

EX: F0W5 is the sixth word in fixed format telemetry of frist frame.

F0W5B0 is the first bit of the sixth word in fixed format telemetry of first frame.

P20W5 is the sixth word in 20H packet.

P20W5B0 is the first bit of the sixth word in 20H packet.

FIXED FORMAT TELEMETRY PROCESSING

The position of fixed format telemetry relative to TM transfer frame is fixed. Processing and displaying the corresponding parameter according to route number.

Fixed Format Telemetry

Table 7-1 FIXED FORMAT TELEMETRY Table

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | F0W0~F0W3 | XW00001 | On-board_TIME_(s) | %u | Func0001 | / | unsigned int | s | / | / |  |
| 2 | F0W4~F0W5 | XW00002 | On-board_TIME_(ms) | %u | Func0001 | / | unsigned int | ms | / | / |  |
| 3 | F1W0 | XW00003 | Spacecraft ID | %X | Func0001 | / | unsigned int |  | / | / |  |
| 4 | F1W1 | XW00004 | Reserved | %X | Func0001 | / | unsigned int |  | / | / |  |
| 5 | F1W2B7 | XW00005 | Transponder 1 transmitter on/off status | %s | Func0001 | / | unsigned int |  | / | 1:on;0:off |  |
| 6 | F1W2B6 | XW00006 | Transponder 1 carrier lock status | %s | Func0001 | / | unsigned int |  | / | 1:lock;0:unlock |  |
| 7 | F1W2B5 | XW00007 | Transponder 1 TC PSK lock status | %s | Func0001 | / | unsigned int |  | / | 1:lock;0:unlock |  |
| 8 | F1W2B4 | XW00008 | Transponder 1 TC PCM lock status | %s | Func0001 | / | unsigned int |  | / | 1:lock;0:unlock |  |
| 9 | F1W2B3 | XW00009 | Transponder 1 coherent/incoherent working status | %s | Func0001 | / | unsigned int |  | / | 1:coherent;0:incoherent |  |
| 10 | F1W2B2 | XW00010 | Transponder 1 ranging on/off working status | %s | Func0001 | / | unsigned int |  | / | 1:ranging on;0:ranging off |  |
| 11 | F1W2B1 | XW00011 | Transponder 1 TC data CRC check status | %s | Func0001 | / | unsigned int |  | / | 1:CRC right;0:CRC false |  |
| 12 | F1W2B0 | XW00012 | Transponder 1 TC channel status self-test | %s | Func0001 | / | unsigned int |  | / | 1:effective;0:ineffective |  |
| 13 | F1W3B7B6 | XW00013 | Transponder 1 SPI interface empty flag | %s | Func0001 | / | unsigned int |  | / | 01:effective;10:ineffective |  |
| 14 | F1W3B5 | XW00014 | Transponder 1 SPI-MOSI data presence or absence | %s | Func0001 | / | unsigned int |  | / | 1:presence;0:absence |  |
| 15 | F1W3B4 | XW00015 | Transponder 1 SPI-MISO data presence or absence | %s | Func0001 | / | unsigned int |  | / | 1:presence;0:absence |  |
| 16 | F1W3B3~B0 | XW00016 | Transponder 1 autonomous resetting count | %d | Func0001 | / | unsigned int |  | / | / |  |
| 17 | F1W4 | XW00017 | Transponder 1 remote channel frequency offset F(S) | %d | Func0006 | 128 | double | kHz | / | / |  |
| 18 | F1W5 | XW00018 | Transponder 1 upload channel SNR | %d | Func0001 | / | unsigned int |  | / | / |  |
| 19 | F2W0 | XW00019 | Transponder 1 transponder AGC telemetry | %d | Func0007 | 0.025882352941176470588235294117647 | unsigned int | V | / | / |  |
| 20 | F2W1 | XW00020 | Transponder 1 transponder power telemetry | %d | Func0007 | 0.025882352941176470588235294117647 | unsigned int | V | / | / |  |
| 21 | F2W2 | XW00021 | Transponder 1 OC instruction count | %d | Func0001 | / | unsigned int |  | / | / |  |
| 22 | F2W3B7~B4 | XW00022 | Transponder 1 FPGA gives ARM reset count | %d | Func0001 | / | unsigned int |  | / | / |  |
| 23 | F2W3B3~B0 | XW00023 | Transponder 1 allows/disables the Satellite Keeping control status | %s | Func0001 | / | unsigned int |  | / | 0001:permission;1000:forbidden |  |
| 24 | F2W4B7 | XW00024 | Transponder 2 transmitter on/off status | %s | Func0001 | / | unsigned int |  | / | 1:on;0:off |  |
| 25 | F2W4B6 | XW00025 | Transponder 2 carrier lock status | %s | Func0001 | / | unsigned int |  | / | 1:lock;0:unlock |  |
| 26 | F2W4B5 | XW00026 | Transponder 2 TC PSK lock status | %s | Func0001 | / | unsigned int |  | / | 1:lock;0:unlock |  |
| 27 | F2W4B4 | XW00027 | Transponder 2 TC PCM lock status | %s | Func0001 | / | unsigned int |  | / | 1:lock;0:unlock |  |
| 28 | F2W4B3 | XW00028 | Transponder 2 coherent/incoherent working status | %s | Func0001 | / | unsigned int |  | / | 1:coherent;0:incoherent |  |
| 29 | F2W4B2 | XW00029 | Transponder 2 ranging on/off working status | %s | Func0001 | / | unsigned int |  | / | 1:ranging on;0:ranging off |  |
| 30 | F2W4B1 | XW00030 | Transponder 2 TC data CRC check status | %s | Func0001 | / | unsigned int |  | / | 1:CRC right;0:CRC false |  |
| 31 | F2W4B0 | XW00031 | Transponder 2 TC channel status self-test | %s | Func0001 | / | unsigned int |  | / | 1:effective;0:ineffective |  |
| 32 | F2W5B7B6 | XW00032 | Transponder 2 SPI interface empty flag | %s | Func0001 | / | unsigned int |  | / | 01:effective;10:ineffective |  |
| 33 | F2W5B5 | XW00033 | Transponder 2 SPI-MOSI data presence or absence | %s | Func0001 | / | unsigned int |  | / | 1:presence;0:absence |  |
| 34 | F2W5B4 | XW00034 | Transponder 2 SPI-MISO data presence or absence | %s | Func0001 | / | unsigned int |  | / | 1:presence;0:absence |  |
| 35 | F2W5B3~B0 | XW00035 | Transponder 2 autonomous reset count | %d | Func0001 | / | unsigned int |  | / | / |  |
| 36 | F3W0 | XW00036 | Transponder 2 remote channel frequency offset F(S) | %d | Func0006 | 128 | double | kHz | / | / |  |
| 37 | F3W1 | XW00037 | Transponder 2 upload channel SNR | %d | Func0001 | / | unsigned int |  | / | / |  |
| 38 | F3W2 | XW00038 | Transponder 2 transponder AGC telemetry | %d | Func0007 | 0.025882352941176470588235294117647 | unsigned int | V | / | / |  |
| 39 | F3W3 | XW00039 | Transponder 2 transponder power telemetry | %d | Func0007 | 0.025882352941176470588235294117647 | unsigned int | V | / | / |  |
| 40 | F3W4 | XW00040 | Transponder 2 OC instruction count | %d | Func0001 | / | unsigned int |  | / | / |  |
| 41 | F3W5B7~B4 | XW00041 | Transponder 2 FPGA gives ARM reset count | %d | Func0001 | / | unsigned int |  | / | / |  |
| 42 | F3W5B3~B0 | XW00042 | Transponder 2 allows/disables the Satellite Keeping control status | %s | Func0001 | / | unsigned int |  | / | 0001:permission;1000:forbidden |  |

Packet Telemetry

Table 8-1 Packet Telemetry Table

| No | DEVICE | Pack Name | Pack ID | Pack length | Pack Bytes | Period | status | Delay pack | remark | reuse[Data Bytes/Bits=Value] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | OBC | OBC_fast_TM | 20 | 49 | 50 | 1 | online | 420 |  |  |
| 2 | OBC | OBC_slow_TM | 40 | 63 | 64 | 16 | online | 440 |  |  |
| 3 | OBC | OBC_TTC_TM | 90 | 61 | 62 | 32 | offline | none |  |  |
| 4 | OBC | CAN_Bus_Communication_TM | A0 | 57 | 58 | 16 | online | 4A0 |  |  |
| 5 | OBC | OBC_FORWARD_DATA_TM | B0 | variable | variable | 32 | offline | none |  |  |
| 6 | OBC | OBC_SRAM_DOWNLOAD_TM | C0 | 67 | 68 | 1 | offline | none |  |  |
| 7 | OBC | One_Wire_Temperature_TM | 50 | 41 | 42 | 32 | online | 450 |  |  |
| 8 | OBC | RTTC_send/respond_TM | 70 | 95 | 96 | 32 | offline | none |  |  |
| 9 | OBC | OBC_RTTC_TM | F0 | 255 | 256 | 32 | offline | none |  |  |
| 10 | OBC | OBC_TTD_TM | E0 | 65 | 66 | 32 | offline | none |  |  |
| 11 | TransponderA | Transponder_A_fast_TM | 22 | 15 | 16 | 1 | online | 422 |  |  |
| 12 | TransponderB | Transponder_B_fast_TM | 25 | 15 | 16 | 1 | online | 425 |  |  |
| 13 | PCU | PCU_fast_TM | 23 | 11 | 12 | 2 | online | 423 |  |  |
| 14 | PCU | PCU_slow_TM | 43 | 25 | 26 | 16 | online | 443 |  |  |
| 15 | AOCC | AOCs_fast_TM1 | 24 | 127 | 128 | 1 | online | 424 |  | P24W0B7~B4=0 |
| 16 | AOCC | AOCs_fast_TM2 | 24 | 127 | 128 | 1 | online | 424 |  | P24W0B7~B4=1 |
| 17 | AOCC | AOCC_GPS_Orbit_Calculation_TM | 24 | 127 | 128 | 1 | online | 424 |  | P24W0B7~B4=3 |
| 18 | AOCC | AOCC_Orbit_Parameter_Injection_ReturnTM | 24 | 127 | 128 | 1 | online | 424 |  | P24W0B7~B4=4 |
| 19 | AOCC | AOCC_Memory_Dump_TM | 24 | 127 | 128 | 1 | online | 424 |  | P24W0B7~B4=5 |
| 20 | AOCC | AOCs_slow_TM1 | 44 | 127 | 128 | 32 | online | 444 |  | P44W0B7~B4=6 |
| 21 | AOCC | AOCs_slow_TM2 | 44 | 127 | 128 | 32 | online | 444 |  | P44W0B7~B4=7 |
| 22 | BMU | BMU_fast_TM | 28 | 13 | 14 | 4 | online | 428 |  |  |
| 23 | TCU | TCU_Anolog_fast_TM | 27 | 11 | 12 | 2 | online | 427 |  |  |
| 24 | TCU | TCU_Temperature_slow_TM | 47 | 63 | 64 | 32 | online | 447 |  |  |
| 25 | TCU | TCU_Configuration_slow_TM | 67 | 107 | 108 | 32 | online | 467 |  |  |
| 26 | DCU | DTS_fast_TM | 2B | 7 | 8 | 2 | onlinewhenpoweron | 42B |  |  |
| 27 | DCU | DTS_slow_TM | 4B | 15 | 16 | 16 | onlinewhenpoweron | 44B |  |  |
| 28 | GPS | GPS_fast_TM | 2A | 19 | 20 | 2 | online | 42A |  |  |
| 29 | GPS | GPS_status_slow_TM | 4A | 127 | 128 | 32 | online | 44A |  |  |
| 30 | GPS | GPS_Orbit_slow_TM | 6A | 86 | 87 | 32 | online | 46A |  |  |
| 31 | MC | MC_fast_TM | 2C | 28 | 29 | 2 | onlinewhenpoweron | 435 |  |  |

OBC

OBC_fast_TM(20H)

Pack ID=20H；Pack length=49（Bytes=50）。

Table 8-2OBC_fast_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P20W0~W1 | XS001 | TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 2 | P20W2 | XS002 | Subcomputers_Indiretc_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 3 | P20W3 | XS003 | Subcomputers_Indiretc_TC__Respond_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 4 | P20W4 | XS004 | OBC_Indirect_TC_Received_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 5 | P20W5 | XS005 | OBC_Indirect_TC_Executed_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 6 | P20W6~W7 | XS006 | Latest_Executed_Indirect_TC | %X | Func0001 | / | unsigned int | / |  | / |  |
| 7 | P20W8 | XS007 | <Channel_1_status_of_TC> | %s | Func0001 | / | unsigned int | / |  | 0:normal;[1,+):error |  |
| 8 | P20W8B7 | XS008 | -SPI_Sync_header_1_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 9 | P20W8B6 | XS009 | -Address_sync_word_1_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 10 | P20W8B5 | XS010 | -Mode_word_1_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 11 | P20W8B4 | XS011 | -CRC_1_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 12 | P20W8B3 | XS012 | Spacecraft_ID_1_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 13 | P20W8B2 | XS013 | -XOR_1_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 14 | P20W8B1 | XS014 | -Length_identification_1_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 15 | P20W8B0 | XS015 | -Version_Number_1_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 16 | P20W9 | XS016 | <Channel_2_status_of_TC> | %s | Func0001 | / | unsigned int | / |  | 0:normal;[1,+):error |  |
| 17 | P20W9B7 | XS017 | -SPI_Sync_header_2_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 18 | P20W9B6 | XS018 | -Address_sync_word_2_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 19 | P20W9B5 | XS019 | -Mode_word_2_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 20 | P20W9B4 | XS020 | -CRC_2_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 21 | P20W9B3 | XS021 | -Spacecraft_identification_2_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 22 | P20W9B2 | XS022 | -XOR_and_2_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 23 | P20W9B1 | XS023 | -Length_identification_2_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 24 | P20W9B0 | XS024 | -Version_Number_2_status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 25 | P20W10 | XS025 | Indiretc_TC_respond_error_subcomputer_ID | %X | Func0001 | / | unsigned int | / |  | / |  |
| 26 | P20W11~W12 | XS026 | Indiretc_TC_respond_error_type | %X | Func0001 | / | unsigned int | / |  | 0:Right;0x0200:PROTOCOL_LEN_ERR;0x0100:TITLE_ERR;0x0080:SUM_ERR;0x0040:INT_LEN_ERR;0x0020:GAP_ERR;0x0010:RES_ERR;0x0008:ADDR_ERR;0x0004:INDEX_ERR;0x0002:FRAME_LEN_ERR |  |
| 27 | P20W13B7 | XS027 | TTC_Task_Permission_Status | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 28 | P20W13B6 | XS028 | TTC_Task_Running_Status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 29 | P20W13B5 | XS029 | TTC_Check_Status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 30 | P20W13B4 | XS030 | Reserved | %s | Func0001 | / | unsigned int | / |  | / |  |
| 31 | P20W13B3 | XS031 | TTD_Permission_Status | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 32 | P20W13B2 | XS032 | TTD_Task_Running_Status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 33 | P20W13B1 | XS033 | TTD_Check_Status | %s | Func0001 | / | unsigned int | / |  | 1:error;0:normal |  |
| 34 | P20W13B0 | XS034 | Reserved | %s | Func0001 | / | unsigned int | / |  | / |  |
| 35 | P20W14~W15 | XS035 | TTC_Number | %u | Func0001 | / | unsigned int | / |  | / |  |
| 36 | P20W16 | XS036 | TTC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 37 | P20W17 | XS037 | TTC__Respond_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 38 | P20W18 | XS038 | TTC_Insert_Status | %s | Func0001 | / | unsigned int | / |  | 1:Title_Error;2:Length_Error;3:Number_Error;4:Number_Overflow;5:Sequence_Error_In_Block;6:Confilict_with_Existing_TTC;102:Normal;153:Init_Finished |  |
| 39 | P20W19~W20 | XS039 | TTD_Number | %u | Func0001 | / | unsigned int | / |  | / |  |
| 40 | P20W21 | XS040 | TTD_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 41 | P20W22 | XS041 | TTD__Respond_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 42 | P20W23 | XS042 | TTD_Insert_Status | %s | Func0001 | / | unsigned int | / |  | 1:Title_Error;2:Length_Error;3:Number_Error;4:Number_Overflow;5:Sequence_Error_In_Block;6:Confilict_with_Existing_TTC;102:Normal;153:Init_Finished |  |
| 43 | P20W24~W25 | XS043 | Current_Task_ID | %u | Func0001 | / | unsigned int | / |  | / |  |
| 44 | P20W26B7 | XS044 | S-L_Separation_Switch_STATUS_(Hardware) | %s | Func0001 | / | unsigned int | / |  | 1:Separated;0:Unseparated |  |
| 45 | P20W26B6 | XS045 | S-L_Separation_Switch_STATUS_(Software) | %s | Func0001 | / | unsigned int | / |  | 1:Separated;0:Unseparated |  |
| 46 | P20W26B5 | XS046 | OBC_WORKING | %s | Func0001 | / | unsigned int | / |  | 1:Main;0:Backup |  |
| 47 | P20W26B4 | XS047 | OBC_Boot_Status | %s | Func0001 | / | unsigned int | / |  | 1:Warm_Boot;0:Cold_Boot |  |
| 48 | P20W26B3~B2 | XS048 | RS422_Using | %s | Func0001 | / | unsigned int | / |  | 2:Transmit_Debug_Code;1:Transmit_Telemetry_Data;0:Disabled |  |
| 49 | P20W26B1~B0 | XS049 | FLASH_TTC_Load_Status | %s | Func0001 | / | unsigned int | / |  | 2:successful;0:failure |  |
| 50 | P20W27B7 | XS050 | Management_task_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 51 | P20W27B6 | XS051 | Telemetry_task_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 52 | P20W27B5 | XS052 | Memory_refresh_task_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 53 | P20W27B4 | XS053 | Up_task_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 54 | P20W27B3 | XS054 | TC_task_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 55 | P20W27B2 | XS055 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 56 | P20W27B1 | XS056 | Watchdog_task_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 57 | P20W27B0 | XS057 | Storage_task_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 58 | P20W28B7 | XS058 | Up_task_1_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 59 | P20W28B6 | XS059 | Up_task_2_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 60 | P20W28B5 | XS060 | Up_task_3_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 61 | P20W28B4 | XS061 | Up_task_4_running_status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 62 | P20W28B3 | XS062 | Up_task_1_check_status | %s | Func0001 | / | unsigned int | / |  | 1:Right;0:Error |  |
| 63 | P20W28B2 | XS063 | Up_task_2_check_status | %s | Func0001 | / | unsigned int |  |  | 1:Right;0:Error |  |
| 64 | P20W28B1 | XS064 | Up_task_3_check_status | %s | Func0001 | / | unsigned int |  |  | 1:Right;0:Error |  |
| 65 | P20W28B0 | XS065 | Up_task_4_check_status | %s | Func0001 | / | unsigned int |  |  | 1:Right;0:Error |  |
| 66 | P20W29B7 | XS066 | RTTC_Task_Permission_Status | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 67 | P20W29B6 | XS067 | RTTC_Task_Running_Status | %s | Func0001 | / | unsigned int | / |  | 1:Running;0:Suspending |  |
| 68 | P20W29B5 | XS068 | RTTC_Start_Status | %s | Func0001 | / | unsigned int | / |  | 1:Confilict;0:Normal |  |
| 69 | P20W29B4 | XS069 | RTTC_Check_Status | %s | Func0001 | / | unsigned int | / |  | 1:Error;0:Right |  |
| 70 | P20W29B3~B0 | XS070 | Reserved | %s | Func0001 | / | unsigned int | / |  | / |  |
| 71 | P20W30 | XS071 | RTTC_Insert_Status | %s | Func0001 | / | unsigned int | / |  | 1:Title_Error;2:Length_Error;3:Number_Error;4:Number_Overflow;5:Confilict_with_Current_Task;102:Normal;153:Init_Finished |  |
| 72 | P20W31~W34B31 | XS072 | Group_0_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 73 | P20W31~W34B30 | XS073 | Group_1_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 74 | P20W31~W34B29 | XS074 | Group_2_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 75 | P20W31~W34B28 | XS075 | Group_3_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 76 | P20W31~W34B27 | XS076 | Group_4_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 77 | P20W31~W34B26 | XS077 | Group_5_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 78 | P20W31~W34B25 | XS078 | Group_6_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 79 | P20W31~W34B24 | XS079 | Group_7_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 80 | P20W31~W34B23 | XS080 | Group_8_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 81 | P20W31~W34B22 | XS081 | Group_9_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 82 | P20W31~W34B21 | XS082 | Group_10_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 83 | P20W31~W34B20 | XS083 | Group_11_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 84 | P20W31~W34B19 | XS084 | Group_12_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 85 | P20W31~W34B18 | XS085 | Group_13_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 86 | P20W31~W34B17 | XS086 | Group_14_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 87 | P20W31~W34B16 | XS087 | Group_15_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 88 | P20W31~W34B15 | XS088 | Group_16_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 89 | P20W31~W34B14 | XS089 | Group_17_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 90 | P20W31~W34B13 | XS090 | Group_18_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 91 | P20W31~W34B12 | XS091 | Group_19_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 92 | P20W31~W34B11 | XS092 | Group_20_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 93 | P20W31~W34B10 | XS093 | Group_21_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 94 | P20W31~W34B9 | XS094 | Group_22_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 95 | P20W31~W34B8 | XS095 | Group_23_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 96 | P20W31~W34B7 | XS096 | Group_24_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 97 | P20W31~W34B6 | XS097 | Group_25_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 98 | P20W31~W34B5 | XS098 | Group_26_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 99 | P20W31~W34B4 | XS099 | Group_27_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 100 | P20W31~W34B3 | XS100 | Group_28_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 101 | P20W31~W34B2 | XS101 | Group_29_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 102 | P20W31~W34B1 | XS102 | Group_30_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 103 | P20W31~W34B0 | XS103 | Group_31_RTTTC_run_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:running;0:stop |  |
| 104 | P20W35B7~B4 | XS104 | CAN_Bus_A_Reset_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 105 | P20W35B3~B0 | XS105 | CAN_Bus_B_Reset_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 106 | P20W36B7~B4 | XS106 | CAN_Bus_A_Stauts | %s | Func0001 | / | unsigned int | / |  | 0:normal;1:error;2:offline |  |
| 107 | P20W36B3~B0 | XS107 | CAN_Bus_B_Stauts | %s | Func0001 | / | unsigned int | / |  | 0:normsl;1:error;2:offline |  |
| 108 | P20W37~W38 | XS108 | <Communication_Safety_Mode_Status> | %X | Func0001 | / | unsigned int | / |  | / |  |
| 109 | P20W37~W38B15~B7 | XS109 | Reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 110 | P20W37~W38B6 | XS110 | GPS_Communication_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 111 | P20W37~W38B5 | XS111 | Battery_Subcomputer_Communication_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 112 | P20W37~W38B4 | XS112 | TCU_Communication_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 113 | P20W37~W38B3 | XS113 | Responder_B_Communication_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 114 | P20W37~W38B2 | XS114 | AOCC_Communication_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 115 | P20W37~W38B1 | XS115 | PCU_Communication_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 116 | P20W37~W38B0 | XS116 | Responder_A_Communication_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 117 | P20W39~W40 | XS117 | <Other_Safety_Mode_Status> | %X | Func0001 | / | unsigned int | / |  | / |  |
| 118 | P20W39~W40B15~B11 | XS118 | Reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 119 | P20W39~W40B10 | XS119 | Phased_array_antenna_working_timeout_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 120 | P20W39~W40B9 | XS120 | Phased_array_antenna_temperature_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 121 | P20W39~W40B8 | XS121 | CCD_temperature_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 122 | P20W39~W40B7 | XS122 | Energy_protect_Safety_Mode(_triggered_by_TCU)_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 123 | P20W39~W40B6 | XS123 | Energy_warning_Safety_Mode_2_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 124 | P20W39~W40B5 | XS124 | Energy_warning_Safety_Mode_1_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 125 | P20W39~W40B4 | XS125 | AOCS_Stop_Control_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 126 | P20W39~W40B3 | XS126 | AOCS_Global_Attitude_Acquisition_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 127 | P20W39~W40B2 | XS127 | Energy_protect_Safety_Mode(_triggered_by_PCU)_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 128 | P20W39~W40B1 | XS128 | TTD_Check_Error_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 129 | P20W39~W40B0 | XS129 | TTC_Check_Error_Safety_Mode_Status | %s | Func0001 | / | unsigned int |  |  | 0:Normal;1:Safety_Mode |  |
| 130 | P20W41~W43 | XS130 | <Safety_Mode_Settings> | %X | Func0001 | / | unsigned int |  |  | / |  |
| 131 | P20W41~W43B23~B8 | XS131 | Reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 132 | P20W41~W43B7 | XS132 | Safety_Mode_Task_ENABLE | %s | Func0001 | / | unsigned int | / |  | 0:Disabled;1:Enabled |  |
| 133 | P20W41~W43B6 | XS133 | Safety_Mode_Criterion_Table_Source | %s | Func0001 | / | unsigned int |  |  | 0:Default_Value;1:Loading_From_Flash |  |
| 134 | P20W41~W43B5~B0 | XS134 | Safety_Mode_Criterion_Table_Status | %s | Func0001 | / | unsigned int |  |  | 0:normal;[1,+):error |  |
| 135 | P20W44B7 | XS135 | one_wire_ID_Init_Status | %s | Func0001 | / | unsigned int | / |  | 1:normal;0:error |  |
| 136 | P20W44B6 | XS136 | one_wire_sensor_power_Status | %s | Func0001 | / | unsigned int | / |  | 1:power_on;0:power_off |  |
| 137 | P20W44B5 | XS137 | one_wire_sensor_Safety_Mode_Permission_Status | %X | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 138 | P20W44B4 | XS138 | one_wire_sensor_Safety_Mode_Status | %s | Func0001 | / | unsigned int | / |  | 1:Safety_Mode;0:Normal |  |
| 139 | P20W44B3~B0 | XS139 | one_wire_sensor_Safety_Mode_Counter | %u | Func0001 | / | unsigned int | / |  | / |  |
| 140 | P20W45 | XS140 | FLASH_Operation_Sector | %u | Func0001 | / | unsigned int | / |  | / |  |
| 141 | P20W46B7~B6 | XS141 | FLSH_Operation_Mode | %s | Func0001 | / | unsigned int | / |  | 2:To_Write;1:To_Erasy;0:Idle |  |
| 142 | P20W46B5~B4 | XS142 | Using_TMR | %s | Func0001 | / | unsigned int | / |  | 2:used;0:unused |  |
| 143 | P20W46B3~B0 | XS143 | FLASH_Status | %s | Func0001 | / | unsigned int | / |  | 9:Failure;6:Successful;4:Writing;2:Erasing;0:Idle |  |
| 144 | P20W47 | XS144 | <OBC_Save_Transmitter_Status> | %X | Func0001 | / | unsigned int | / |  | / |  |
| 145 | P20W47B7~B6 | XS145 | OBC_Control_Transmitter_A_Enable | %s | Func0001 | / | unsigned int | / |  | 2:Enabled;1:Disabled;0:Invalid |  |
| 146 | P20W47B5~B4 | XS146 | Transmitter_A_Status_Saved_In_OBC | %s | Func0001 | / | unsigned int | / |  | 2:A_On;1:A_Off;0:Invalid |  |
| 147 | P20W47B3~B2 | XS147 | OBC_Control_Transmitter_B_Enable | %s | Func0001 | / | unsigned int | / |  | 2:Enabled;1:Disabled;0:Invalid |  |
| 148 | P20W47B1~B0 | XS148 | Transmitter_B_Status_Saved_In_OBC | %s | Func0001 | / | unsigned int | / |  | 2:B_On;1:B_Off;0:Invalid |  |
| 149 | P20W48 | XS149 | <OBC_Data_Recover_Status> | %X | Func0001 | / | unsigned int | / |  | / |  |
| 150 | P20W48B7 | XS150 | OBC_Data_Recover_Enable | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 151 | P20W48B6 | XS151 | OBC_Data_Recover_Failure | %s | Func0001 | / | unsigned int | / |  | 1:happened;0:Not_happened |  |
| 152 | P20W48B5 | XS152 | Payload_Task_Broken | %s | Func0001 | / | unsigned int | / |  | 1:happened;0:Not_happened |  |
| 153 | P20W48B4 | XS153 | Payload_Task_Protect_Safety_Mode_Enable | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 154 | P20W48B3 | XS154 | Payload_Task_Protect_Safety_Mode_Status | %s | Func0001 | / | unsigned int | / |  | 1:Hanppened;0:Not_happened |  |
| 155 | P20W48B2~B0 | XS155 | Data_Recovery_Tsak_Working_Mode | %s | Func0001 | / | unsigned int | / |  | 0:Require_Data;1:Normal;2:Recover_Basic_Settings;3:Recover_TTC;4:Recover_TTD;5:Recover_Task_Finished;6:Syn_TTC;7:Syn_TTD |  |
| 156 | P20W49 | XS156 | Reserved | %X | Func0001 | / | unsigned int | / |  | / |  |

OBC_slow_TM(40H)

Pack ID=40H；Pack length=63（Bytes=64）。

Table 8-3OBC_slow_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P40W0~W1 | XH001 | <Subcomputer_On/Off_Line_Status> | %X | Func0001 | / | unsigned int |  |  | / |  |
| 2 | P40W0~W1B15 | XH002 | Responder_A_On/Off_Line_Status | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 3 | P40W0~W1B14 | XH003 | PCU_On/Off_Line_Status | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 4 | P40W0~W1B13 | XH004 | AOCC_On/Off_Line_Status | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 5 | P40W0~W1B12 | XH005 | Responder_B_On/Off_Line_Status | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 6 | P40W0~W1B11 | XH006 | TCU_On/Off_Line_Status | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 7 | P40W0~W1B10 | XH007 | Battery_Subcomputer_On/Off_Line_Status | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 8 | P40W0~W1B9 | XH008 | GPS_On/Off_Line_Status | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 9 | P40W0~W1B8 | XH009 | DTS_On/Off_Line_Status | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 10 | P40W0~W1B7 | XH010 | PMC_On/Off_Line_Status | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 11 | P40W0~W1B6~B0 | XH011 | Reserved | %X | Func0001 | / | unsigned int | / |  | / |  |
| 12 | P40W2~W3 | XH012 | <Subcomputer_Bus_Status> | %X | Func0001 | / | unsigned int |  |  | / |  |
| 13 | P40W2~W3B15 | XH013 | Responder_A_Bus_Status | %s | Func0001 | / | unsigned int | / |  | 1:CAN_BUS_A;0:CAN_BUS_B |  |
| 14 | P40W2~W3B14 | XH014 | PCU_Bus_Status | %s | Func0001 | / | unsigned int | / |  | 1:CAN_BUS_A;0:CAN_BUS_B |  |
| 15 | P40W2~W3B13 | XH015 | AOCC_Bus_Status | %s | Func0001 | / | unsigned int | / |  | 1:CAN_BUS_A;0:CAN_BUS_B |  |
| 16 | P40W2~W3B12 | XH016 | Responder_B_Bus_Status | %s | Func0001 | / | unsigned int | / |  | 1:CAN_BUS_A;0:CAN_BUS_B |  |
| 17 | P40W2~W3B11 | XH017 | TCU_Bus_Status | %s | Func0001 | / | unsigned int | / |  | 1:CAN_BUS_A;0:CAN_BUS_B |  |
| 18 | P40W2~W3B10 | XH018 | Battery_Subcomputer_Bus_Status | %s | Func0001 | / | unsigned int | / |  | 1:CAN_BUS_A;0:CAN_BUS_B |  |
| 19 | P40W2~W3B9 | XH019 | GPS_Bus_Status | %s | Func0001 | / | unsigned int | / |  | 1:CAN_BUS_A;0:CAN_BUS_B |  |
| 20 | P40W2~W3B8 | XH020 | DTS_Bus_Status | %s | Func0001 | / | unsigned int | / |  | 1:CAN_BUS_A;0:CAN_BUS_B |  |
| 21 | P40W2~W3B7 | XH021 | PMC_Bus_Status | %s | Func0001 | / | unsigned int | / |  | 1:CAN_BUS_A;0:CAN_BUS_B |  |
| 22 | P40W2~W3B6~B0 | XH022 | Reserved | %s | Func0001 | / | unsigned int | / |  | / |  |
| 23 | P40W4 | XH023 | <Communication_Safety_Mode_Enable_Status> | %X | Func0001 | / | unsigned int |  |  | / |  |
| 24 | P40W4B7 | XH024 | Responder_A_Communication_Safety_Mode_Enable | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 25 | P40W4B6 | XH025 | PCU_Communication_Safety_Mode_Enable | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 26 | P40W4B5 | XH026 | AOCC_Communication_Safety_Mode_Enable | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 27 | P40W4B4 | XH027 | Responder_B_Communication_Safety_Mode_Enable | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 28 | P40W4B3 | XH028 | TCU_Communication_Safety_Mode_Enable | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 29 | P40W4B2 | XH029 | Battery_Subcomputer_Communication_Safety_Mode_Enable | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 30 | P40W4B1 | XH030 | GPS_Communication_Safety_Mode_Enable | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 31 | P40W4B0 | XH031 | Reserved | %X | Func0001 | / | unsigned int | / |  | / |  |
| 32 | P40W5~W11B55~B48 | XH032 | Responder_A_Communication_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 33 | P40W5~W11B47~B40 | XH033 | PCU_Communication_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 34 | P40W5~W11B39~B32 | XH034 | AOCC_Communication_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 35 | P40W5~W11B31~B24 | XH035 | Responder_B_Communication_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 36 | P40W5~W11B23~B16 | XH036 | TCU_Communication_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 37 | P40W5~W11B15~B8 | XH037 | Battery_Subcomputer_Communication_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 38 | P40W5~W11B7~B0 | XH038 | GPS_Communication_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 39 | P40W12~W15 | XH039 | <TM_pack_on/off_line_status> | %X | Func0001 | / | unsigned int |  |  | / |  |
| 40 | P40W12~W15B31 | XH040 | -pack_20 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 41 | P40W12~W15B30 | XH041 | -pack_40 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 42 | P40W12~W15B29 | XH042 | -pack_70 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 43 | P40W12~W15B28 | XH043 | -pack_90 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 44 | P40W12~W15B27 | XH044 | -pack_A0 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 45 | P40W12~W15B26 | XH045 | -pack_B0 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 46 | P40W12~W15B25 | XH046 | -pack_C0 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 47 | P40W12~W15B24 | XH047 | -pack_E0 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 48 | P40W12~W15B23 | XH048 | -pack_F0 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 49 | P40W12~W15B22 | XH049 | -pack_50 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 50 | P40W12~W15B21 | XH050 | -pack_60 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 51 | P40W12~W15B20 | XH051 | -pack_22 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 52 | P40W12~W15B19 | XH052 | -pack_25 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 53 | P40W12~W15B18 | XH053 | -pack_23 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 54 | P40W12~W15B17 | XH054 | -pack_43 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 55 | P40W12~W15B16 | XH055 | -pack_24 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 56 | P40W12~W15B15 | XH056 | -pack_44 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 57 | P40W12~W15B14 | XH057 | -pack_64 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 58 | P40W12~W15B13 | XH058 | -pack_A4 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 59 | P40W12~W15B12 | XH059 | -pack_28 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 60 | P40W12~W15B11 | XH060 | -pack27 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 61 | P40W12~W15B10 | XH061 | -pack_47 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 62 | P40W12~W15B9 | XH062 | -pack_67 | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 63 | P40W12~W15B8 | XH063 | -pack_2A | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 64 | P40W12~W15B7 | XH064 | -pack_4A | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 65 | P40W12~W15B6 | XH065 | -pack_6A | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 66 | P40W12~W15B5 | XH066 | -pack_AA | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 67 | P40W12~W15B4 | XH067 | -pack_2C | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 68 | P40W12~W15B3 | XH068 | -pack_2B | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 69 | P40W12~W15B2 | XH069 | -pack_4B | %s | Func0001 | / | unsigned int | / |  | 1:online;0:offline |  |
| 70 | P40W12~W15B1~B0 | XH070 | reserved | %X | Func0001 | / | unsigned int | / |  | / |  |
| 71 | P40W16 | XH071 | Delay_TM_sampling_rate | %u | Func0001 | / | unsigned int | / |  | / |  |
| 72 | P40W17 | XH072 | OBC_Reset_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 73 | P40W18~W21 | XH073 | <RTTC_check_status> | %X | Func0001 | / | unsigned int |  |  | / |  |
| 74 | P40W18~W21B31 | XH074 | Group_0_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 75 | P40W18~W21B30 | XH075 | Group_1_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 76 | P40W18~W21B29 | XH076 | Group_2_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 77 | P40W18~W21B28 | XH077 | Group_3_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 78 | P40W18~W21B27 | XH078 | Group_4_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 79 | P40W18~W21B26 | XH079 | Group_5_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 80 | P40W18~W21B25 | XH080 | Group_6_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 81 | P40W18~W21B24 | XH081 | Group_7_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 82 | P40W18~W21B23 | XH082 | Group_8_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 83 | P40W18~W21B22 | XH083 | Group_9_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 84 | P40W18~W21B21 | XH084 | Group_10_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 85 | P40W18~W21B20 | XH085 | Group_11_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 86 | P40W18~W21B19 | XH086 | Group_12_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 87 | P40W18~W21B18 | XH087 | Group_13_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 88 | P40W18~W21B17 | XH088 | Group_14_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 89 | P40W18~W21B16 | XH089 | Group_15_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 90 | P40W18~W21B15 | XH090 | Group_16_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 91 | P40W18~W21B14 | XH091 | Group_17_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 92 | P40W18~W21B13 | XH092 | Group_18_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 93 | P40W18~W21B12 | XH093 | Group_19_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 94 | P40W18~W21B11 | XH094 | Group_20_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 95 | P40W18~W21B10 | XH095 | Group_21_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 96 | P40W18~W21B9 | XH096 | Group_22_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 97 | P40W18~W21B8 | XH097 | Group_23_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 98 | P40W18~W21B7 | XH098 | Group_24_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 99 | P40W18~W21B6 | XH099 | Group_25_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 100 | P40W18~W21B5 | XH100 | Group_26_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 101 | P40W18~W21B4 | XH101 | Group_27_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 102 | P40W18~W21B3 | XH102 | Group_28_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 103 | P40W18~W21B2 | XH103 | Group_29_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 104 | P40W18~W21B1 | XH104 | Group_30_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 105 | P40W18~W21B0 | XH105 | Group_31_RTTTC_check_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:error;0:right |  |
| 106 | P40W22~W25 | XH106 | <RTTC_saved_to_FLASH_Status> | %X | Func0001 | / | unsigned int |  |  | / |  |
| 107 | P40W22~W25B31 | XH107 | Group_0_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 108 | P40W22~W25B30 | XH108 | Group_1_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 109 | P40W22~W25B29 | XH109 | Group_2_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 110 | P40W22~W25B28 | XH110 | Group_3_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 111 | P40W22~W25B27 | XH111 | Group_4_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 112 | P40W22~W25B26 | XH112 | Group_5_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 113 | P40W22~W25B25 | XH113 | Group_6_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 114 | P40W22~W25B24 | XH114 | Group_7_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 115 | P40W22~W25B23 | XH115 | Group_8_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 116 | P40W22~W25B22 | XH116 | Group_9_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 117 | P40W22~W25B21 | XH117 | Group_10_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 118 | P40W22~W25B20 | XH118 | Group_11_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 119 | P40W22~W25B19 | XH119 | Group_12_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 120 | P40W22~W25B18 | XH120 | Group_13_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 121 | P40W22~W25B17 | XH121 | Group_14_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 122 | P40W22~W25B16 | XH122 | Group_15_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 123 | P40W22~W25B15 | XH123 | Group_16_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 124 | P40W22~W25B14 | XH124 | Group_17_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 125 | P40W22~W25B13 | XH125 | Group_18_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 126 | P40W22~W25B12 | XH126 | Group_19_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 127 | P40W22~W25B11 | XH127 | Group_20_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 128 | P40W22~W25B10 | XH128 | Group_21_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 129 | P40W22~W25B9 | XH129 | Group_22_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 130 | P40W22~W25B8 | XH130 | Group_23_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 131 | P40W22~W25B7 | XH131 | Group_24_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 132 | P40W22~W25B6 | XH132 | Group_25_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 133 | P40W22~W25B5 | XH133 | Group_26_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 134 | P40W22~W25B4 | XH134 | Group_27_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 135 | P40W22~W25B3 | XH135 | Group_28_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 136 | P40W22~W25B2 | XH136 | Group_29_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 137 | P40W22~W25B1 | XH137 | Group_30_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 138 | P40W22~W25B0 | XH138 | Group_31_RTTTC_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int | / |  | 1:saved;0:unsaved |  |
| 139 | P40W26~W44 | XH139 | <On_Board_Time_Settings> | %X | Func0001 | / | unsigned int | / |  | / |  |
| 140 | P40W26~W44B151 | XH140 | Time_calibration_with_GPS_ENABLE | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 141 | P40W26~W44B150~B148 | XH141 | -Reserved | %X | Func0001 | / | unsigned int | / |  | / |  |
| 142 | P40W26~W44B147~B144 | XH142 | -Periodic_Adjust_Time_STATUS | %s | Func0001 | / | unsigned int | / |  | 6:Positive;9:Negative;0:None |  |
| 143 | P40W26~W44B143~B112 | XH143 | -Periodic_Adjust_Time_(S) | %u | Func0001 | / | unsigned int | / |  | / |  |
| 144 | P40W26~W44B111~B96 | XH144 | -Periodic_Adjust_Time_(ms) | %u | Func0001 | / | unsigned int | / |  | / |  |
| 145 | P40W26~W44B95~B64 | XH145 | -OBC_TIME_(s) | %u | Func0001 | / | unsigned int | / |  | / |  |
| 146 | P40W26~W44B63~B48 | XH146 | -OBC_TIME_(ms) | %u | Func0001 | / | unsigned int | / |  | / |  |
| 147 | P40W26~W44B47~B16 | XH147 | -GPS__TIME__(s) | %u | Func0001 | / | unsigned int | / |  | / |  |
| 148 | P40W26~W44B15~B0 | XH148 | -GPS__TIME__(ms) | %u | Func0001 | / | unsigned int | / |  | / |  |
| 149 | P40W45~W46 | XH149 | <Safety_Mode_Enable_Status> | %X | Func0001 | / | unsigned int | / |  | / |  |
| 150 | P40W45~W46B15 | XH150 | TTC_Check_Error_Safety_Mode_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 151 | P40W45~W46B14 | XH151 | TTD_Check_Error_Safety_Mode_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 152 | P40W45~W46B13 | XH152 | Energy_protect_Safety_Mode(_triggered_by_PCU)_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 153 | P40W45~W46B12 | XH153 | AOCS_Global_Attitude_Acquisition_Safety_Mode_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 154 | P40W45~W46B11 | XH154 | AOCS_Stop_Control_Safety_Mode_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 155 | P40W45~W46B10 | XH155 | Energy_warning_Safety_Mode_1_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 156 | P40W45~W46B9 | XH156 | Energy_warning_Safety_Mode_2_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 157 | P40W45~W46B8 | XH157 | Energy_protect_Safety_Mode(_triggered_by_TCU)_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 158 | P40W45~W46B7 | XH158 | CCD_temperature_Safety_Mode_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 159 | P40W45~W46B6 | XH159 | Phased_array_antenna_temperature_Safety_Mode_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 160 | P40W45~W46B5 | XH160 | Phased_array_antenna_working_timeout_Safety_Mode_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 161 | P40W45~W46B4 | XH161 | S-L_Separation_Switch_Timeout_Safety_Mode_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 162 | P40W45~W46B3 | XH162 | TC_monitor_Timeout_Safety_Mode_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 163 | P40W45~W46B2~B0 | XH163 | reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 164 | P40W47~W53B55~B52 | XH164 | TTC_Check_Error_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 165 | P40W47~W53B51~B48 | XH165 | TTD_Check_Error_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 166 | P40W47~W53B47~B44 | XH166 | Energy_protect_Safety_Mode(_triggered_by_PCU)_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 167 | P40W47~W53B43~B40 | XH167 | AOCS_Global_Attitude_Acquisition_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 168 | P40W47~W53B39~B36 | XH168 | AOCS_Stop_Control_Safety_ModeCOUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 169 | P40W47~W53B35~B32 | XH169 | Energy_warning_Safety_Mode_1COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 170 | P40W47~W53B31~B28 | XH170 | Energy_warning_Safety_Mode_2_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 171 | P40W47~W53B27~B24 | XH171 | Energy_protect_Safety_Mode(_triggered_by_TCU)_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 172 | P40W47~W53B23~B20 | XH172 | CCD_temperature_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 173 | P40W47~W53B19~B16 | XH173 | Phased_array_antenna_temperature_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 174 | P40W47~W53B15~B12 | XH174 | Phased_array_antenna_working_timeout_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 175 | P40W47~W53B11~B8 | XH175 | S-L_Separation_Switch_Timeout_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 176 | P40W47~W53B7~B4 | XH176 | TC_monitor_Timeout_Safety_Mode_COUNTER | %u | Func0001 | / | unsigned int |  |  | / |  |
| 177 | P40W47~W53B3~B0 | XH177 | Reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 178 | P40W54~W55 | XH178 | S-L_Separation_Switch_Timeout_Countdown | %u | Func0001 | / | unsigned int | s |  | / |  |
| 179 | P40W56~W57 | XH179 | TC_monitor_Timeout_Countdown | %u | Func0001 | / | unsigned int | m |  | / |  |
| 180 | P40W58 | XH180 | reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 181 | P40W59~W60 | XH181 | reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 182 | P40W62~W63 | XH182 | <up_task_status> | %X | Func0001 | / | unsigned int | / |  | / |  |
| 183 | P40W61B7 | XH183 | Up_Task1_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int |  |  | 1:saved;0:unsaved |  |
| 184 | P40W61B6 | XH184 | Up_Task2_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int |  |  | 1:saved;0:unsaved |  |
| 185 | P40W61B5 | XH185 | Up_Task3_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int |  |  | 1:saved;0:unsaved |  |
| 186 | P40W61B4 | XH186 | Up_Task4_saved_to_FLASH_FLAG | %s | Func0001 | / | unsigned int |  |  | 1:saved;0:unsaved |  |
| 187 | P40W61B3~B0 | XH187 | Up_Task_Auto_Load_Enable | %s | Func0001 | / | unsigned int |  |  | 1:Enabled;0:Disabled |  |
| 188 | P40W62~W63 | XH188 | Reserved | %X | Func0001 | / | unsigned int | / |  | / |  |

OBC_TTC_TM(90H)

Pack ID=90H；Pack length=61（Bytes=62）。

Table 8-4OBC_TTC_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P90W0~W1 | XA001 | TTC_1_Task_ID | %X | Func0001 | / | unsigned int |  | / | / |  |
| 2 | P90W2~W5 | XA002 | TTC_1_Start_TIME | %X | Func0001 | / | unsigned int |  | / | / |  |
| 3 | P90W6~W9 | XA003 | TTC_1_CODE | %X | Func0001 | / | unsigned int |  | / | / |  |
| 4 | P90W10~W11 | XA004 | TTC_2_Task_ID | %X | Func0001 | / | unsigned int |  | / | / |  |
| 5 | P90W12~W15 | XA005 | TTC_2_Start_TIME | %X | Func0001 | / | unsigned int |  | / | / |  |
| 6 | P90W16~W19 | XA006 | TTC_2_CODE | %X | Func0001 | / | unsigned int |  | / | / |  |
| 7 | P90W20~W21 | XA007 | TTC_3_Task_ID | %X | Func0001 | / | unsigned int |  | / | / |  |
| 8 | P90W22~W25 | XA008 | TTC_3_Start_TIME | %X | Func0001 | / | unsigned int |  | / | / |  |
| 9 | P90W26~W29 | XA009 | TTC_3_CODE | %X | Func0001 | / | unsigned int |  | / | / |  |
| 10 | P90W30~W31 | XA010 | TTC_4_Task_ID | %X | Func0001 | / | unsigned int |  | / | / |  |
| 11 | P90W32~W35 | XA011 | TTC_4_Start_TIME | %X | Func0001 | / | unsigned int |  | / | / |  |
| 12 | P90W36~W39 | XA012 | TTC_4_CODE | %X | Func0001 | / | unsigned int |  | / | / |  |
| 13 | P90W40~W41 | XA013 | TTC_5_Task_ID | %X | Func0001 | / | unsigned int |  | / | / |  |
| 14 | P90W42~W45 | XA014 | TTC_5_Start_TIME | %X | Func0001 | / | unsigned int |  | / | / |  |
| 15 | P90W46~W49 | XA015 | TTC_5_CODE | %X | Func0001 | / | unsigned int |  | / | / |  |
| 16 | P90W50~W51 | XA016 | TTC_6_Task_ID | %X | Func0001 | / | unsigned int |  | / | / |  |
| 17 | P90W52~W55 | XA017 | TTC_6_Start_TIME | %X | Func0001 | / | unsigned int |  | / | / |  |
| 18 | P90W56~W59 | XA018 | TTC_6_CODE | %X | Func0001 | / | unsigned int |  | / | / |  |
| 19 | P90W60~W61 | XA019 | TTC_First_download_index | %X | Func0001 | / | unsigned int |  | / | / |  |

CAN_Bus_Communication_TM(A0H)

Pack ID=A0H；Pack length=57（Bytes=58）。

Table 8-5CAN_Bus_Communication_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PA0W0~W1 | XB001 | CANbus_Sent_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 2 | PA0W2~W3 | XB002 | CANbus_Responded_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 3 | PA0W4~W5 | XB003 | Responder_A_Sent_To_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 4 | PA0W6~W7 | XB004 | Responder_A_Responded_From_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 5 | PA0W8~W9 | XB005 | PCU_Sent_To_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 6 | PA0W10~W11 | XB006 | PCU_Responded_From_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 7 | PA0W12~W13 | XB007 | AOCC_Sent_To_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 8 | PA0W14~W15 | XB008 | AOCC_Responded_From_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 9 | PA0W16~W17 | XB009 | Responder_B_Sent_To_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 10 | PA0W18~W19 | XB010 | Responder_B_Responded_From_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 11 | PA0W20~W21 | XB011 | TCU_Sent_To_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 12 | PA0W22~W23 | XB012 | TCU_Responded_From_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 13 | PA0W24~W25 | XB013 | Battery_subcomputer_Sent_To_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 14 | PA0W26~W27 | XB014 | Battery_subcomputer_Responded_From_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 15 | PA0W28~W29 | XB015 | GPS_Sent_To_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 16 | PA0W30~W31 | XB016 | GPS_Responded_From_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 17 | PA0W32~W33 | XB017 | DTS_Sent_To_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 18 | PA0W34~W35 | XB018 | DTS_Responded_From_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 19 | PA0W36~W37 | XB019 | PMC_Sent_To_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 20 | PA0W38~W39 | XB020 | PMC_Responded_From_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 21 | PA0W40~W41 | XB021 | Responder_A_TM_invalid_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 22 | PA0W42~W43 | XB022 | PCU_TM_invalid_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 23 | PA0W44~W45 | XB023 | AOCC_TM_invalid_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 24 | PA0W46~W47 | XB024 | Responder_B_TM_invalid_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 25 | PA0W48~W49 | XB025 | TCU_TM_invalid_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 26 | PA0W50~W51 | XB026 | Battery_subcomputer_TM_invalid_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 27 | PA0W52~W53 | XB027 | GPS_TM_invalid_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 28 | PA0W54~W55 | XB028 | DTS_TM_invalid_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 29 | PA0W56~W57 | XB029 | PMC_TM_invalid_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |

OBC_FORWARD_DATA_TM(B0H)

Pack ID=B0H；Pack lengthvariable。

Table 8-6OBC_FORWARD_DATA_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PB0W0~W255 | XC001 | uplink_Data | %X | Func0001 | / | unsigned int |  |  | / |  |

OBC_SRAM_DOWNLOAD_TM(C0H)

Pack ID=C0H；Pack length=67（Bytes=68）。

Table 8-7OBC_SRAM_DOWNLOAD_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PC0W0~W3 | XD001 | memory_segment_address | %X | Func0001 | / | unsigned int | / |  | / |  |
| 2 | PC0W4~W67 | XD002 | Specify_memory_data | %X | Func0001 | / | unsigned int | / |  | / |  |

One_Wire_Temperature_TM(50H)

Pack ID=50H；Pack length=41（Bytes=42）。

Table 8-8One_Wire_Temperature_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P50W0W1 | XE001 | OBC_MAIN_Temperature | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 2 | P50W2W3 | XE002 | OBC_BACKUP_Temperature | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 3 | P50W4W5 | XE003 | TCU_MAIN_Temperature | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 4 | P50W6W7 | XE004 | TCU_BACKUP_Temperature | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 5 | P50W8~W9 | XE005 | DS1 Momentum wheel X | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 6 | P50W10~W11 | XE006 | DS2 Momentum wheel S | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 7 | P50W12~W13 | XE007 | DS3 Gyroscope | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 8 | P50W14~W15 | XE008 | DS4 Center Control Unit | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 9 | P50W16~W17 | XE009 | DS5 Magnetic Torquer A | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 10 | P50W18~W19 | XE010 | DS6 Star sensor A(+X) | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 11 | P50W20~W21 | XE011 | DS7 Star sensor A(-X) | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 12 | P50W22~W23 | XE012 | DS8 Analogue Sun Sensor | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 13 | P50W24~W25 | XE013 | DS9 Sceondary power box | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 14 | P50W26~W27 | XE014 | DS10 DC/DC Module 1(close to –X board) | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 15 | P50W28~W29 | XE015 | DS11 DC/DC Module 3(close to battery) | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 16 | P50W30~W31 | XE016 | DS12 Distributing pipe | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 17 | P50W32~W33 | XE017 | DS13 Stack radiation board 2(close to Transponder) | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 18 | P50W34~W35 | XE018 | DS14 Stack radiation board 3(close to power board) | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 19 | P50W36~W37 | XE019 | DS15 Camera installation heat insulation pad 1(+Z) | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 20 | P50W38~W39 | XE020 | DS16 Camera installation heat insulation pad 2(-Z) | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |
| 21 | P50W40~W41 | XE021 | DS17 Interface temperature measurement point of phased array antenna | %f | Func0007 | 0.0625 | Cpmplement | ℃ |  | / |  |

RTTC_send/respond_TM(70H)

Pack ID=70H；Pack length=95（Bytes=96）。

Table 8-9RTTC_send/respond_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P70W0 | XG001 | Group_0_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 2 | P70W1 | XG002 | Group_1_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 3 | P70W2 | XG003 | Group_2_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 4 | P70W3 | XG004 | Group_3_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 5 | P70W4 | XG005 | Group_4_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 6 | P70W5 | XG006 | Group_5_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 7 | P70W6 | XG007 | Group_6_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 8 | P70W7 | XG008 | Group_7_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 9 | P70W8 | XG009 | Group_8_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 10 | P70W9 | XG010 | Group_9_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 11 | P70W10 | XG011 | Group_10_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 12 | P70W11 | XG012 | Group_11_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 13 | P70W12 | XG013 | Group_12_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 14 | P70W13 | XG014 | Group_13_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 15 | P70W14 | XG015 | Group_14_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 16 | P70W15 | XG016 | Group_15_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 17 | P70W16 | XG017 | Group_16_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 18 | P70W17 | XG018 | Group_17_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 19 | P70W18 | XG019 | Group_18_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 20 | P70W19 | XG020 | Group_19_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 21 | P70W20 | XG021 | Group_20_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 22 | P70W21 | XG022 | Group_21_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 23 | P70W22 | XG023 | Group_22_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 24 | P70W23 | XG024 | Group_23_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 25 | P70W24 | XG025 | Group_24_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 26 | P70W25 | XG026 | Group_25_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 27 | P70W26 | XG027 | Group_26_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 28 | P70W27 | XG028 | Group_27_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 29 | P70W28 | XG029 | Group_28_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 30 | P70W29 | XG030 | Group_29_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 31 | P70W30 | XG031 | Group_30_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 32 | P70W31 | XG032 | Group_31_RTTC_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 33 | P70W32 | XG033 | RTTTC_0_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 34 | P70W33 | XG034 | RTTTC_0_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 35 | P70W34 | XG035 | RTTTC_1_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 36 | P70W35 | XG036 | RTTTC_1_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 37 | P70W36 | XG037 | RTTTC_2_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 38 | P70W37 | XG038 | RTTTC_2_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 39 | P70W38 | XG039 | RTTTC_3_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 40 | P70W39 | XG040 | RTTTC_3_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 41 | P70W40 | XG041 | RTTTC_4_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 42 | P70W41 | XG042 | RTTTC_4_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 43 | P70W42 | XG043 | RTTTC_5_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 44 | P70W43 | XG044 | RTTTC_5_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 45 | P70W44 | XG045 | RTTTC_6_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 46 | P70W45 | XG046 | RTTTC_6_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 47 | P70W46 | XG047 | RTTTC_7_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 48 | P70W47 | XG048 | RTTTC_7_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 49 | P70W48 | XG049 | RTTTC_8_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 50 | P70W49 | XG050 | RTTTC_8_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 51 | P70W50 | XG051 | RTTTC_9_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 52 | P70W51 | XG052 | RTTTC_9_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 53 | P70W52 | XG053 | RTTTC_10_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 54 | P70W53 | XG054 | RTTTC_10_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 55 | P70W54 | XG055 | RTTTC_11_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 56 | P70W55 | XG056 | RTTTC_11_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 57 | P70W56 | XG057 | RTTTC_12_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 58 | P70W57 | XG058 | RTTTC_12_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 59 | P70W58 | XG059 | RTTTC_13_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 60 | P70W59 | XG060 | RTTTC_13_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 61 | P70W60 | XG061 | RTTTC_14_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 62 | P70W61 | XG062 | RTTTC_14_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 63 | P70W62 | XG063 | RTTTC_15_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 64 | P70W63 | XG064 | RTTTC_15_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 65 | P70W64 | XG065 | RTTTC_16_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 66 | P70W65 | XG066 | RTTTC_16_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 67 | P70W66 | XG067 | RTTTC_17_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 68 | P70W67 | XG068 | RTTTC_17_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 69 | P70W68 | XG069 | RTTTC_18_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 70 | P70W69 | XG070 | RTTTC_18_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 71 | P70W70 | XG071 | RTTTC_19_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 72 | P70W71 | XG072 | RTTTC_19_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 73 | P70W72 | XG073 | RTTTC_20_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 74 | P70W73 | XG074 | RTTTC_20_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 75 | P70W74 | XG075 | RTTTC_21_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 76 | P70W75 | XG076 | RTTTC_21_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 77 | P70W76 | XG077 | RTTTC_22_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 78 | P70W77 | XG078 | RTTTC_22_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 79 | P70W78 | XG079 | RTTTC_23_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 80 | P70W79 | XG080 | RTTTC_23_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 81 | P70W80 | XG081 | RTTTC_24_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 82 | P70W81 | XG082 | RTTTC_24_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 83 | P70W82 | XG083 | RTTTC_25_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 84 | P70W83 | XG084 | RTTTC_25_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 85 | P70W84 | XG085 | RTTTC_26_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 86 | P70W85 | XG086 | RTTTC_26_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 87 | P70W86 | XG087 | RTTTC_27_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 88 | P70W87 | XG088 | RTTTC_27_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 89 | P70W88 | XG089 | RTTTC_28_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 90 | P70W89 | XG090 | RTTTC_28_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 91 | P70W90 | XG091 | RTTTC_29_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 92 | P70W91 | XG092 | RTTTC_29_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 93 | P70W92 | XG093 | RTTTC_30_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 94 | P70W93 | XG094 | RTTTC_30_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 95 | P70W94 | XG095 | RTTTC_31_TC_Sent_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 96 | P70W95 | XG096 | RTTTC_31_TC_Responded_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |

OBC_RTTC_TM(F0H)

Pack ID=F0H；Pack length=255（Bytes=256）。

Table 8-10OBC_RTTC_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PF0W0 | XI001 | RTTC_Group_NUMBER | %u | Func0001 | / | unsigned int | / | [0,31] | / |  |
| 2 | PF0W1W2B15~B12 | XI002 | TC_1_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 3 | PF0W1W2B11~B0 | XI003 | TC_1_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 4 | PF0W3~W6 | XI004 | TC_1_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 5 | PF0W7W8B15~B12 | XI005 | TC_2_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 6 | PF0W7W8B11~B0 | XI006 | TC_2_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 7 | PF0W9~W12 | XI007 | TC_2_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 8 | PF0W13W14B15~B12 | XI008 | TC_3_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 9 | PF0W13W14B11~B0 | XI009 | TC_3_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 10 | PF0W15~W18 | XI010 | TC_3_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 11 | PF0W19W20B15~B12 | XI011 | TC_4_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 12 | PF0W19W20B11~B0 | XI012 | TC_4_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 13 | PF0W21~W24 | XI013 | TC_4_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 14 | PF0W25W26B15~B12 | XI014 | TC_5_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 15 | PF0W25W26B11~B0 | XI015 | TC_5_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 16 | PF0W27~W30 | XI016 | TC_5_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 17 | PF0W31W32B15~B12 | XI017 | TC_6_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 18 | PF0W31W32B11~B0 | XI018 | TC_6_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 19 | PF0W33~W36 | XI019 | TC_6_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 20 | PF0W37W38B15~B12 | XI020 | TC_7_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 21 | PF0W37W38B11~B0 | XI021 | TC_7_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 22 | PF0W39~W42 | XI022 | TC_7_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 23 | PF0W43W44B15~B12 | XI023 | TC_8_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 24 | PF0W43W44B11~B0 | XI024 | TC_8_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 25 | PF0W45~W48 | XI025 | TC_8_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 26 | PF0W49W50B15~B12 | XI026 | TC_9_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 27 | PF0W49W50B11~B0 | XI027 | TC_9_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 28 | PF0W51~W54 | XI028 | TC_9_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 29 | PF0W55W56B15~B12 | XI029 | TC_10_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 30 | PF0W55W56B11~B0 | XI030 | TC_10_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 31 | PF0W57~W60 | XI031 | TC_10_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 32 | PF0W61W62B15~B12 | XI032 | TC_11_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 33 | PF0W61W62B11~B0 | XI033 | TC_11_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 34 | PF0W63~W66 | XI034 | TC_11_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 35 | PF0W67W68B15~B12 | XI035 | TC_12_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 36 | PF0W67W68B11~B0 | XI036 | TC_12_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 37 | PF0W69~W72 | XI037 | TC_12_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 38 | PF0W73W74B15~B12 | XI038 | TC_13_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 39 | PF0W73W74B11~B0 | XI039 | TC_13_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 40 | PF0W75~W78 | XI040 | TC_13_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 41 | PF0W79W80B15~B12 | XI041 | TC_14_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 42 | PF0W79W80B11~B0 | XI042 | TC_14_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 43 | PF0W81~W84 | XI043 | TC_14_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 44 | PF0W85W86B15~B12 | XI044 | TC_15_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 45 | PF0W85W86B11~B0 | XI045 | TC_15_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 46 | PF0W87~W90 | XI046 | TC_15_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 47 | PF0W91W92B15~B12 | XI047 | TC_16_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 48 | PF0W91W92B11~B0 | XI048 | TC_16_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 49 | PF0W93~W96 | XI049 | TC_16_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 50 | PF0W97W98B15~B12 | XI050 | TC_17_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 51 | PF0W97W98B11~B0 | XI051 | TC_17_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 52 | PF0W99~W102 | XI052 | TC_17_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 53 | PF0W103W104B15~B12 | XI053 | TC_18_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 54 | PF0W103W104B11~B0 | XI054 | TC_18_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 55 | PF0W105~W108 | XI055 | TC_18_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 56 | PF0W109W110B15~B12 | XI056 | TC_19_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 57 | PF0W109W110B11~B0 | XI057 | TC_19_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 58 | PF0W111~W114 | XI058 | TC_19_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 59 | PF0W115W116B15~B12 | XI059 | TC_20_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 60 | PF0W115W116B11~B0 | XI060 | TC_20_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 61 | PF0W117~W120 | XI061 | TC_20_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 62 | PF0W121W122B15~B12 | XI062 | TC_21_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 63 | PF0W121W122B11~B0 | XI063 | TC_21_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 64 | PF0W123~W126 | XI064 | TC_21_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 65 | PF0W127W128B15~B12 | XI065 | TC_22_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 66 | PF0W127W128B11~B0 | XI066 | TC_22_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 67 | PF0W129~W132 | XI067 | TC_22_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 68 | PF0W133W134B15~B12 | XI068 | TC_23_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 69 | PF0W133W134B11~B0 | XI069 | TC_23_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 70 | PF0W135~W138 | XI070 | TC_23_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 71 | PF0W139W140B15~B12 | XI071 | TC_24_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 72 | PF0W139W140B11~B0 | XI072 | TC_24_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 73 | PF0W141~W144 | XI073 | TC_24_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 74 | PF0W145W146B15~B12 | XI074 | TC_25_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 75 | PF0W145W146B11~B0 | XI075 | TC_25_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 76 | PF0W147~W150 | XI076 | TC_25_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 77 | PF0W151W152B15~B12 | XI077 | TC_26_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 78 | PF0W151W152B11~B0 | XI078 | TC_26_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 79 | PF0W153~W156 | XI079 | TC_26_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 80 | PF0W157W158B15~B12 | XI080 | TC_27_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 81 | PF0W157W158B11~B0 | XI081 | TC_27_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 82 | PF0W159~W162 | XI082 | TC_27_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 83 | PF0W163W164B15~B12 | XI083 | TC_28_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 84 | PF0W163W164B11~B0 | XI084 | TC_28_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 85 | PF0W165~W168 | XI085 | TC_28_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 86 | PF0W163W164B15~B12 | XI086 | TC_29_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 87 | PF0W163W164B11~B0 | XI087 | TC_29_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 88 | PF0W165~W168 | XI088 | TC_29_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 89 | PF0W175W176B15~B12 | XI089 | TC_30_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 90 | PF0W175W176B11~B0 | XI090 | TC_30_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 91 | PF0W177~W180 | XI091 | TC_30_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 92 | PF0W181W182B15~B12 | XI092 | TC_31_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 93 | PF0W181W182B11~B0 | XI093 | TC_31_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 94 | PF0W183~W186 | XI094 | TC_31_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 95 | PF0W187W188B15~B12 | XI095 | TC_32_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 96 | PF0W187W188B11~B0 | XI096 | TC_32_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 97 | PF0W189~W192 | XI097 | TC_32_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 98 | PF0W193W194B15~B12 | XI098 | TC_33_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 99 | PF0W193W194B11~B0 | XI099 | TC_33_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 100 | PF0W195~W198 | XI100 | TC_33_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 101 | PF0W199W200B15~B12 | XI101 | TC_34_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 102 | PF0W199W200B11~B0 | XI102 | TC_34_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 103 | PF0W201~W204 | XI103 | TC_34_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 104 | PF0W205W206B15~B12 | XI104 | TC_35_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 105 | PF0W205W206B11~B0 | XI105 | TC_35_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 106 | PF0W207~W210 | XI106 | TC_35_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 107 | PF0W211W212B15~B12 | XI107 | TC_36_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 108 | PF0W211W212B11~B0 | XI108 | TC_36_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 109 | PF0W213~W216 | XI109 | TC_36_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 110 | PF0W217W218B15~B12 | XI110 | TC_37_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 111 | PF0W217W218B11~B0 | XI111 | TC_37_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 112 | PF0W219~W222 | XI112 | TC_37_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 113 | PF0W223W224B15~B12 | XI113 | TC_38_Segment_NUMBER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 114 | PF0W223W224B11~B0 | XI114 | TC_38_Relative_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 115 | PF0W225~W228 | XI115 | TC_38_Command_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 116 | PF0W229~W255 | XI116 | reserved | %X | Func0001 | / | unsigned int | / |  | / |  |

OBC_TTD_TM(E0H)

Pack ID=E0H；Pack length=65（Bytes=66）。

Table 8-11OBC_TTD_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PE0W0~W1 | XK001 | TTD_1_Task_ID | %u | Func0001 | / | unsigned int | / |  | / |  |
| 2 | PE0W2~W5 | XK002 | TTD_1_Start_TIME | %u | Func0001 | / | unsigned int | / |  | / |  |
| 3 | PE0W6 | XK003 | TTD_1_Virtual_Channel | %X | Func0001 | / | unsigned int | / |  | / |  |
| 4 | PE0W7~W63 | XK004 | TTD_1_CODE | %X | Func0001 | / | unsigned int | / |  | / |  |
| 5 | PE0W64~W65 | XK005 | TTD_First_Download_Index | %u | Func0001 | / | unsigned int | / |  | / |  |

TransponderA

Transponder_A_fast_TM(22H)

Pack ID=22H；Pack length=15（Bytes=16）。

Table 8-12Transponder_A_fast_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P22W0B7 | TA001 | Transponder 1 transmitter on/off status | %s | Func0001 | / | unsigned int |  |  | 1:on;0:off |  |
| 2 | P22W0B6 | TA002 | Transponder 1 carrier lock status | %s | Func0001 | / | unsigned int |  |  | 1:lock;0:unlock |  |
| 3 | P22W0B5 | TA003 | Transponder 1 TC PSK lock status | %s | Func0001 | / | unsigned int |  |  | 1:lock;0:unlock |  |
| 4 | P22W0B4 | TA004 | Transponder 1 TC PCM lock status | %s | Func0001 | / | unsigned int |  |  | 1:lock;0:unlock |  |
| 5 | P22W0B3 | TA005 | Transponder 1 coherent/incoherent working status | %s | Func0001 | / | unsigned int |  |  | 1:coherent;0:incoherent |  |
| 6 | P22W0B2 | TA006 | Transponder 1 ranging on/off working status | %s | Func0001 | / | unsigned int |  |  | 1:ranging on;0:ranging off |  |
| 7 | P22W0B1 | TA007 | Transponder 1 TC data CRC check status | %s | Func0001 | / | unsigned int |  |  | 1:CRC right;0:CRC false |  |
| 8 | P22W0B0 | TA008 | Transponder 1 TC channel status self-test | %s | Func0001 | / | unsigned int |  |  | 1:effective;0:ineffective |  |
| 9 | P22W1B7~B6 | TA009 | Transponder 1 SPI interface empty flag | %s | Func0001 | / | unsigned int |  |  | 01:effective;10:ineffective |  |
| 10 | P22W1B5 | TA010 | Transponder 1 SPI-MOSI data presence or absence | %s | Func0001 | / | unsigned int |  |  | 1:presence;0:absence |  |
| 11 | P22W1B4 | TA011 | Transponder 1 SPI-MISO data presence or absence | %s | Func0001 | / | unsigned int |  |  | 1:presence;0:absence |  |
| 12 | P22W1B3~B0 | TA012 | Transponder 1 autonomous resetting count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 13 | P22W2 | TA013 | Transponder 1 remote channel frequency offset F(S) | %d | Func0006 | 128 | double | kHz | [-128,127] | / |  |
| 14 | P22W3 | TA014 | Transponder 1 upload channel SNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 15 | P22W4 | TA015 | Transponder 1 transponder AGC telemetry | %d | Func0007 | 0.0258 | unsigned int |  |  | / |  |
| 16 | P22W5 | TA016 | Transponder 1 transponder power telemetry | %d | Func0007 | 0.02588 | unsigned int |  |  | / |  |
| 17 | P22W6 | TA017 | Transponder 1 OC instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 18 | P22W7B7~B4 | TA018 | Transponder 1 FPGA gives ARM reset count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 19 | P22W7B3~B0 | TA019 | Transponder 1 allows/disables the Satellite Keeping control status | %s | Func0001 | / | unsigned int |  |  | 0001:permission;1000:forbidden |  |
| 20 | P22W8 | TA020 | Transponder 1 +3.5V power telemetry | %d | Func0007 | 0.02588 | unsigned int |  |  | / |  |
| 21 | P22W9 | TA021 | Transponder 1 temperature telemetry | %d | Func0015 | 10000,3.3,-6.01188,4622.53337,-86421.72414 | float |  |  | / |  |
| 22 | P22W10B7~B4 | TA022 | Transponder 1 baseband execution instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 23 | P22W10B3~B0 | TA023 | Transponder 1 base band receives instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 24 | P22W11B7~B4 | TA024 | Transponder 1 SPI telemetry input frame count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 25 | P22W11B3~B0 | TA025 | Transponder 1 SPI TC output frame count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 26 | P22W12B7~B4 | TA026 | Transponder 1 CAN bus receives data type indirect instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 27 | P22W12B3~B0 | TA027 | Transponder 1 CAN A bus resetting count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 28 | P22W13B7~B4 | TA028 | Transponder 1 CAN B bus resetting count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 29 | P22W13B3~B0 | TA029 | Transponder 1 ARM execute data type indirect instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 30 | P22W14B7~B4 | TA030 | Transponder 1 ARM sends data type indirect instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 31 | P22W14B3~B0 | TA031 | Transponder 1 FPGA received reset instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 32 | P22W15 | TA032 | Reserved | %d | Func0001 | / | unsigned int |  |  | / |  |

TransponderB

Transponder_B_fast_TM(25H)

Pack ID=25H；Pack length=15（Bytes=16）。

Table 8-13Transponder_B_fast_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P25W0B7 | TB001 | Transponder 2 transmitter on/off status | %s | Func0001 | / | unsigned int |  |  | 1:on;0:off |  |
| 2 | P25W0B6 | TB002 | Transponder 2 carrier lock status | %s | Func0001 | / | unsigned int |  |  | 1:lock;0:unlock |  |
| 3 | P25W0B5 | TB003 | Transponder 2 TC PSK lock status | %s | Func0001 | / | unsigned int |  |  | 1:lock;0:unlock |  |
| 4 | P25W0B4 | TB004 | Transponder 2 TC PCM lock status | %s | Func0001 | / | unsigned int |  |  | 1:lock;0:unlock |  |
| 5 | P25W0B3 | TB005 | Transponder 2 coherent/incoherent working status | %s | Func0001 | / | unsigned int |  |  | 1:coherent;0:incoherent |  |
| 6 | P25W0B2 | TB006 | Transponder 2 ranging on/off working status | %s | Func0001 | / | unsigned int |  |  | 1:ranging on;0:ranging off |  |
| 7 | P25W0B1 | TB007 | Transponder 2 TC data CRC check status | %s | Func0001 | / | unsigned int |  |  | 1:CRC right;0:CRC false |  |
| 8 | P25W0B0 | TB008 | Transponder 2 TC channel status self-test | %s | Func0001 | / | unsigned int |  |  | 1:effective;0:ineffective |  |
| 9 | P25W1B7~B6 | TB009 | Transponder 2 SPI interface empty flag | %s | Func0001 | / | unsigned int |  |  | 01:effective;10:ineffective |  |
| 10 | P25W1B5 | TB010 | Transponder 2 SPI-MOSI data presence or absence | %s | Func0001 | / | unsigned int |  |  | 1:presence;0:absence |  |
| 11 | P25W1B4 | TB011 | Transponder 2 SPI-MISO data presence or absence | %s | Func0001 | / | unsigned int |  |  | 1:presence;0:absence |  |
| 12 | P25W1B3~B0 | TB012 | Transponder 2 autonomous reset count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 13 | P25W2 | TB013 | Transponder 2 remote channel frequency offset F(S) | %d | Func0006 | 128 | double | kHz | [-128,127] | / |  |
| 14 | P25W3 | TB014 | Transponder 2 upload channel SNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 15 | P25W4 | TB015 | Transponder 2 transponder AGC telemetry | %d | Func0007 | 0.0258 | unsigned int |  |  | / |  |
| 16 | P25W5 | TB016 | Transponder 2 transponder power telemetry | %d | Func0007 | 0.02588 | unsigned int |  |  | / |  |
| 17 | P25W6 | TB017 | Transponder 2 OC instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 18 | P25W7B7~B4 | TB018 | Transponder 2 FPGA gives ARM reset count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 19 | P25W7B3~B0 | TB019 | Transponder 2 allows/disables the Satellite Keeping control status | %s | Func0001 | / | unsigned int |  |  | 0001:permission;1000:forbidden |  |
| 20 | P25W8 | TB020 | Transponder 2 +3.5V power telemetry | %d | Func0007 | 0.02588 | unsigned int |  |  | / |  |
| 21 | P25W9 | TB021 | Transponder 2 temperature telemetry | %d | Func0015 | 10000,3.3,-6.01188,4622.53337,-86421.72414 | float |  |  | / |  |
| 22 | P25W10B7~B4 | TB022 | Transponder 2 baseband execution instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 23 | P25W10B3~B0 | TB023 | Transponder 2 base band receives instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 24 | P25W11B7~B4 | TB024 | Transponder 2 SPI telemetry input frame count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 25 | P25W11B3~B0 | TB025 | Transponder 2 SPI TC output frame count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 26 | P25W12B7~B4 | TB026 | Transponder 2 CAN bus receives data type indirect instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 27 | P25W12B3~B0 | TB027 | Transponder 2 CAN A bus reset count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 28 | P25W13B7~B4 | TB028 | Transponder 2 CAN B bus reset count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 29 | P25W13B3~B0 | TB029 | Transponder 2 ARM execute data type indirect instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 30 | P25W14B7~B4 | TB030 | Transponder 2 ARM sends data type indirect instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 31 | P25W14B3~B0 | TB031 | Transponder 2 FPGA received reset instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 32 | P25W15 | TB032 | Reserved | %d | Func0001 | / | unsigned int |  |  | / |  |

PCU

PCU_fast_TM(23H)

Pack ID=23H；Pack length=11（Bytes=12）。

Table 8-14PCU_fast_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P23W0 | NS001 | 28V bus voltage | %6.2f | Func0010 | 0.2393,0 | unsigned int | V |  | / |  |
| 2 | P23W1 | NS002 | +12V voltage | %6.2f | Func0010 | 0.107105,0 | unsigned int | V |  | / |  |
| 3 | P23W2 | NS003 | +5V voltage | %6.2f | Func0010 | 0.04279365,0 | unsigned int | V |  | / |  |
| 4 | P23W3 | NS004 | -12V voltage | %+6.2f | Func0010 | 0.021054,-14.512886 | unsigned int | V |  | / |  |
| 5 | P23W4 | NS005 | Batt_voltage | %6.2f | Func0010 | 0.1576,0.051 | unsigned int | V |  | / |  |
| 6 | P23W5 | NS006 | Bus Load current | %6.2f | Func0010 | 0.0290,0.0085 | unsigned int | A |  | / |  |
| 7 | P23W6 | NS007 | Solar array current | %6.2f | Func0010 | 0.0402,-0.0467 | unsigned int | A |  | / |  |
| 8 | P23W7 | NS008 | Battery Charging current | %6.2f | Func0010 | 0.0400,-0.017 | unsigned int | A |  | / |  |
| 9 | P23W8 | NS009 | Battery Discharge current | %6.2f | Func0010 | 0.0395,-0.0663 | unsigned int | A |  | / |  |
| 10 | P23W9 | NS010 | Batt_temperature 1 | %6.2f | Func0015 | 10000,286.875,-4.36216,4081.70194,-94033.78039 | unsigned int | ℃ |  | / |  |
| 11 | P23W10B7 | NS011 | Battery Discharge switch status | %s | Func0001 | / | unsigned int | / |  | 0:on;1:off |  |
| 12 | P23W10B6 | NS012 | Transponder 2 power switch status | %s | Func0001 | / | unsigned int | / |  | 0:off;1:on |  |
| 13 | P23W10B5 | NS013 | Solar panel release power status | %s | Func0001 | / | unsigned int | / |  | 0:off;1:on |  |
| 14 | P23W10B4 | NS014 | -Y panel deployment status | %s | Func0001 | / | unsigned int | / |  | 0:not deployed;1:deployed |  |
| 15 | P23W10B3 | NS015 | +Y panel deployment status | %s | Func0001 | / | unsigned int | / |  | 0:not deployed;1:deployed |  |
| 16 | P23W10B2 | NS016 | Battery heater status | %s | Func0001 | / | unsigned int | / |  | 0:heated;1:not heated |  |
| 17 | P23W10B1 | NS017 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 18 | P23W10B0 | NS018 | ±Y panel deployment power status | %s | Func0001 | / | unsigned int | / |  | 0:off;1:on |  |
| 19 | P23W11B7 | NS019 | Overdischarge flag | %s | Func0001 | / | unsigned int | / |  | 1:overdischarged;0:not overdischarged |  |
| 20 | P23W11B6 | NS020 | BMU power switch status | %s | Func0001 | / | unsigned int | / |  | 1:on;0:off |  |
| 21 | P23W11B5 | NS021 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 22 | P23W11B4 | NS022 | FLASH erasing enable | %s | Func0001 | / | unsigned int | / |  | 1:enabled;0:disenabled |  |
| 23 | P23W11B3 | NS023 | Upload OC task status | %s | Func0001 | / | unsigned int | / |  | 1:started up;0:not started up |  |
| 24 | P23W11B2 | NS024 | Upload CAN task status | %s | Func0001 | / | unsigned int | / |  | 1:started up;0:not started up |  |
| 25 | P23W11B1 | NS025 | Upload POWER task status | %s | Func0001 | / | unsigned int | / |  | 1:started up;0:not started up |  |
| 26 | P23W11B0 | NS026 | Upload task status | %s | Func0001 | / | unsigned int | / |  | 1:started up;0:not started up |  |

PCU_slow_TM(43H)

Pack ID=43H；Pack length=25（Bytes=26）。

Table 8-15PCU_slow_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P43W0B7~B4 | NH001 | PCU_Reset_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 2 | P43W0B3~B0 | NH002 | PCU_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 3 | P43W1 | NH003 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 4 | P43W2~W3 | NH004 | Batt_Current_CAPACITY | %6.2f | Func0010 | 0.001,0 | unsigned int | / |  | / |  |
| 5 | P43W4~W5 | NH005 | Batt_Charged_CAPACITY | %6.2f | Func0010 | 0.001,0 | unsigned int | / |  | / |  |
| 6 | P43W6~W7 | NH006 | Batt_Discharged_CAPACITY | %6.2f | Func0010 | 0.001,0 | unsigned int | / |  | / |  |
| 7 | P43W8 | NH007 | Energy security status | %s | Func0001 | / | unsigned int | / |  | [0,0]:normal;(0,255]:abnormal |  |
| 8 | P43W9 | NH008 | Bus overvoltage protection flag | %s | Func0001 | / | unsigned int | / |  | [0,0]:normal;(0,255]:abnormal |  |
| 9 | P43W10B7 | NH009 | PCU self-test status | %s | Func0001 | / | unsigned int | / |  | 0:normal;1:abnormal |  |
| 10 | P43W10B6 | NH010 | PCU use status | %s | Func0001 | / | unsigned int | / |  | 0:Sub-computer B;1:Sub-computer A |  |
| 11 | P43W10B5 | NH011 | CAPACITY calculation flag | %s | Func0001 | / | unsigned int | / |  | 0:not calculating;1:calculating |  |
| 12 | P43W10B4 | NH012 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 13 | P43W10B3 | NH013 | Overdischarge FLAG | %s | Func0001 | / | unsigned int | / |  | 0:not overdischargeed;1:overdischargeed |  |
| 14 | P43W10B2 | NH014 | Battery automatically switch on FLAG | %s | Func0001 | / | unsigned int | / |  | 0:not reconnected;1:reconnected |  |
| 15 | P43W10B1 | NH015 | main OBC on switch TC send status | %s | Func0001 | / | unsigned int | / |  | 0:not formed;1:formed |  |
| 16 | P43W10B0 | NH016 | redundant OBC on switch TC send status | %s | Func0001 | / | unsigned int | / |  | 0:not formed;1:formed |  |
| 17 | P43W11B7 | NH017 | Overdischarge_protect_ENABLE | %s | Func0001 | / | unsigned int | / |  | 0:disenabled;1:enabled |  |
| 18 | P43W11B6 | NH018 | Battery automatically switch on ENABLE | %s | Func0001 | / | unsigned int | / |  | 0:disenabled;1:enabled |  |
| 19 | P43W11B5 | NH019 | Safe mode send unlock enable | %s | Func0001 | / | unsigned int | / |  | 0:disenabled;1:enabled |  |
| 20 | P43W11B4 | NH020 | OBC reset enable status | %s | Func0001 | / | unsigned int | / |  | 0:disenabled;1:enabled |  |
| 21 | P43W11B3~B0 | NH021 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 22 | P43W12 | NH022 | Batt_Heaters_Control_SETTINGS | %u | Func0001 | / | unsigned int | / |  | / |  |
| 23 | P43W13 | NH023 | Batt_Heaters_Stop_TEMPERATURE | %6.2f | Func0015 | 10000,286.875,-4.36216,4081.70194,-94033.78039 | unsigned int | / |  | / |  |
| 24 | P43W14 | NH024 | Batt_Heaters_Start_TEMPERATURE | %6.2f | Func0015 | 10000,286.875,-4.36216,4081.70194,-94033.78039 | unsigned int | / |  | / |  |
| 25 | P43W15 | NH025 | Charge/Discharge ratio | %6.2f | Func0010 | 0.01,0 | unsigned int | / |  | / |  |
| 26 | P43W16 | NH026 | Battery voltage alarm threshold | %6.2f | Func0010 | 0.1576,0.051 | unsigned int | / |  | / |  |
| 27 | P43W17 | NH027 | Battery voltage overdischarge threshold | %6.2f | Func0010 | 0.1576,0.051 | unsigned int | / |  | / |  |
| 28 | P43W18 | NH028 | Battery voltage pre-alarm threshold | %6.2f | Func0010 | 0.1576,0.051 | unsigned int | / |  | / |  |
| 29 | P43W19 | NH029 | Battery capacity alarm threshold | %6.2f | Func0010 | 0.256,0 | unsigned int | / |  | / |  |
| 30 | P43W20~W21 | NH030 | The key parameter three takes two states | %u | Func0001 | / | unsigned int | / |  | / |  |
| 31 | P43W22 | NH031 | CAN bus reset count | %u | Func0001 | / | unsigned int | / |  | / |  |
| 32 | P43W23~W25 | NH032 | Safe mode counting | %u | Func0001 | / | unsigned int | / |  | / |  |

AOCC

AOCC_fast_TM1(24H)

Pack ID=24H；Pack length=127（Bytes=128）。

Table 8-16AOCs_fast_TM1Table

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P24W0B7~B4 | TMK101 | Telemetry frame feature code | %s | Func0001 | / | unsigned int |  |  | 0x0:Fast Frame 1 Telemetry; 0x1:Fast Frame 2 Telemetry; 0x3:GPS Orbit Telemetry; 0x4:Injected Orbit Parameter Telemetry; 0x5:Fast Frame Telemetry of Memory Downloaded; 0x6:Slow Frame 1 Telemetry; 0x7:Slow Frame 2 Telemetry |  |
| 2 | P24W0B3~W3B0 | TMK102 | Satellite time | %+8.3f | Func0001 | / | unsigned int |  | (0,268435455) | / |  |
| 3 | P24W4 | TMK103 | Work mode word | %s | Func0001 | / | unsigned int |  |  | 0x21: Global Attitude Acquisition Mode(AAM) - angular velocity damping; 0x22: Global Attitude Acquisition Mode(AAM) - sun search; 0x23: Global Attitude Acquisition Mode(AAM) - sun orientation; 0x24: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Earth Pointing Mode(EPM); 0x25: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Sun Pointing Mode(SPM); 0x30: Sun Pointing Mode(SPM); 0x43: Maneuver Mode - towards to Sun Pointing Mode(SPM); 0x45: Maneuver Mode - towards to Earth Pointing Mode(EPM); 0x46: Maneuver Mode - towards to Attitude Maintaining Mode(AMM); 0x50: Earth Pointing Mode(EPM); 0x60: Attitude Maintaining Mode(AMM); 0xC0: Stop Control Mode(SCM); |  |
| 4 | P24W5~W6 | TMK104 | Roll angle estimate | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 5 | P24W7~W8 | TMK105 | Pitch angle estimate | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 6 | P24W9~W10 | TMK106 | Yaw angle estimate | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 7 | P24W11~W12 | TMK107 | Roll angular velocity estimate | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 8 | P24W13~W14 | TMK108 | Pitch angular velocity estimate | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 9 | P24W15~W16 | TMK109 | Yaw angular velocity estimate | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 10 | P24W17~W18 | TMK110 | Roll angle controlled variable | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 11 | P24W19~W20 | TMK111 | Pitch angle controlled variable | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 12 | P24W21~W22 | TMK112 | Yaw angle controlled variable | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 13 | P24W23~W24 | TMK113 | Roll angular velocity controlled variable | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 14 | P24W25~W26 | TMK114 | Pitch angular velocity controlled variable | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 15 | P24W27~W28 | TMK115 | Yaw angular velocity controlled variable | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 16 | P24W29~W31 | TMK116 | Gyro X angular velocity | %+8.5f | Func0007 | 0.000053495 | complement | deg/s | (-15,15) | / |  |
| 17 | P24W32~W34 | TMK117 | Gyro Y angular velocity | %+8.5f | Func0007 | 0.000053248 | complement | deg/s | (-15,15) | / |  |
| 18 | P24W35~W37 | TMK118 | Gyro Z angular velocity | %+8.5f | Func0007 | 0.000052414 | complement | deg/s | (-15,15) | / |  |
| 19 | P24W38~W40 | TMK119 | Gyro S angular velocity | %+8.5f | Func0007 | 0.0000536064 | complement | deg/s | (-15,15) | / |  |
| 20 | P24W41B7 | TMK120_1 | See-sun flag of ASS-alpha angle | %s | Func0001 | / | unsigned int |  |  | 0x00:See the Sun; 0x01:Not see the sun |  |
| 21 | P24W41B6 | TMK120_2 | Sign of ASS-alpha angle | %s | Func0001 | / | unsigned int |  |  | 0x00:Negative angle; 0x01:Positive angle |  |
| 22 | P24W41B5~W42B0 | TMK120_3 | ASS-alpha angle data | %s | Func0017 | TMK120_2,0.005 | unsigned int |  | (-81.9,81.9) | / |  |
| 23 | P24W43B7 | TMK121_1 | See-sun flag of ASS-beta angle | %s | Func0001 | / | unsigned int |  |  | 0x00:See the Sun; 0x01:Not see the sun |  |
| 24 | P24W43B6 | TMK121_2 | Sign of ASS-beta angle | %s | Func0001 | / | unsigned int |  |  | 0x00:Negative angle; 0x01:Positive angle |  |
| 25 | P24W43B5~W44B0 | TMK121_3 | ASS-beta angle data | %s | Func0017 | TMK121_2,0.005 | unsigned int |  | (-81.9,81.9) | / |  |
| 26 | P24W45B7 | TMK122_1 | See-sun flag of DSS-alpha angle | %s | Func0001 | / | unsigned int |  |  | 0x00:See the Sun; 0x01:Not see the sun |  |
| 27 | P24W45B6 | TMK122_2 | Sign of DSS-alpha angle | %s | Func0001 | / | unsigned int |  |  | 0x00:Negative angle; 0x01:Positive angle |  |
| 28 | P24W45B5~W46B0 | TMK122_3 | DSS-alpha angle data | %s | Func0017 | TMK122_2,0.005 | unsigned int |  | (-81.9,81.9) | / |  |
| 29 | P24W47B7 | TMK123_1 | See-sun flag of DSS-beta angle | %s | Func0001 | / | unsigned int |  |  | 0x00:See the Sun; 0x01:Not see the sun |  |
| 30 | P24W47B6 | TMK123_2 | Sign of DSS-beta angle | %s | Func0001 | / | unsigned int |  |  | 0x00:Negative angle; 0x01:Positive angle |  |
| 31 | P24W47B5~W48B0 | TMK123_3 | DSS-beta angle data | %s | Func0017 | TMK123_2,0.005 | unsigned int |  | (-81.9,81.9) | / |  |
| 32 | P24W49~W50 | TMK124 | Magnetometer AX axis output | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 33 | P24W51~W52 | TMK125 | Magnetometer AY axis output | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 34 | P24W53~W54 | TMK126 | Magnetometer AZ axis output | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 35 | P24W55~W56 | TMK127 | Magnetometer BX axis output | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 36 | P24W57~W58 | TMK128 | Magnetometer BY axis output | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 37 | P24W59~W60 | TMK129 | Magnetometer BZ axis output | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 38 | P24W61~W62 | TMK130 | Star sensor A quaternion Q1 | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 39 | P24W63~W64 | TMK131 | Star sensor A quaternion Q2 | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 40 | P24W65~W66 | TMK132 | Star sensor A quaternion Q3 | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 41 | P24W67~W68 | TMK133 | Star sensor A quaternion Q0 | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 42 | P24W69 | TMK134 | Star sensor A sync pulse counter | %+8.3f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 43 | P24W70~W71 | TMK135 | Star sensor A data generation time | %+8.5f | Func0001 | / | unsigned int |  | (0,1000) | / |  |
| 44 | P24W72 | TMK136 | Star sensor A observation star number | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 45 | P24W73 | TMK137 | Star sensor A recognition star number | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 46 | P24W74B7 | TMK138_1 | Star sensor A_ attitude data validity | %s | Func0001 | / | unsigned int |  |  | 0x00:Attitude information is not available; 0x01:Attitude information is normal |  |
| 47 | P24W74B6 | TMK138_2 | Star sensor A_ strong light interference | %s | Func0001 | / | unsigned int |  |  | 0x00:No strong light interference was found; 0x01:Strong light interference was found |  |
| 48 | P24W74B5 | TMK138_3 | Star sensor A_ time overflow | %s | Func0001 | / | unsigned int |  |  | 0x00:No time overflow; 0x01:Time overflow |  |
| 49 | P24W74B4~B3 | TMK138_4 | star sensor A_Omega rate quality | %s | Func0001 | / | unsigned int |  |  | 0x00: Unavailable Rate; 0x01: Rate from Attitude Difference; 0x02: Rate from MEMS Gyroscope; 0x03: Rate from Gyroscope and Attitude Information Filter; |  |
| 50 | P24W74B2~B0 | TMK138_5 | Star sensor A_ working mode | %s | Func0001 | / | unsigned int |  |  | 0x01:STANDBY; 0x02:MODE_A; 0x03:MODE_B; 0x04:MODE_C; 0x05:IMAGE; 0x06:CLIBRA |  |
| 51 | P24W75~W76 | TMK139 | Star sensor B quaternion Q1 | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 52 | P24W77~W78 | TMK140 | Star sensor B quaternion Q2 | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 53 | P24W79~W80 | TMK141 | Star sensor B quaternion Q3 | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 54 | P24W81~W82 | TMK142 | Star sensor B quaternion Q0 | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 55 | P24W83 | TMK143 | Star sensor B sync pulse counter | %+8.3f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 56 | P24W84~W85 | TMK144 | Star sensor B data generation time | %+8.5f | Func0001 | / | unsigned int |  | (0,1000) | / |  |
| 57 | P24W86 | TMK145 | Star sensor B observation star number | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 58 | P24W87 | TMK146 | Star sensor B identification star number | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 59 | P24W88B7 | TMK147_1 | Star sensor B_ attitude data validity | %s | Func0001 | / | unsigned int |  |  | 0x00:Attitude information is not available; 0x01:Attitude information is normal |  |
| 60 | P24W88B6 | TMK147_2 | Star sensor B_ strong light interference | %s | Func0001 | / | unsigned int |  |  | 0x00:No strong light interference was found; 0x01:Strong light interference was found |  |
| 61 | P24W88B5 | TMK147_3 | Star sensor B_ time overflow | %s | Func0001 | / | unsigned int |  |  | 0x00:No time overflow; 0x01:Time overflow |  |
| 62 | P24W88B4~B3 | TMK147_4 | Star sensor B_Omega rate quality | %s | Func0001 | / | unsigned int |  |  | 0x00: Unavailable Rate; 0x01: Rate from Attitude Difference; 0x02: Rate from MEMS Gyroscope; 0x03: Rate from Gyroscope and Attitude Information Filter; |  |
| 63 | P24W88B2~B0 | TMK147_5 | Star sensor B_ working mode | %s | Func0001 | / | unsigned int |  |  | 0x01:STANDBY;0x02:MODE_A;0x03:MODE_B;0x04:MODE_C;0x05:IMAGE;0x06:CLIBRA |  |
| 64 | P24W89B7~W90B4 | TMK148 | Momentum wheel X control torque | %+8.5f | Func0007 | 0.01 | complement |  | (-20.4,20.4) | / |  |
| 65 | P24W90B3~W91B0 | TMK149 | Momentum wheel Y control torque | %+8.5f | Func0007 | 0.01 | complement |  | (-20.4,20.4) | / |  |
| 66 | P24W92B7~W93B4 | TMK150 | Momentum wheel Z control torque | %+8.5f | Func0007 | 0.01 | complement |  | (-20.4,20.4) | / |  |
| 67 | P24W93B3~W94B0 | TMK151 | Momentum wheel S control torque | %+8.5f | Func0007 | 0.01 | complement |  | (-20.4,20.4) | / |  |
| 68 | P24W95~W96 | TMK152 | Momentum wheel X speed | %+8.5f | Func0007 | 0.25 | complement |  | (-8191.75,8191.75) | / |  |
| 69 | P24W97~W98 | TMK153 | Momentum wheel Y speed | %+8.5f | Func0007 | 0.25 | complement |  | (-8191.75,8191.75) | / |  |
| 70 | P24W99~W100 | TMK154 | Momentum wheel Z speed | %+8.5f | Func0007 | 0.25 | complement |  | (-8191.75,8191.75) | / |  |
| 71 | P24W101~W102 | TMK155 | Momentum wheel S speed | %+8.5f | Func0007 | 0.25 | complement |  | (-8191.75,8191.75) | / |  |
| 72 | P24W103 | TMK156 | Momentum wheel X net torque | %+8.5f | Func0007 | 0.126 | complement |  | (-16,16) | / |  |
| 73 | P24W104 | TMK157 | Momentum wheel Y net torque | %+8.5f | Func0007 | 0.126 | complement |  | (-16,16) | / |  |
| 74 | P24W105 | TMK158 | Momentum wheel Z net torque | %+8.5f | Func0007 | 0.126 | complement |  | (-16,16) | / |  |
| 75 | P24W106 | TMK159 | Momentum wheel S net torque | %+8.5f | Func0007 | 0.126 | complement |  | (-16,16) | / |  |
| 76 | P24W107 | TMK160 | Magnetic torque X pulse width | %+8.5f | Func0007 | 2 | complement |  | (-250,250) | / |  |
| 77 | P24W108 | TMK161 | Magnetic torque Y pulse width | %+8.5f | Func0007 | 2 | complement |  | (-250,250) | / |  |
| 78 | P24W109 | TMK162 | Magnetic torque Z pulse width | %+8.5f | Func0007 | 2 | complement |  | (-250,250) | / |  |
| 79 | P24W110 | TMK163 | Magnetic torque X electric current | %+8.3f | Func0007 | 0.03921568627450980392156862745098 | complement |  | (-5,5) | / |  |
| 80 | P24W111 | TMK164 | Magnetic torque Y electric current | %+8.3f | Func0007 | 0.03921568627450980392156862745098 | complement |  | (-5,5) | / |  |
| 81 | P24W112 | TMK165 | Magnetic torque Z electric current | %+8.3f | Func0007 | 0.03921568627450980392156862745098 | complement | μs | (-5,5) | / |  |
| 82 | P24W113B7 | TMK166_1 | Gyro X angular velocity state | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal angular velocity; 0x01:Saturated angular velocity |  |
| 83 | P24W113B6 | TMK166_2 | Gyro X optical power state | %s | Func0001 | / | unsigned int |  |  | 0x00:Optical power is normal; 0x01:Optical power is excessive |  |
| 84 | P24W113B5 | TMK166_3 | Gyro Y angular velocity state | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal angular velocity; 0x01:Saturated angular velocity |  |
| 85 | P24W113B4 | TMK166_4 | Gyro Y optical power state | %s | Func0001 | / | unsigned int |  |  | 0x00:Optical power is normal; 0x01:Optical power is excessive |  |
| 86 | P24W113B3 | TMK166_5 | Gyro Z angular velocity state | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal angular velocity; 0x01:Saturated angular velocity |  |
| 87 | P24W113B2 | TMK166_6 | Gyro Z optical power state | %s | Func0001 | / | unsigned int |  |  | 0x00:Optical power is normal; 0x01:Optical power is excessive |  |
| 88 | P24W113B1 | TMK166_7 | Gyro S angular velocity state | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal angular velocity; 0x01:Saturated angular velocity |  |
| 89 | P24W113B0 | TMK166_8 | Gyro S optical power state | %s | Func0001 | / | unsigned int |  |  | 0x00:Optical power is normal; 0x01:Optical power is excessive |  |
| 90 | P24W114B7 | TMK167_1 | Star sensor A power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 91 | P24W114B6 | TMK167_2 | Star sensor B power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 92 | P24W114B5 | TMK167_3 | Gyro X power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 93 | P24W114B4 | TMK167_4 | Gyro Y power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 94 | P24W114B3 | TMK167_5 | Gyro Z power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 95 | P24W114B2 | TMK167_6 | Gyro S power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 96 | P24W114B1 | TMK167_7 | Magnetometer A power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 97 | P24W114B0 | TMK167_8 | Magnetometer B power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 98 | P24W115B7 | TMK168_1 | Momentum wheel X power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 99 | P24W115B6 | TMK168_2 | Momentum wheel Y power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 100 | P24W115B5 | TMK168_3 | Momentum wheel Z power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 101 | P24W115B4 | TMK168_4 | Momentum wheel S power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 102 | P24W115B3 | TMK168_5 | Magnetic torque X power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 103 | P24W115B2 | TMK168_6 | Magnetic torque Y power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 104 | P24W115B1 | TMK168_7 | Magnetic torque Z power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 105 | P24W115B0 | TMK168_8 | On-chip sun sensor(DSS) power-on state | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 106 | P24W116B7 | TMK169_1 | Frame0_ power-on reset flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Power on; 0x01:Reset |  |
| 107 | P24W116B6 | TMK169_2 | Frame0_continuous reset flag | %s | Func0001 | / | unsigned int |  |  | 0x00:The number of consecutive resets is less than or equal to 5; 0x01:The number of consecutive resets is more than 5 |  |
| 108 | P24W116B5 | TMK169_3 | Frame0_ program loading flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Valid; 0x01:Invalid |  |
| 109 | P24W116B4~B2 | TMK169_4 | Frame0_ control task timeouts | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 110 | P24W116B1~W117B7 | TMK169_5 | Frame0_Alternate | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 111 | P24W117B6 | TMK169_6 | Frame0_accumulation and status flags | %s | Func0001 | / | unsigned int |  |  | 0x00:Correct; 0x01:Incorrect |  |
| 112 | P24W117B5~B3 | TMK169_7 | Frame0_Alternate | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 113 | P24W117B5 | TMK169_D21 | FLASH non-boot program area 1 status | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal; 0x01:Abnormal |  |
| 114 | P24W117B4 | TMK169_D20 | FLASH non-boot program area 2 status | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal; 0x01:Abnormal |  |
| 115 | P24W117B3 | TMK169_D19 | FLASH non-boot program area 3 status | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal; 0x01:Abnormal |  |
| 116 | P24W117B2 | TMK169_8 | Frame0_RAM status flag | %s | Func0001 | / | unsigned int |  |  | 0x00:RAM is normal; 0x01:RAM is abnormal |  |
| 117 | P24W117B1~W118B6 | TMK169_9 | Single-particle flipping number single error | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 118 | P24W118B5~B4 | TMK169_10 | Single-particle flipping number double error | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 119 | P24W118B3 | TMK169_11 | FLASH non-boot area total status | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal; 0x01:Abnormal |  |
| 120 | P24W118B2~B0 | TMK169_12 | FLASH non-boot area 1 error count | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 121 | P24W119B7~B5 | TMK169_13 | FLASH non-boot area 2 error count | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 122 | P24W119B4~B2 | TMK169_14 | FLASH non-boot area 3 error count | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 123 | P24W119B1~B0 | TMK169_15 | Software active reset cumulative count | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 124 | P24W120B7~B3 | TMK170_1 | Frame1_ control task running time | %s | Func0001 | / | unsigned int |  |  | 0x00: Between 0ms and 10ms; 0x01: Between 10ms and 20ms; 0x02: Between 20ms and 30ms; 0x03: Between 30ms and 40ms; 0x04: Between 40ms and 50ms; 0x05: Between 50ms and 60ms; 0x06: Between 60ms and 70ms; 0x07: Between 70ms and 80ms; 0x08: Between 80ms and 90ms; 0x09: Between 90ms and 100ms; 0x0A: Between 100ms and 110ms; 0x0B: Between 110ms and 120ms; 0x0C: Between 120ms and 130ms; 0x0D: Between 130ms and 140 ms; 0x0E: Between 140ms and 150 ms; 0x0F: Between 150ms and 160 ms; 0x10: Between 160ms and 170 ms; 0x11: Between 170ms and 180 ms; 0x12: Between 180ms and 190 ms; 0x13: Between 190ms and 200 ms; 0x14: Between 200ms and 210 ms; 0x15: Between 210ms and 220 ms; 0x16: Between 220ms and 230 ms; 0x17: Between 230ms and 240 ms; 0x1F: Control Task Running Timeout |  |
| 125 | P24W120B2~B0 | TMK170_2 | Frame1_Reserved | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 126 | P24W121B7~B6 | TMK170_3 | Frame1_System Management Task Run Time | %s | Func0001 | / | unsigned int |  |  | 0x00:Between 0ms and 10ms; 0x01:Between 10ms and 20ms; 0x03:System management tasks run timeout |  |
| 127 | P24W121B5~B4 | TMK170_4 | Frame1_System Management Task Timeouts | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 128 | P24W121B3 | TMK170_5 | Frame1_ system management task downgrade operation | %s | Func0001 | / | unsigned int |  |  | 0x00:Full Function Operation; 0x01:Degraded Function Operation |  |
| 129 | P24W121B2 | TMK170_6 | Frame1_ controls the task stack status | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal; 0x01:Abnormal |  |
| 130 | P24W121B1 | TMK170_7 | Frame1_Reserved | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 131 | P24W121B0 | TMK170_8 | Frame1_System Management Task Stack Status | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal; 0x01:Abnormal |  |
| 132 | P24W122B7 | TMK170_9 | Frame1_Monitor_Run Flag | %s | Func0001 | / | unsigned int |  |  | 0x01:Monitoring; 0x00:Running |  |
| 133 | P24W122B6 | TMK170_10 | Frame1_Idle Task Stack Status | %s | Func0001 | / | unsigned int |  |  | 0x00:Normal; 0x01:Abnormal |  |
| 134 | P24W122B5 | TMK170_11 | Frame1_Reserved | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 135 | P24W122B4~B2 | TMK170_12 | Frame1_Reset cumulative count | %+8.3f | Func0001 | / | unsigned int |  |  | / |  |
| 136 | P24W122B1 | TMK170_13 | Frame1_FLASH self-test status | %s | Func0001 | / | unsigned int |  |  | 0x00:Not self-checked; 0x01:Self-checked |  |
| 137 | P24W122B0 | TMK170_14 | Frame1_FLASH repair status | %s | Func0001 | / | unsigned int |  |  | 0x00: Forbidden; 0x01:Enable |  |
| 138 | P24W123B7 | TMK170_15 | Frame1_auto-switch enable_forbidden command word | %s | Func0001 | / | unsigned int |  |  | 0x0:Forbidden; 0x1:Enabled |  |
| 139 | P24W123B6 | TMK170_16 | Frame1_switch command word | %s | Func0001 | / | unsigned int |  |  | 0x0:Invalid switch; 0x1:Valid switch |  |
| 140 | P24W123B5 | TMK170_17 | Frame1_CCU AB flag | %s | Func0001 | / | unsigned int |  |  | 0x0:A; 0x1:B |  |
| 141 | P24W123B4 | TMK170_18 | Frame1_CCU A power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 142 | P24W123B3 | TMK170_19 | Frame1_CCU B power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 143 | P24W123B2 | TMK170_20 | Frame1_FT power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 144 | P24W123B1 | TMK170_21 | Frame1_ fiber optic gyroscope main power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 145 | P24W123B0 | TMK170_22 | Frame1_ fiber optic gyroscope backup power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Power off; 0x01:Power on |  |
| 146 | P24W124 | TMK171 | Injection successful accumulation times | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 147 | P24W125 | TMK172 | Number of data instructions received | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 148 | P24W126 | TMK173 | Newly received data instructions | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 149 | P24W127 | TMK174 | Request to return important data | %s | Func0001 | / | unsigned int |  |  | 0x55:Request the satellite subsystem to send back important data |  |

AOCC_fast_TM2(24H)

Pack ID=24H；Pack length=127（Bytes=128）。

Table 8-17AOCs_fast_TM2Table

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P24W0B7~B4 | TMK201 | Telemetry frame feature code | %s | Func0001 | / | unsigned int |  |  | 0x0:Fast Frame 1 Telemetry; 0x1:Fast Frame 2 Telemetry; 0x3:GPS Orbit Telemetry; 0x4:Injected Orbit Parameter Telemetry; 0x5:Fast Frame Telemetry of Memory Downloaded; 0x6:Slow Frame 1 Telemetry; 0x7:Slow Frame 2 Telemetry |  |
| 2 | P24W0B3~W3B0 | TMK202 | Satellite time | %+8.3f | Func0001 | / | unsigned int |  | (0,268435455) | / |  |
| 3 | P24W4 | TMK203 | Work mode word | %s | Func0001 | / | unsigned int |  |  | 0x21: Global Attitude Acquisition Mode(AAM) - angular velocity damping; 0x22: Global Attitude Acquisition Mode(AAM) - sun search; 0x23: Global Attitude Acquisition Mode(AAM) - sun orientation; 0x24: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Earth Pointing Mode(EPM); 0x25: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Sun Pointing Mode(SPM); 0x30: Sun Pointing Mode(SPM); 0x43: Maneuver Mode - towards to Sun Pointing Mode(SPM); 0x45: Maneuver Mode - towards to Earth Pointing Mode(EPM); 0x46: Maneuver Mode - towards to Attitude Maintaining Mode(AMM); 0x50: Earth Pointing Mode(EPM); 0x60: Attitude Maintaining Mode(AMM); 0xC0: Stop Control Mode(SCM); |  |
| 4 | P24W5~W6 | TMK204 | Roll angle estimate | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 5 | P24W7~W8 | TMK205 | Pitch angle estimate | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 6 | P24W9~W10 | TMK206 | Yaw angle estimate | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 7 | P24W11~W12 | TMK207 | Roll angular velocity estimate | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 8 | P24W13~W14 | TMK208 | Pitch angular velocity estimate | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 9 | P24W15~W16 | TMK209 | Yaw angular velocity estimate | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 10 | P24W17~W18 | TMK210 | Roll angle controlled variable | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 11 | P24W19~W20 | TMK211 | Pitch angle controlled variable | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 12 | P24W21~W22 | TMK212 | Yaw angle controlled variable | %+8.5f | Func0007 | 0.0055 | complement | deg | (-180,180) | / |  |
| 13 | P24W23~W24 | TMK213 | Roll angular velocity controlled variable | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 14 | P24W25~W26 | TMK214 | Pitch angular velocity controlled variable | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 15 | P24W27~W28 | TMK215 | Yaw angular velocity controlled variable | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 16 | P24W29 | TMK216 | Continuous unsuccessful number of communication with Gyro X | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 17 | P24W30 | TMK217 | Continuous unsuccessful number of communication with Gyro Y | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 18 | P24W31 | TMK218 | Continuous unsuccessful number of communication with Gyro Z | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 19 | P24W32 | TMK219 | Continuous unsuccessful number of communication with Gyro S | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 20 | P24W33 | TMK220 | Continuous unsuccessful communication with magnetometer A | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 21 | P24W34 | TMK221 | Continuous unsuccessful communication with magnetometer B | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 22 | P24W35 | TMK222 | Continuous unsuccessful communication with the on-chip sun sensor(DSS) | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 23 | P24W36 | TMK223 | Continuous unsuccessful communication with star sensor A | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 24 | P24W37 | TMK224 | Continuous unsuccessful communication with star sensor B | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 25 | P24W38 | TMK225 | Continuous unsuccessful number of communication with momentum wheel X | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 26 | P24W39 | TMK226 | Continuous unsuccessful number of communication with momentum wheel Y | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 27 | P24W40 | TMK227 | Continuous unsuccessful number of communication with momentum wheel Z | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 28 | P24W41 | TMK228 | Continuous unsuccessful number of communication with momentum wheel S | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 29 | P24W42~W43 | TMK229 | The attitude quaternion q1 of the body relative to the inertial system | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 30 | P24W44~W45 | TMK230 | The attitude quaternion q2 of the body relative to the inertial system | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 31 | P24W46~W47 | TMK231 | The attitude quaternion q3 of the body relative to the inertial system | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 32 | P24W48~W49 | TMK232 | Quaternion q1 of the body calculated by Magnometer | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 33 | P24W50~W51 | TMK233 | Quaternion q2 of the body calculated by Magnometer | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 34 | P24W52~W53 | TMK234 | Quaternion q3 of the body calculated by Magnometer | %+8.5f | Func0007 | 0.00003051850947599719 | complement |  | (-1,1) | / |  |
| 35 | P24W54~W55 | TMK235 | Roll angle measured by star sensor | %+8.5f | Func0007 | 0.0055 | complement |  | (-180,180) | / |  |
| 36 | P24W56~W57 | TMK236 | Pitch angle measured by star sensor | %+8.5f | Func0007 | 0.0055 | complement |  | (-180,180) | / |  |
| 37 | P24W58~W59 | TMK237 | Yaw angle measured by star sensor | %+8.5f | Func0007 | 0.0055 | complement |  | (-180,180) | / |  |
| 38 | P24W60~W61 | TMK238 | Roll angle measured by double-vector | %+8.5f | Func0007 | 0.0055 | complement |  | (-180,180) | / |  |
| 39 | P24W62~W63 | TMK239 | Pitch angle measured by double-vector | %+8.5f | Func0007 | 0.0055 | complement |  | (-180,180) | / |  |
| 40 | P24W64~W65 | TMK240 | Yaw angle measured by double-vector | %+8.5f | Func0007 | 0.0055 | complement |  | (-180,180) | / |  |
| 41 | P24W66~W67 | TMK241 | Sox - component X of the Sun vector relative to the orbit | %+8.5f | Func0008 | 32767 | complement |  | (-1,1) | / |  |
| 42 | P24W68~W69 | TMK242 | Soy - component Y of the Sun vector relative to the orbit | %+8.5f | Func0008 | 32767 | complement |  | (-1,1) | / |  |
| 43 | P24W70~W71 | TMK243 | Soz - component Z of the Sun vector relative to the orbit | %+8.5f | Func0008 | 32767 | complement |  | (-1,1) | / |  |
| 44 | P24W72~W73 | TMK244 | Bbx - component X of the Magnetic field vector relative to the body | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 45 | P24W74~W75 | TMK245 | Bby - component Y of the Magnetic field vector relative to the body | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 46 | P24W76~W77 | TMK246 | Bbz - component Z of the Magnetic field vector relative to the body | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 47 | P24W78~W79 | TMK247 | Box - component X of the Magnetic field vector relative to the orbit | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 48 | P24W80~W81 | TMK248 | Boy - component Y of the Magnetic field vector relative to the orbit | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 49 | P24W82~W83 | TMK249 | Boz - component Z of the Magnetic field vector relative to the orbit | %+8.5f | Func0007 | 2 | complement |  | (-65534,65534) | / |  |
| 50 | P24W84~W85 | TMK250 | Magnetic angle Eta_m | %+8.5f | Func0007 | 0.0055 | complement |  | (-180,180) | / |  |
| 51 | P24W86 | TMK251 | Angle between star sensor A's optical axis and the sun vector | %+8.5f | Func0007 | 0.7 | unsigned int |  | (0,180) | / |  |
| 52 | P24W87 | TMK252 | Angle between star sensor A's optical axis and the horizon | %+8.5f | Func0007 | 1 | complement |  | (-69,120) | / |  |
| 53 | P24W88 | TMK253 | Angle between star sensor B's optical axis and the sun vector | %+8.5f | Func0007 | 0.7 | unsigned int |  | (0,180) | / |  |
| 54 | P24W89 | TMK254 | Angle between star sensor B's optical axis and the horizon | %+8.5f | Func0007 | 1 | complement |  | (-69,120) | / |  |
| 55 | P24W90 | TMK255 | Attitude maneuver preset time | %+8.5f | Func0007 | 2 | unsigned int |  |  | / |  |
| 56 | P24W91 | TMK256 | Star sensor A exposure and current satellite time difference | %+8.5f | Func0007 | 2 | unsigned int |  | (0,510) | / |  |
| 57 | P24W92 | TMK257 | Star sensor B exposure and current satellite time difference | %+8.5f | Func0007 | 2 | unsigned int |  | (0,510) | / |  |
| 58 | P24W93~W94 | TMK258 | Drift angle | %+8.5f | Func0007 | 0.0055 | complement |  | (-180,180) | / |  |
| 59 | P24W95 | TMK260 | ASS angle-alpha angle calculated by star sensors | %+8.5f | Func0007 | 0.55 | complement |  | (-69.85,69.85) | / |  |
| 60 | P24W96 | TMK261 | ASS angle-beta angle calculated by star sensors | %+8.5f | Func0007 | 0.55 | complement |  | (-69.85,69.85) | / |  |
| 61 | P24W97 | TMK262 | DSS angle-alpha angle calculated by star sensors | %+8.5f | Func0007 | 0.55 | complement |  | (-69.85,69.85) | / |  |
| 62 | P24W98 | TMK263 | DSS angle-beta angle calculated by star sensors | %+8.5f | Func0007 | 0.55 | complement |  | (-69.85,69.85) | / |  |
| 63 | P24W99 | TMK264 | ASS angle-alpha angle calculated by satellit atittude | %+8.5f | Func0007 | 0.55 | complement |  | (-69.85,69.85) | / |  |
| 64 | P24W100 | TMK265 | ASS angle-beta angle calculated by satellit atittude | %+8.5f | Func0007 | 0.55 | complement |  | (-69.85,69.85) | / |  |
| 65 | P24W101 | TMK266 | DSS angle-alpha angle calculated by satellit atittude | %+8.5f | Func0007 | 0.55 | complement |  | (-69.85,69.85) | / |  |
| 66 | P24W102 | TMK267 | DSS angle-beta angle calculated by satellit atittude | %+8.5f | Func0007 | 0.55 | complement |  | (-69.85,69.85) | / |  |
| 67 | P24W103B7 | TMK268_1 | Gyro S health state | %s | Func0001 | / | unsigned int |  |  | 0x00:Unhealthy; 0x01:Healthy |  |
| 68 | P24W103B6 | TMK268_2 | Gyro Z health state | %s | Func0001 | / | unsigned int |  |  | 0x00:Unhealthy; 0x01:Healthy |  |
| 69 | P24W103B5 | TMK268_3 | Gyro Y health state | %s | Func0001 | / | unsigned int |  |  | 0x00:Unhealthy; 0x01:Healthy |  |
| 70 | P24W103B4 | TMK268_4 | Gyro X health state | %s | Func0001 | / | unsigned int |  |  | 0x00:Unhealthy; 0x01:Healthy |  |
| 71 | P24W103B3 | TMK268_5 | Gyro S introduction state | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 72 | P24W103B2 | TMK268_6 | Gyro Z introduction state | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 73 | P24W103B1 | TMK268_7 | Gyro Y introduction state | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 74 | P24W103B0 | TMK268_8 | Gyro X introduction state | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 75 | P24W104B7~B6 | TMK269_1 | Sun sensors health state | %s | Func0001 | / | unsigned int |  |  | 0x00: Both sun sensors are healthy; 0x01: Only ASS is healthy; 0x02: Only DSS is healthy; 0x03: Both sun sensors are unhealthy; |  |
| 76 | P24W104B5~B4 | TMK269_2 | Sun sensors valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Both sun sensors are valid; 0x01: Only ASS is valid; 0x02: Only DSS is valid; 0x03: Both sun sensors are invalid; |  |
| 77 | P24W104B3 | TMK269_3 | Gyro S valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 78 | P24W104B2 | TMK269_4 | Gyro Z valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 79 | P24W104B1 | TMK269_5 | Gyro Y valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 80 | P24W104B0 | TMK269_6 | Gyro X valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 81 | P24W105B7~B6 | TMK270_1 | Star sensors using flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Both STS A and STS B are not in use; 0x01: STS A is in use; 0x02: STS B is in use; 0x03: Both STS A and STS B are in use |  |
| 82 | P24W105B5~B4 | TMK270_2 | Star sensors working state | %s | Func0001 | / | unsigned int |  |  | 0x00: Both STS A and STS B are not at work; 0x01: STS A is at work; 0x02: STS B is at work; 0x03: Both STS A and STS B are at work |  |
| 83 | P24W105B3 | TMK270_3 | Star sensor A valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 84 | P24W105B2 | TMK270_4 | Star sensor B valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 85 | P24W105B1 | TMK270_5 | NULL | %s | Func0001 | / | unsigned int |  |  | / |  |
| 86 | P24W105B0 | TMK270_6 | Second pulse valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 87 | P24W106B7 | TMK271_1 | Momentum wheel S introduction state | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 88 | P24W106B6 | TMK271_2 | Momentum wheel Z introduction state | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 89 | P24W106B5 | TMK271_3 | Momentum wheel Y introduction state | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 90 | P24W106B4 | TMK271_4 | Momentum wheel X introduction state | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 91 | P24W106B3 | TMK271_5 | Momentum wheel S health state | %s | Func0001 | / | unsigned int |  |  | 0x00: Abnormal; 0x01: Normal |  |
| 92 | P24W106B2 | TMK271_6 | Momentum wheel Z health state | %s | Func0001 | / | unsigned int |  |  | 0x00: Abnormal; 0x01: Normal |  |
| 93 | P24W106B1 | TMK271_7 | Momentum wheel Y health state | %s | Func0001 | / | unsigned int |  |  | 0x00: Abnormal; 0x01: Normal |  |
| 94 | P24W106B0 | TMK271_8 | Momentum wheel X health state | %s | Func0001 | / | unsigned int |  |  | 0x00: Abnormal; 0x01: Normal |  |
| 95 | P24W107B7 | TMK272_1 | Orbit parameters uploading failure flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Success; 0x01: Failure |  |
| 96 | P24W107B6~B4 | TMK272_2 | Correcting time flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Prohibited proofreading time; 0x01: Uniform proofreading time; 0x02: Full proofreading time; 0x03: Incremental proofreading time; 0x04: Starcraft separation proofreading time; 0x05: Starcraft UTC proofreading time; 0x06: GPS proofreading time |  |
| 97 | P24W107B3 | TMK272_3 | Communication success flag with terrestrial serial port | %s | Func0001 | / | unsigned int |  |  | 0x00: Success; 0x01: Failure |  |
| 98 | P24W107B2 | TMK272_4 | Feature parameter saving instruction is disable or started | %s | Func0001 | / | unsigned int |  |  | 0x00: Disable; 0x01: Started |  |
| 99 | P24W107B1 | TMK272_5 | Ground test-instruction permission flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Forbidden; 0x01: Enabled |  |
| 100 | P24W107B0 | TMK272_6 | Ground closed loop test enabled state | %s | Func0001 | / | unsigned int |  |  | 0x00: Forbidden; 0x01: Enabled |  |
| 101 | P24W108B7 | TMK273_1 | Control computer anti-radiation-self-diagnosis flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 102 | P24W108B6 | TMK273_2 | Gyro X autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 103 | P24W108B5 | TMK273_3 | Gyro Y autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 104 | P24W108B4 | TMK273_4 | Gyro Z autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 105 | P24W108B3 | TMK273_5 | Gyro S autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 106 | P24W108B2 | TMK273_6 | Gyro anti-radiation-self-diagnosis flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 107 | P24W108B1 | TMK273_7 | Magnetometer A autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 108 | P24W108B0 | TMK273_8 | Magnetometer B autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 109 | P24W109B7 | TMK274_1 | DSS autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 110 | P24W109B6 | TMK274_2 | Star sensor A autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 111 | P24W109B5 | TMK274_3 | Star sensor B autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 112 | P24W109B4 | TMK274_4 | Star sensor anti-radiation-self-diagnosis flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 113 | P24W109B3 | TMK274_5 | Momentum wheel X autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 114 | P24W109B2 | TMK274_6 | Momentum wheel Y autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 115 | P24W109B1 | TMK274_7 | Momentum wheel Z autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 116 | P24W109B0 | TMK274_8 | Momentum wheel S autonomous power-on flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 117 | P24W110B7 | TMK275_1 | Momentum wheel anti-radiation-self-diagnosis flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Allowed; 0x01: Not allowed |  |
| 118 | P24W110B6 | TMK275_2 | GPS orbit calculation introduction flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 119 | P24W110B5 | TMK275_3 | GPS orbit check introduction flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 120 | P24W110B4 | TMK275_4 | GPS orbit data valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 121 | P24W110B3~B1 | TMK275_5 | Reset entry | %s | Func0001 | / | unsigned int |  |  | 0x00: Reserved; 0x01: Initialization mode 1; 0x02: Initialization mode 2; 0x03: Initialization mode 3; 0x04: Initialization mode 4; 0x05: Initialization mode 5 |  |
| 122 | P24W110B0 | TMK275_6 | CAN bus status | %s | Func0001 | / | unsigned int |  |  | 0x00: B bus; 0x01: A bus |  |
| 123 | P24W111B7 | TMK276_1 | Magnetometer self-diagnosis flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 124 | P24W111B6 | TMK276_2 | Momentum wheel self-diagnosis flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 125 | P24W111B5 | TMK276_3 | Star sensor self-diagnosis flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 126 | P24W111B4 | TMK276_4 | Gyro self-diagnosis flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 127 | P24W111B3 | TMK276_5 | Star sensor installation deviation calculation flag | %s | Func0001 | / | unsigned int |  |  | 0x00: No calculation; 0x01: Calculation |  |
| 128 | P24W111B2 | TMK276_6 | Star sensor installation deviation correction flag | %s | Func0001 | / | unsigned int |  |  | 0x00: No correction; 0x01: Correction |  |
| 129 | P24W111B1 | TMK276_7 | Star sensor A light aberration correction permission flag | %s | Func0001 | / | unsigned int |  |  | 0x00: No correction; 0x01: Correction |  |
| 130 | P24W111B0 | TMK276_8 | Star sensor B light aberration correction permission flag | %s | Func0001 | / | unsigned int |  |  | 0x00: No correction; 0x01: Correction |  |
| 131 | P24W112B7 | TMK277_1 | Rate damping method | %s | Func0001 | / | unsigned int |  |  | 0x00: Momentum wheel; 0x01: Magnetic Torque |  |
| 132 | P24W112B6 | TMK277_2 | Magnetic damping method | %s | Func0001 | / | unsigned int |  |  | 0x00: Gyro; 0x01: Magnetometer |  |
| 133 | P24W112B5 | TMK277_3 | Momentum wheel in-use flag in rate damping | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 134 | P24W112B4 | TMK277_4 | Sensor used for attitude adjustment to Earth Pointing Mode(EPM) | %s | Func0001 | / | unsigned int |  |  | 0x00: star sensor; 0x01: Magnetometer |  |
| 135 | P24W112B3 | TMK277_5 | Sensor used for attitude adjustment to Sun Pointing Mode(SPM) | %s | Func0001 | / | unsigned int |  |  | 0x00: star sensor; 0x01: Magnetometer |  |
| 136 | P24W112B2 | TMK277_6 | MagUnloadFlag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 137 | P24W112B1 | TMK277_7 | MagCalFlag | %s | Func0001 | / | unsigned int |  |  | 0x00: No use; 0x01: Use |  |
| 138 | P24W112B0 | TMK277_8 | Capture-earth command | %s | Func0001 | / | unsigned int |  |  | 0x00:Not captured;0x01:Captured |  |
| 139 | P24W113B7 | TMK278_1 | Capture-sun command | %s | Func0001 | / | unsigned int |  |  | 0x00:Not switch to;0x01:Switch to |  |
| 140 | P24W113B6 | TMK278_2 | SSCatchSunFlag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 141 | P24W113B5 | TMK278_3 | Magnetometer in-use flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Magnetometer A;0x01:Magnetometer B |  |
| 142 | P24W113B4 | TMK278_4 | Long term running attitude mode | %s | Func0001 | / | unsigned int |  |  | 0x00: Sun Pointing Mode(SPM); 0x01: Earth Pointing Mode(EPM) |  |
| 143 | P24W113B3 | TMK278_5 | Auto-switch flag between EPM and SPM | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 144 | P24W113B2 | TMK278_6 | Drift angle introduction flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 145 | P24W113B1 | TMK278_7 | Quaternion filter mode flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Double vectors; 0x01: Magnetometer |  |
| 146 | P24W113B0 | TMK278_8 | Attitude determination pattern auto-switch flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 147 | P24W114B7~B6 | TMK279_1 | Yaw calibration direction flag | %s | Func0001 | / | unsigned int |  |  | 0x00:No maneuver;0x01:Positive 90;0x02:Negative 90 |  |
| 148 | P24W114B5 | TMK279_2 | STSUpdateQbiFlag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 149 | P24W114B4 | TMK279_3 | AttBiasValid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 150 | P24W114B3 | TMK279_4 | Magnetometer A fault flag | %s | Func0001 | / | unsigned int |  |  | 0x0: Abnormal; 0x1: Normal |  |
| 151 | P24W114B2 | TMK279_5 | AttDeterSHInitFlag | %s | Func0001 | / | unsigned int |  |  | 0x00:No reset;0x01:Reset |  |
| 152 | P24W114B1 | TMK279_6 | GPSEnforceUseFlag | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 153 | P24W114B0 | TMK279_7 | Magnetometer B fault flag | %s | Func0001 | / | unsigned int |  |  | 0x0: Abnormal; 0x1: Normal |  |
| 154 | P24W115B7 | TMK280_1 | Star sensor capture success flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not successful; 0x01: Successful |  |
| 155 | P24W115B6 | TMK280_2 | Geomagnetic capture success flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not successful; 0x01: Successful |  |
| 156 | P24W115B5 | TMK280_3 | SunZoneFlag | %s | Func0001 | / | unsigned int |  |  | 0x01:Solar Zone;0x00:Shadow Zone |  |
| 157 | P24W115B4 | TMK280_4 | DssMonitorFlag | %s | Func0001 | / | unsigned int |  |  | 0x00: No use; 0x01: Use |  |
| 158 | P24W115B3~B2 | TMK280_5 | MagCatchSunValidFlag | %s | Func0001 | / | unsigned int |  |  | 0x00:Not available; 0x01:Analog sun sensor(ASS) available; 0x02:On-chip digital sun sensor(DSS) available |  |
| 159 | P24W115B1~B0 | TMK280_6 | Attitude determination flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Others; 0x01:Star sensor(STS) and Gyro; 0x02:Star sensor(STS) without Gyro; 0x3:Magnetometer |  |
| 160 | P24W116B7 | TMK281_1 | GyroWaitForFaultDiag flag | %s | Func0001 | / | unsigned int |  |  | 0x00:No abnormality;0x01:Abnormality |  |
| 161 | P24W116B6 | TMK281_2 | Double-vector attitude determination valid flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Invalid; 0x01: Valid |  |
| 162 | P24W116B5 | TMK281_4 | MAM star sensor introduction flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 163 | P24W116B4 | TMK281_5 | AMM star sensor introduction flag | %s | Func0001 | / | unsigned int |  |  | 0x00:Not introduced; 0x01:Introduced |  |
| 164 | P24W116B3~B0 | TMK281_6 | Reasons of switch into AAM | %s | Func0001 | / | unsigned int |  |  | 0x00: Initialization mode 1; 0x01: Initialization mode 3; 0x02: Data instruction; 0x03: Attitude angle out-of-tolerance in Sun Pointing Mode(SPM); 0x04: Attitude angular velocity out-of-tolerance in Sun Pointing Mode(SPM); 0x05: STS invalid in Sun Pointing Mode(SPM); 0x06: Attitude angle out-of-tolerance in Earth Pointing Mode(EPM); 0x07: Attitude angular velovity out-of-tolerance in Earth Pointing Mode(EPM); 0x08: STS invalid in Earth Pointing Mode(EPM); 0x09: Attitude maneuver overtime; 0x0A: Bias-flight overtime; 0x0B: Sun sensor invalid in Sun Pointing Mode(EPM) |  |
| 165 | P24W117 | TMK282 | Program injection frame identifier | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 166 | P24W118~W119 | TMK283 | Satellite time difference | %+8.5f | Func0007 | 100 | complement |  | (-3.2767,3.2767) | / |  |
| 167 | P24W120B7 | TMK284_7 | Gyro X optical signal input state | %s | Func0001 | / | unsigned int |  |  | 0x00:Yes; 0x01:No |  |
| 168 | P24W120B6 | TMK284_6 | Gyro Y optical signal input state | %s | Func0001 | / | unsigned int |  |  | 0x00:Yes; 0x01:No |  |
| 169 | P24W120B5 | TMK284_5 | Gyro Z optical signal input state | %s | Func0001 | / | unsigned int |  |  | 0x00:Yes; 0x01:No |  |
| 170 | P24W120B4 | TMK284_4 | Gyro S optical signal input state | %s | Func0001 | / | unsigned int |  |  | 0x00:Yes; 0x01:No |  |
| 171 | P24W120B3 | TMK284_3 | Flag of allowing momentum wheel low-speed start using high current | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 172 | P24W120B2 | TMK284_2 | AOCC communication abnormal auto-switch flag | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 173 | P24W120B1~B0 | TMK284_1 | RSV | %s | Func0001 | / | unsigned int |  |  | 0x00: Not allowed; 0x01: Allowed |  |
| 174 | P24W121~W122 | TMK285 | Drift angular velocity | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 175 | P24W123~W124 | TMK286 | Gyro balance equation difference | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 176 | P24W125~W126 | TMK287 | Star sensor AB optical axis angle | %+8.5f | Func0007 | 0.0055 | complement |  | (-180,180) | / |  |
| 177 | P24W127 | TMK288 | Request to return important data | %s | Func0001 | / | unsigned int |  |  | 0x55: Request the satellite subsystem to send back important data |  |

AOCC_GPS_Orbit_Calculation_TM(24H)

Pack ID=24H；Pack length=127（Bytes=128）。

Table 8-18AOCC_GPS_Orbit_Calculation_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P24W0B7~B4 | TMK301 | Telemetry frame feature code | %s | Func0001 | / | unsigned int |  |  | 0x0:Fast Frame 1 Telemetry; 0x1:Fast Frame 2 Telemetry; 0x3:GPS Orbit Telemetry; 0x4:Injected Orbit Parameter Telemetry; 0x5:Fast Frame Telemetry of Memory Downloaded; 0x6:Slow Frame 1 Telemetry; 0x7:Slow Frame 2 Telemetry |  |
| 2 | P24W0B3~W3B0 | TMK302 | Satellite time | %+8.3f | Func0001 | / | unsigned int |  | (0,268435455) | / |  |
| 3 | P24W4 | TMK303 | Work mode word | %s | Func0001 | / | unsigned int |  |  | 0x21: Global Attitude Acquisition Mode(AAM) - angular velocity damping; 0x22: Global Attitude Acquisition Mode(AAM) - sun search; 0x23: Global Attitude Acquisition Mode(AAM) - sun orientation; 0x24: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Earth Pointing Mode(EPM); 0x25: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Sun Pointing Mode(SPM); 0x30: Sun Pointing Mode(SPM); 0x43: Maneuver Mode - towards to Sun Pointing Mode(SPM); 0x45: Maneuver Mode - towards to Earth Pointing Mode(EPM); 0x46: Maneuver Mode - towards to Attitude Maintaining Mode(AMM); 0x50: Earth Pointing Mode(EPM); 0x60: Attitude Maintaining Mode(AMM); 0xC0: Stop Control Mode(SCM); |  |
| 4 | P24W5~W8 | TMK304 | GPS time | %+8.5f | Func0001 | / | unsigned int |  | (0,4294967295) | / |  |
| 5 | P24W9~W12 | TMK305 | GPS position X | %+8.5f | Func0007 | 0.1 | complement |  | (-2147483647,2147483647) | / |  |
| 6 | P24W13~W16 | TMK306 | GPS position Y | %+8.5f | Func0007 | 0.1 | complement |  | (-2147483647,2147483647) | / |  |
| 7 | P24W17~W20 | TMK307 | GPS position Z | %+8.5f | Func0007 | 0.1 | complement |  | (-2147483647,2147483647) | / |  |
| 8 | P24W21~W24 | TMK308 | GPS speed X | %+8.5f | Func0007 | 0.01 | complement |  | (-2147483647,2147483647) | / |  |
| 9 | P24W25~W28 | TMK309 | GPS speed Y | %+8.5f | Func0007 | 0.01 | complement |  | (-2147483647,2147483647) | / |  |
| 10 | P24W29~W32 | TMK310 | GPS speed Z | %+8.5f | Func0007 | 0.01 | complement |  | (-2147483647,2147483647) | / |  |
| 11 | P24W33B7 | TMK311_3 | RSV | %s | Func0001 | / | unsigned int |  |  | / |  |
| 12 | P24W33B6~B4 | TMK311_1 | GPS data status | %s | Func0001 | / | unsigned int |  |  | 0x06: Orbit extrapolation data; 0x05: Absolute positioning data; 0x03: Orbit determination filtering data |  |
| 13 | P24W33B3~B0 | TMK311_2 | Available stars | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 14 | P24W34B7~B6 | TMK311_3 | Working mode | %s | Func0001 | / | unsigned int |  |  | 0x1: Single GPS |  |
| 15 | P24W34B5~B4 | TMK311_4 | Orbit determination output enable flag | %s | Func0001 | / | unsigned int |  |  | 0x3:Orbit data output allowed; 0x2:Orbit data output forbidden |  |
| 16 | P24W34B3~B0 | TMK311_5 | Solution state | %s | Func0001 | / | unsigned int |  |  | 0x0: Unsolved; 0x5: Single GPS Solution |  |
| 17 | P24W35~W126 | RSV | RSV | %d | Func0001 | / | unsigned int | / | / | / |  |
| 18 | P24W127 | TMK376 | Request to return important data | %s | Func0001 | / | unsigned int |  |  | 0x55: Request the satellite subsystem to send back important data |  |

AOCC_Orbit_Parameter_Injection_ReturnTM(24H)

Pack ID=24H；Pack length=127（Bytes=128）。

Table 8-19AOCC_Orbit_Parameter_Injection_ReturnTMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P24W0B7~B4 | TMK401 | Telemetry frame feature code | %s | Func0001 | / | unsigned int |  |  | 0x0:Fast Frame 1 Telemetry; 0x1:Fast Frame 2 Telemetry; 0x3:GPS Orbit Telemetry; 0x4:Injected Orbit Parameter Telemetry; 0x5:Fast Frame Telemetry of Memory Downloaded; 0x6:Slow Frame 1 Telemetry; 0x7:Slow Frame 2 Telemetry |  |
|  | P24W0B3~W3B0 | TMK402 | Satellite time | %+8.3f | Func0001 | / | unsigned int |  | (0,268435455) | / |  |
|  | P24W4 | TMK403 | Work mode word | %s | Func0001 | / | unsigned int |  |  | 0x21: Global Attitude Acquisition Mode(AAM) - angular velocity damping; 0x22: Global Attitude Acquisition Mode(AAM) - sun search; 0x23: Global Attitude Acquisition Mode(AAM) - sun orientation; 0x24: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Earth Pointing Mode(EPM); 0x25: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Sun Pointing Mode(SPM); 0x30: Sun Pointing Mode(SPM); 0x43: Maneuver Mode - towards to Sun Pointing Mode(SPM); 0x45: Maneuver Mode - towards to Earth Pointing Mode(EPM); 0x46: Maneuver Mode - towards to Attitude Maintaining Mode(AMM); 0x50: Earth Pointing Mode(EPM); 0x60: Attitude Maintaining Mode(AMM); 0xC0: Stop Control Mode(SCM); |  |
|  | P24W31~W38 | TMK430 | Injected orbital parameter reference time | %+8.5f | Func0004 | / | double |  |  | / |  |
|  | P24W39~W42 | TMK431 | t0 orbital quasi-flat half long axis minus earth radius | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W43~W46 | TMK432 | First-order long-term coefficient of the semi-major axis | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W47~W50 | TMK433 | t0 Pseudo-orbital inclination | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W51~W54 | TMK434 | t0 Quasi-level ascending point | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W55~W58 | TMK435 | First-order long-term term coefficient of the right ascension | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W59~W62 | TMK436 | ksi First-order long-term term coefficient | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W63~W66 | TMK437 | eta First-order long-term term coefficient | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W67~W70 | TMK438 | First-order long-term term coefficient | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W71~W74 | TMK439 | eta_m | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W75~W78 | TMK440 | ksi_eta Long period variable amplitude a | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W79~W82 | TMK441 | ksi_eta Long period variable amplitude b | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W83~W86 | TMK442 | t0 lambda Initial value | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W87~W90 | TMK443 | lambda Long-term coefficient | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W91~W94 | TMK444 | lambda Second-order long-term term coefficient | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W95~W98 | TMK445 | Greenwich stellar time | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W99~W102 | TMK446 | t0 Sun near angle | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W103~W106 | TMK447 | t0 Peripheral angle of the sun | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W107~W110 | TMK448 | t0 ascending node of the sun | %+8.5f | Func0003 | / | float |  |  | / |  |
|  | P24W111~W118 | TMK449 | UTC from 090101 to the S-R separation | %+8.5f | Func0004 | / | double |  |  | / |  |
|  | P24W119~W126 | RSV | RSV | %d | Func0001 | / | unsigned int | / | / |  |  |
|  | P24W127 | TMK476 | Request to return important data | %s | Func0001 | / | unsigned int |  |  | 0x55: Request the satellite subsystem to send back important data |  |

AOCC_Memory_Dump_TM(24H)

Pack ID=24H；Pack length=127（Bytes=128）。

Table 8-20AOCC_Memory_Dump_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P24W0B7~B4 | TMK501 | Telemetry frame feature code | %s | Func0001 | / | unsigned int |  |  | 0x0:Fast Frame 1 Telemetry; 0x1:Fast Frame 2 Telemetry; 0x3:GPS Orbit Telemetry; 0x4:Injected Orbit Parameter Telemetry; 0x5:Fast Frame Telemetry of Memory Downloaded; 0x6:Slow Frame 1 Telemetry; 0x7:Slow Frame 2 Telemetry |  |
| 2 | P24W0B3~W3B0 | TMK502 | Satellite time | %+8.3f | Func0001 | / | unsigned int |  | (0,268435455) | / |  |
| 3 | P24W4 | TMK503 | Work mode word | %s | Func0001 | / | unsigned int |  |  | 0x21: Global Attitude Acquisition Mode(AAM) - angular velocity damping; 0x22: Global Attitude Acquisition Mode(AAM) - sun search; 0x23: Global Attitude Acquisition Mode(AAM) - sun orientation; 0x24: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Earth Pointing Mode(EPM); 0x25: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Sun Pointing Mode(SPM); 0x30: Sun Pointing Mode(SPM); 0x43: Maneuver Mode - towards to Sun Pointing Mode(SPM); 0x45: Maneuver Mode - towards to Earth Pointing Mode(EPM); 0x46: Maneuver Mode - towards to Attitude Maintaining Mode(AMM); 0x50: Earth Pointing Mode(EPM); 0x60: Attitude Maintaining Mode(AMM); 0xC0: Stop Control Mode(SCM); |  |
| 4 | P24W5 | TMK504 | Total number of telemetry frames under the memory | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 5 | P24W6 | TMK505 | Frame count | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 6 | P24W7~W14 | TMK506_1 | Data content 1 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 7 | P24W15~W22 | TMK506_2 | Data content 2 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 8 | P24W23~W30 | TMK506_3 | Data content 3 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 9 | P24W31~W38 | TMK506_4 | Data content 4 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 10 | P24W39~W46 | TMK506_5 | Data content 5 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 11 | P24W47~W54 | TMK506_6 | Data content 6 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 12 | P24W55~W62 | TMK506_7 | Data content 7 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 13 | P24W63~W70 | TMK506_8 | Data content 8 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 14 | P24W71~W78 | TMK506_9 | Data content 9 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 15 | P24W79~W86 | TMK506_10 | Data content 10 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 16 | P24W87~W94 | TMK506_11 | Data content 11 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 17 | P24W95~W102 | TMK506_12 | Data content 12 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 18 | P24W103~W110 | TMK506_13 | Data content 13 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 19 | P24W111~W118 | TMK506_14 | Data content 14 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 20 | P24W119~W126 | TMK506_15 | Data content 15 | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 21 | P24W127 | TMK576 | Request to return important data | %s | Func0001 | / | unsigned int |  |  | 0x55: Request the satellite subsystem to send back important data |  |

AOCC_slow_TM1(44H)

Pack ID=44H；Pack length=127（Bytes=128）。

Table 8-21AOCs_slow_TM1Table

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P44W0B7~B4 | TMK601 | Telemetry frame feature code | %s | Func0001 | / | unsigned int |  |  | 0x0:Fast Frame 1 Telemetry; 0x1:Fast Frame 2 Telemetry; 0x3:GPS Orbit Telemetry; 0x4:Injected Orbit Parameter Telemetry; 0x5:Fast Frame Telemetry of Memory Downloaded; 0x6:Slow Frame 1 Telemetry; 0x7:Slow Frame 2 Telemetry |  |
| 2 | P44W0B3~W3B0 | TMK602 | Satellite time | %+8.3f | Func0001 | / | unsigned int |  | (0,268435455) | / |  |
| 3 | P44W4 | TMK603 | Work mode word | %s | Func0001 | / | unsigned int |  |  | 0x21: Global Attitude Acquisition Mode(AAM) - angular velocity damping; 0x22: Global Attitude Acquisition Mode(AAM) - sun search; 0x23: Global Attitude Acquisition Mode(AAM) - sun orientation; 0x24: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Earth Pointing Mode(EPM); 0x25: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Sun Pointing Mode(SPM); 0x30: Sun Pointing Mode(SPM); 0x43: Maneuver Mode - towards to Sun Pointing Mode(SPM); 0x45: Maneuver Mode - towards to Earth Pointing Mode(EPM); 0x46: Maneuver Mode - towards to Attitude Maintaining Mode(AMM); 0x50: Earth Pointing Mode(EPM); 0x60: Attitude Maintaining Mode(AMM); 0xC0: Stop Control Mode(SCM); |  |
| 4 | P44W5~W6 | TMK604 | Orbital inclination i0 | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 5 | P44W7~W10 | TMK605 | Semi-long axis a | %+8.5f | Func0007 | 0.1 | unsigned int |  | (0,8000000) | / |  |
| 6 | P44W11~W12 | TMK606 | Ascending node omega | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 7 | P44W13 | TMK607 | Eccentricity e | %+8.5f | Func0007 | 0.00005 | unsigned int |  | (0,0.01) | / |  |
| 8 | P44W14~W15 | TMK608 | Perigee | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 9 | P44W16~W17 | TMK609 | Latitude angle u | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 10 | P44W18~W19 | TMK610 | Sub-satellite geography longitude | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 11 | P44W20~W21 | TMK611 | Sub-satellite geographic latitude | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 12 | P44W22 | TMK612 | Orbital angular velocity | %+8.5f | Func0007 | 0.0001 | complement |  | (-0.0128,0.0127) | / |  |
| 13 | P44W23 | TMK613 | Momentum wheel X integral diagnosis value | %+8.5f | Func0007 | 0.003 | unsigned int |  | (0,0.765) | / |  |
| 14 | P44W24 | TMK614 | Momentum wheel Y integral diagnosis value | %+8.5f | Func0007 | 0.003 | unsigned int |  | (0,0.765) | / |  |
| 15 | P44W25 | TMK615 | Momentum wheel Z integral diagnosis value | %+8.5f | Func0007 | 0.003 | unsigned int |  | (0,0.765) | / |  |
| 16 | P44W26 | TMK720 | Momentum wheel S integral diagnosis value | %+8.5f | Func0007 | 0.003 | unsigned int |  | (0,0.765) | / |  |
| 17 | P44W27~W28 | TMK616 | Sun is oriented continuously without sun time | %+8.5f | Func0001 | / | unsigned int |  | (0,65535) | / |  |
| 18 | P44W29~W30 | TMK617 | Rate damping time | %+8.5f | Func0001 | / | unsigned int |  | (0,65535) | / |  |
| 19 | P44W31 | TMK618 | Analog solar current 1 voltage acquisition | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 20 | P44W32 | TMK619 | Analog solar current 2 voltage acquisition | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 21 | P44W33 | TMK620 | Analog solar current 3 voltage acquisition | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 22 | P44W34 | TMK621 | Analog solar current 4 voltage acquisition | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 23 | P44W35 | TMK622 | Control computer 1 2V power supply telemetry | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 24 | P44W36 | TMK623 | Control computer 1 5V power supply telemetry | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 25 | P44W37 | TMK624 | Control computer 12V power supply telemetry | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 26 | P44W38 | TMK625 | Control computer negative 12V power supply telemetry | %+8.5f | Func0007 | -0.0196078431372549 | unsigned int |  | (-5,0) | / |  |
| 27 | P44W39 | TMK626 | Control computer 3 3V power supply telemetry | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 28 | P44W40~W41 | TMK627 | Momentum wheel X motor current | %+8.5f | Func0007 | 62.5 | complement |  |  | / |  |
| 29 | P44W42 | TMK628 | Momentum wheel X bearing temperature | %+8.5f | Func0007 | 100 | unsigned int |  |  | / |  |
| 30 | P44W43 | TMK629 | Momentum wheel X signal power supply 12V voltage | %+8.5f | Func0007 | 0.1 | unsigned int |  |  | / |  |
| 31 | P44W44 | TMK630 | Momentum wheel X signal power supply 5V voltage | %+8.5f | Func0007 | 25 | unsigned int |  |  | / |  |
| 32 | P44W45 | TMK631 | Momentum wheel X signal power supply 3 point 3V voltage | %+8.5f | Func0007 | 15 | unsigned int |  |  | / |  |
| 33 | P44W46 | TMK632 | Momentum wheel X signal power supply 3 point 3V current | %+8.5f | Func0007 | 2 | unsigned int |  |  | / |  |
| 34 | P44W47 | TMK633 | Momentum wheel X signal power supply 1 point 2V voltage | %+8.5f | Func0007 | 10 | unsigned int |  |  | / |  |
| 35 | P44W48 | TMK634 | Momentum wheel X power voltage telemetry | %+8.5f | Func0007 | 0.15 | unsigned int |  |  | / |  |
| 36 | P44W49~W50 | TMK635 | Momentum wheel Y motor current | %+8.5f | Func0007 | 62.5 | complement |  |  | / |  |
| 37 | P44W51 | TMK636 | Momentum wheel Y bearing temperature | %+8.5f | Func0007 | 100 | unsigned int |  |  | / |  |
| 38 | P44W52 | TMK637 | Momentum wheel Y signal power supply 12V voltage | %+8.5f | Func0007 | 0.1 | unsigned int |  |  | / |  |
| 39 | P44W53 | TMK638 | Momentum wheel Y signal power supply 5V voltage | %+8.5f | Func0007 | 25 | unsigned int |  |  | / |  |
| 40 | P44W54 | TMK639 | Momentum wheel Y signal power supply 3V voltage at 3:00 | %+8.5f | Func0007 | 15 | unsigned int |  |  | / |  |
| 41 | P44W55 | TMK640 | Momentum wheel Y signal power supply 3 point 3V current | %+8.5f | Func0007 | 2 | unsigned int |  |  | / |  |
| 42 | P44W56 | TMK641 | Momentum wheel Y signal power supply 1 point 2V voltage | %+8.5f | Func0007 | 10 | unsigned int |  |  | / |  |
| 43 | P44W57 | TMK642 | Momentum wheel Y power and voltage telemetry | %+8.5f | Func0007 | 0.15 | unsigned int |  |  | / |  |
| 44 | P44W58~W59 | TMK643 | Momentum wheel Z motor current | %+8.5f | Func0007 | 62.5 | complement |  |  | / |  |
| 45 | P44W60 | TMK644 | Momentum wheel Z bearing temperature | %+8.5f | Func0007 | 100 | unsigned int |  |  | / |  |
| 46 | P44W61 | TMK645 | Momentum wheel Z signal power supply 12V voltage | %+8.5f | Func0007 | 0.1 | unsigned int |  |  | / |  |
| 47 | P44W62 | TMK646 | Momentum wheel Z signal power supply 5V voltage | %+8.5f | Func0007 | 25 | unsigned int |  |  | / |  |
| 48 | P44W63 | TMK647 | Momentum wheel Z signal power supply 3 point 3V voltage | %+8.5f | Func0007 | 15 | unsigned int |  |  | / |  |
| 49 | P44W64 | TMK648 | Momentum wheel Z signal power supply 3 point 3V current | %+8.5f | Func0007 | 2 | unsigned int |  |  | / |  |
| 50 | P44W65 | TMK649 | Momentum wheel Z signal power supply 1 point 2V voltage | %+8.5f | Func0007 | 10 | unsigned int |  |  | / |  |
| 51 | P44W66 | TMK650 | Momentum wheel Z power voltage telemetry | %+8.5f | Func0007 | 0.15 | unsigned int |  |  | / |  |
| 52 | P44W67~W68 | TMK651 | Momentum wheel S motor current | %+8.5f | Func0007 | 62.5 | complement |  |  | / |  |
| 53 | P44W69 | TMK652 | Momentum wheel S bearing temperature | %+8.5f | Func0007 | 100 | unsigned int |  |  | / |  |
| 54 | P44W70 | TMK653 | Momentum wheel S signal power supply 12V voltage | %+8.5f | Func0007 | 0.1 | unsigned int |  |  | / |  |
| 55 | P44W71 | TMK654 | Momentum wheel S signal power supply 5V voltage | %+8.5f | Func0007 | 25 | unsigned int |  |  | / |  |
| 56 | P44W72 | TMK655 | Momentum wheel S signal power supply 3 point 3V voltage | %+8.5f | Func0007 | 15 | unsigned int |  |  | / |  |
| 57 | P44W73 | TMK656 | Momentum wheel S signal power supply 3 point 3V current | %+8.5f | Func0007 | 2 | unsigned int |  |  | / |  |
| 58 | P44W74 | TMK657 | Momentum wheel S signal power supply 1 point 2V voltage | %+8.5f | Func0007 | 10 | unsigned int |  |  | / |  |
| 59 | P44W75 | TMK658 | Momentum wheel S power and voltage telemetry | %+8.5f | Func0007 | 0.15 | unsigned int |  |  | / |  |
| 60 | P44W76 | TMK659 | Cumulative unsuccessful communication with gyro X | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 61 | P44W77 | TMK660 | Accumulated unsuccessful communication with gyro Y | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 62 | P44W78 | TMK661 | Accumulated unsuccessful communication with gyro Z | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 63 | P44W79 | TMK662 | Accumulated unsuccessful communication with gyro S | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 64 | P44W80 | TMK663 | Cumulative unsuccessful communication with magnetometer A | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 65 | P44W81 | TMK664 | Cumulative unsuccessful communication with magnetometer B | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 66 | P44W82 | TMK665 | Accumulated unsuccessful communications with the on-chip sun | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 67 | P44W83 | TMK666 | Cumulative unsuccessful communication with starmin A | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 68 | P44W84 | TMK667 | Cumulative unsuccessful communication with starmin B | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 69 | P44W85 | TMK668 | Cumulative unsuccessful communication with momentum wheel X | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 70 | P44W86 | TMK669 | Cumulative unsuccessful communication with momentum wheel Y | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 71 | P44W87 | TMK670 | Cumulative unsuccessful communication with momentum wheel Z | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 72 | P44W88 | TMK671 | Cumulative unsuccessful communication with momentum wheel S | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 73 | P44W89 | TMK672 | CAN bus initialization times | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 74 | P44W89B7~B4 | TMK672D7D4 | CAN bus A initialization times | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 75 | P44W89B3~B0 | TMK673D3D0 | CAN bus B initialization times | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 76 | P44W90~W91 | TMK673 | Star sensor calibration of target Angle phi_AR1 | %+8.5f | Func0007 | 0.0001 | complement |  | (-25.4,25.4) | / |  |
| 77 | P44W92~W93 | TMK674 | Star sensor calibration of target Angle phi_AR2 | %+8.5f | Func0007 | 0.0001 | complement |  | (-25.4,25.4) | / |  |
| 78 | P44W94~W95 | TMK675 | Star sensor calibration of target Angle psi_AR | %+8.5f | Func0007 | 0.0001 | complement |  | (-25.4,25.4) | / |  |
| 79 | P44W96 | TMK676 | Gyro X-axis constant drift estimation | %+8.5f | Func0007 | 0.2 | complement |  | (-25.4,25.4) | / |  |
| 80 | P44W97 | TMK677 | Gyro Y-axis constant drift estimation | %+8.5f | Func0007 | 0.2 | complement |  | (-25.4,25.4) | / |  |
| 81 | P44W98 | TMK678 | Gyro Z-axis constant drift estimation | %+8.5f | Func0007 | 0.2 | complement |  | (-25.4,25.4) | / |  |
| 82 | P44W99 | TMK679 | Gyro X-axis constant drift valuation_magnetic positioning | %+8.5f | Func0007 | 0.2 | complement |  | (-25.4,25.4) | / |  |
| 83 | P44W100 | TMK680 | Gyro Y-axis constant drift valuation_magnetic positioning | %+8.5f | Func0007 | 0.2 | complement |  | (-25.4,25.4) | / |  |
| 84 | P44W101 | TMK681 | Gyro Z-axis constant drift valuation_magnetic positioning | %+8.5f | Func0007 | 0.2 | complement |  | (-25.4,25.4) | / |  |
| 85 | P44W102 | TMK682 | Momentum wheel X nominal angular momentum | %+8.5f | Func0007 | 0.0028 | complement |  | (-0.36,0.36) | / |  |
| 86 | P44W103 | TMK683 | Momentum wheel Y nominal angular momentum | %+8.5f | Func0007 | 0.0028 | complement |  | (-0.36,0.36) | / |  |
| 87 | P44W104 | TMK684 | Momentum wheel Z nominal angular momentum | %+8.5f | Func0007 | 0.0028 | complement |  | (-0.36,0.36) | / |  |
| 88 | P44W105 | TMK685 | Momentum wheel S nominal angular momentum | %+8.5f | Func0007 | 0.0028 | complement |  | (-0.36,0.36) | / |  |
| 89 | P44W106B7~B4 | TMK686 | Momentum wheel X cumulative reset times | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 90 | P44W106B3~B0 | TMK687 | Momentum wheel X_EDAC single error count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 91 | P44W107B7~B4 | TMK688 | Momentum wheel X_EDAC double error times | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 92 | P44W107B3~B0 | TMK689 | Momentum wheel X_RAM area self-test failure count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 93 | P44W108B7~B4 | TMK690 | Momentum wheel Y cumulative reset times | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 94 | P44W108B3~B0 | TMK691 | Momentum wheel Y_EDAC single error count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 95 | P44W109B7~B4 | TMK692 | Momentum wheel Y_EDAC double error times | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 96 | P44W109B3~B0 | TMK693 | Momentum wheel Y_RAM area self-test failure count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 97 | P44W110B7~B4 | TMK694 | Momentum wheel Z cumulative reset times | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 98 | P44W110B3~B0 | TMK695 | Momentum wheel Z_EDAC single error count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 99 | P44W111B7~B4 | TMK696 | Momentum wheel Z_EDAC double error times | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 100 | P44W111B3~B0 | TMK697 | Momentum wheel Z_RAM area self-test failure count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 101 | P44W112B7~B4 | TMK698 | Momentum wheel S cumulative reset times | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 102 | P44W112B3~B0 | TMK699 | Momentum wheel S_EDAC single error count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 103 | P44W113B7~B4 | TMK700 | Momentum wheel S_EDAC double error times | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 104 | P44W113B3~B0 | TMK701 | Momentum wheel S_RAM area self-test fault count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 105 | P44W114B7~B4 | TMK702 | Fiber Optic Gyro X cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 106 | P44W114B3~B0 | TMK703 | Fiber optic gyro Y cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 107 | P44W115B7~B4 | TMK704 | Fiber optic gyro Z cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 108 | P44W115B3~B0 | TMK705 | Fiber Optic Gyro S cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 109 | P44W116B7~B4 | TMK706 | Magnetometer A cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 110 | P44W116B3~B0 | TMK707 | Magnetometer B cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 111 | P44W117B7~B4 | TMK708 | Momentum wheel X cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 112 | P44W117B3~B0 | TMK709 | Momentum wheel Y cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 113 | P44W118B7~B4 | TMK710 | Momentum wheel Z cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 114 | P44W118B3~B0 | TMK711 | Momentum wheel S cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 115 | P44W119B7~B4 | TMK712 | Star sensor A cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 116 | P44W119B3~B0 | TMK713 | Star sensor B cumulative restart count | %+8.5f | Func0001 | / | unsigned int |  | (0,15) | / |  |
| 117 | P44W120B7 | RSV | RSV | %d | Func0001 | / | unsigned int | / | / | / |  |
| 118 | P44W120B6~B5 | TMK714_1 | System software telemetry word 3_ location of the error address | %s | Func0001 | / | unsigned int |  |  | 0x00: No error; 0x01: ROM boot system software area; 0x02: RAM; 0x03: Other areas |  |
| 119 | P44W120B4~W122B0 | TMK714_2 | System software telemetry word 3_ error address | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |
| 120 | P44W123 | TMK714_3 | System software telemetry word 3_trap | %s | Func0001 | / | unsigned int |  |  | 0x01: Instruction access trap; 0x02: Illegal instruction trap; 0x03: Privilege instruction trap; 0x04: FPU invalid trap; 0x07: Memory address misaligned trap; 0x08: FPU exception trap; 0x09: Data access exception; 0x0A: Label overflow trap; 0x24: Forbidden co-processing trap; 0x2B: Buffer error; 0x0B: Hardware break-point trap; 0x20: Register stack EDAC fault; 0x28: co-processor exception; 0x2A: divided by 0; 0x11: AHB bus error interrupt |  |
| 121 | P44W124~W127 | TMK715 | Traps error memory address | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |

AOCC_slow_TM2(44H)

Pack ID=44H；Pack length=127（Bytes=128）。

Table 8-22AOCs_slow_TM2Table

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P44W0B7~B4 | TMK801 | Telemetry frame feature code | %s | Func0001 | / | unsigned int |  |  | 0x0:Fast Frame 1 Telemetry; 0x1:Fast Frame 2 Telemetry; 0x3:GPS Orbit Telemetry; 0x4:Injected Orbit Parameter Telemetry; 0x5:Fast Frame Telemetry of Memory Downloaded; 0x6:Slow Frame 1 Telemetry; 0x7:Slow Frame 2 Telemetry |  |
| 2 | P44W0B3~W3B0 | TMK802 | Satellite time | %+8.3f | Func0001 | / | unsigned int |  | (0,268435455) | / |  |
| 3 | P44W4 | TMK803 | Segment mark | %s | Func0001 | / | unsigned int |  |  | 0x21: Global Attitude Acquisition Mode(AAM) - angular velocity damping; 0x22: Global Attitude Acquisition Mode(AAM) - sun search; 0x23: Global Attitude Acquisition Mode(AAM) - sun orientation; 0x24: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Earth Pointing Mode(EPM); 0x25: Global Attitude Acquisition Mode(AAM) - attitude adjustment to Sun Pointing Mode(SPM); 0x30: Sun Pointing Mode(SPM); 0x43: Maneuver Mode - towards to Sun Pointing Mode(SPM); 0x45: Maneuver Mode - towards to Earth Pointing Mode(EPM); 0x46: Maneuver Mode - towards to Attitude Maintaining Mode(AMM); 0x50: Earth Pointing Mode(EPM); 0x60: Attitude Maintaining Mode(AMM); 0xC0: Stop Control Mode(SCM); |  |
| 4 | P44W5~W6 | TMK804 | Orbital inclination i0 | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 5 | P44W7~W10 | TMK805 | Semi-major axis a | %+8.5f | Func0007 | 0.1 | unsigned int |  | (0,8000000) | / |  |
| 6 | P44W11~W12 | TMK806 | Ascending node | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 7 | P44W13 | TMK807 | Eccentricity e | %+8.5f | Func0007 | 0.00005 | unsigned int |  | (0,0.01) | / |  |
| 8 | P44W14~W15 | TMK808 | Close angle | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 9 | P44W16~W17 | TMK809 | Latitude angle u | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 10 | P44W18~W19 | TMK810 | Sub-satellite geography longitude | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 11 | P44W20~W21 | TMK811 | Sub-satellite geographic latitude | %+8.5f | Func0007 | 0.006 | complement |  | (-180,180) | / |  |
| 12 | P44W22 | TMK812 | Orbital angular velocity | %+8.5f | Func0007 | 0.0001 | complement |  | (-0.0128,0.0127) | / |  |
| 13 | P44W23 | TMK813 | Momentum wheel X integral diagnosis value | %+8.5f | Func0007 | 0.003 | unsigned int |  | (0,0.765) | / |  |
| 14 | P44W24 | TMK814 | Momentum wheel Y integral diagnosis value | %+8.5f | Func0007 | 0.003 | unsigned int |  | (0,0.765) | / |  |
| 15 | P44W25 | TMK815 | Momentum wheel Z integral diagnosis value | %+8.5f | Func0007 | 0.003 | unsigned int |  | (0,0.765) | / |  |
| 16 | P44W26 | TMK928 | Momentum wheel S integral diagnosis value | %+8.5f | Func0007 | 0.003 | unsigned int |  | (0,0.765) | / |  |
| 17 | P44W27~W28 | TMK816 | The sun is oriented continuously without sun time | %+8.5f | Func0001 | / | unsigned int |  | (0,65535) | / |  |
| 18 | P44W29~W30 | TMK817 | Rate damping time | %+8.5f | Func0001 | / | unsigned int |  | (0,65535) | / |  |
| 19 | P44W31 | TMK818 | Analog solar current 1 voltage acquisition | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 20 | P44W32 | TMK819 | Analog solar current 2 voltage acquisition | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 21 | P44W33 | TMK820 | Analog solar current 3 voltage acquisition | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 22 | P44W34 | TMK821 | Analog solar current 4 voltage acquisition | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 23 | P44W35 | TMK822 | Control computer 1 point 2V power supply telemetry | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 24 | P44W36 | TMK823 | Control computer 1 point 5V power supply telemetry | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 25 | P44W37 | TMK824 | Control computer 12V power supply telemetry | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 26 | P44W38 | TMK825 | Control computer negative 12V power supply telemetry | %+8.5f | Func0007 | -0.0196078431372549 | unsigned int |  | (-5,0) | / |  |
| 27 | P44W39 | TMK826 | Control computer 3 point 3V power supply telemetry | %+8.5f | Func0007 | 0.0196078431372549 | unsigned int |  | (0,5) | / |  |
| 28 | P44W40~W41 | TMK827 | Momentum wheel X motor current | %+8.5f | Func0007 | 62.5 | complement |  |  | / |  |
| 29 | P44W42 | TMK828 | Momentum wheel X bearing temperature | %+8.5f | Func0007 | 100 | unsigned int |  |  | / |  |
| 30 | P44W43 | TMK829 | Momentum wheel X signal power supply 12V voltage | %+8.5f | Func0007 | 0.1 | unsigned int |  |  | / |  |
| 31 | P44W44 | TMK830 | Momentum wheel X signal power supply 5V voltage | %+8.5f | Func0007 | 25 | unsigned int |  |  | / |  |
| 32 | P44W45 | TMK831 | Momentum wheel X signal power supply 3 points 3V voltage | %+8.5f | Func0007 | 15 | unsigned int |  |  | / |  |
| 33 | P44W46 | TMK832 | Momentum wheel X signal power supply 3 points 3V current | %+8.5f | Func0007 | 2 | unsigned int |  |  | / |  |
| 34 | P44W47 | TMK833 | Momentum wheel X signal power supply 1 point 2V voltage | %+8.5f | Func0007 | 10 | unsigned int |  |  | / |  |
| 35 | P44W48 | TMK834 | Momentum wheel X power voltage telemetry | %+8.5f | Func0007 | 0.15 | unsigned int |  |  | / |  |
| 36 | P44W49~W50 | TMK835 | Momentum wheel Y motor current | %+8.5f | Func0007 | 62.5 | complement |  |  | / |  |
| 37 | P44W51 | TMK836 | Momentum wheel Y bearing temperature | %+8.5f | Func0007 | 100 | unsigned int |  |  | / |  |
| 38 | P44W52 | TMK837 | Momentum wheel Y signal power supply 12V voltage | %+8.5f | Func0007 | 0.1 | unsigned int |  |  | / |  |
| 39 | P44W53 | TMK838 | Momentum wheel Y signal power supply 5V voltage | %+8.5f | Func0007 | 25 | unsigned int |  |  | / |  |
| 40 | P44W54 | TMK839 | Momentum wheel Y signal power supply 3 points 3V voltage | %+8.5f | Func0007 | 15 | unsigned int |  |  | / |  |
| 41 | P44W55 | TMK840 | Momentum wheel Y signal power supply 3 points 3V current | %+8.5f | Func0007 | 2 | unsigned int |  |  | / |  |
| 42 | P44W56 | TMK841 | Momentum wheel Y signal power supply 1 point 2V voltage | %+8.5f | Func0007 | 10 | unsigned int |  |  | / |  |
| 43 | P44W57 | TMK842 | Momentum wheel Y power voltage telemetry | %+8.5f | Func0007 | 0.15 | unsigned int |  |  | / |  |
| 44 | P44W58~W59 | TMK843 | Momentum wheel Z motor current | %+8.5f | Func0007 | 62.5 | complement |  |  | / |  |
| 45 | P44W60 | TMK844 | Momentum wheel Z bearing temperature | %+8.5f | Func0007 | 100 | unsigned int |  |  | / |  |
| 46 | P44W61 | TMK845 | Momentum wheel Z signal power supply 12V voltage | %+8.5f | Func0007 | 0.1 | unsigned int |  |  | / |  |
| 47 | P44W62 | TMK846 | Momentum wheel Z signal power supply 5V voltage | %+8.5f | Func0007 | 25 | unsigned int |  |  | / |  |
| 48 | P44W63 | TMK847 | Momentum wheel Z signal power supply 3 points 3V voltage | %+8.5f | Func0007 | 15 | unsigned int |  |  | / |  |
| 49 | P44W64 | TMK848 | Momentum wheel Z signal power supply 3 points 3V current | %+8.5f | Func0007 | 2 | unsigned int |  |  | / |  |
| 50 | P44W65 | TMK849 | Momentum wheel Z signal power supply 1 point 2V voltage | %+8.5f | Func0007 | 10 | unsigned int |  |  | / |  |
| 51 | P44W66 | TMK850 | Momentum wheel Z power voltage telemetry | %+8.5f | Func0007 | 0.15 | unsigned int |  |  | / |  |
| 52 | P44W67~W68 | TMK851 | Momentum wheel S motor current | %+8.5f | Func0007 | 62.5 | complement |  |  | / |  |
| 53 | P44W69 | TMK852 | Momentum wheel S bearing temperature | %+8.5f | Func0007 | 100 | unsigned int |  |  | / |  |
| 54 | P44W70 | TMK853 | Momentum wheel S signal power supply 12V voltage | %+8.5f | Func0007 | 0.1 | unsigned int |  |  | / |  |
| 55 | P44W71 | TMK854 | Momentum wheel S signal power supply 5V voltage | %+8.5f | Func0007 | 25 | unsigned int |  |  | / |  |
| 56 | P44W72 | TMK855 | Momentum wheel S signal power supply 3 points 3V voltage | %+8.5f | Func0007 | 15 | unsigned int |  |  | / |  |
| 57 | P44W73 | TMK856 | Momentum wheel S signal power supply 3 points 3V current | %+8.5f | Func0007 | 2 | unsigned int |  |  | / |  |
| 58 | P44W74 | TMK857 | Momentum wheel S signal power supply 1 point 2V voltage | %+8.5f | Func0007 | 10 | unsigned int |  |  | / |  |
| 59 | P44W75 | TMK858 | Momentum wheel S power voltage telemetry | %+8.5f | Func0007 | 0.15 | unsigned int |  |  | / |  |
| 60 | P44W76 | TMK859 | Accumulated unsuccessful times with Gyro X communication | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 61 | P44W77 | TMK860 | The number of unsuccessful attempts to communicate with the gyro Y | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 62 | P44W78 | TMK861 | The number of unsuccessful attempts to communicate with the gyro Z | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 63 | P44W79 | TMK862 | The number of unsuccessful attempts to communicate with the gyro S | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 64 | P44W80 | TMK863 | The number of unsuccessful attempts to communicate with the magnetometer A | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 65 | P44W81 | TMK864 | The number of unsuccessful attempts to communicate with the magnetometer B | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 66 | P44W82 | TMK865 | The number of unsuccessful attempts to communicate with the on-chip sun | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 67 | P44W83 | TMK866 | The number of unsuccessful attempts to communicate with star sensor A | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 68 | P44W84 | TMK867 | The number of unsuccessful attempts to communicate with star sensor B | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 69 | P44W85 | TMK868 | The number of unsuccessful attempts to communicate with the momentum wheel X | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 70 | P44W86 | TMK869 | The number of unsuccessful attempts to communicate with the momentum wheel Y | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 71 | P44W87 | TMK870 | The number of unsuccessful attempts to communicate with the momentum wheel Z | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 72 | P44W88 | TMK871 | The number of unsuccessful attempts to communicate with the momentum wheel S | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 73 | P44W89 | TMK872 | CAN bus initialization times | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 74 | P44W90B7 | TMK901_8 | RSV | %d | Func0001 | / | unsigned int | / | / | / |  |
| 75 | P44W90B6 | TMK901_7 | RSV | %d | Func0001 | / | unsigned int | / | / | / |  |
| 76 | P44W90B5 | TMK901_6 | Star sensor A_DDR storage area data health status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 77 | P44W90B4 | TMK901_5 | Star sensor A_Star Table Storage Area Data Health Status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 78 | P44W90B3 | TMK901_4 | Star sensor A_ program storage area data health status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 79 | P44W90B2 | TMK901_3 | Star sensor A_ boot storage area data health status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 80 | P44W90B1 | TMK901_2 | Star sensor A_APS device working status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 81 | P44W90B0 | TMK901_1 | Star sensor A_ gyro device working status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 82 | P44W91 | TMK902 | Star sensor A_RTS instruction count | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 83 | P44W92 | TMK903 | Star sensor A all instruction count | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 84 | P44W93 | TMK904 | Star sensor A product temperature | %+8.5f | Func0002 | / | complement |  | (-127,127) | / |  |
| 85 | P44W94B7 | TMK905_8 | RSV | %d | Func0001 | / | unsigned int | / | / | / |  |
| 86 | P44W94B6 | TMK905_7 | RSV | %d | Func0001 | / | unsigned int | / | / | / |  |
| 87 | P44W94B5 | TMK905_6 | Star sensor B_DDR storage area data health status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 88 | P44W94B4 | TMK905_5 | Star sensor B_Star Table Storage Area Data Health Status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 89 | P44W94B3 | TMK905_4 | Star sensor B_ program storage area data health status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 90 | P44W94B2 | TMK905_3 | Star sensor B_ boot storage area data health status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 91 | P44W94B1 | TMK905_2 | Star sensor B_APS device working status | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 92 | P44W94B0 | TMK905_1 | Star sensor B_ gyro device working state | %s | Func0001 | / | unsigned int |  |  | 0x00: Normal; 0x01: Abnormal |  |
| 93 | P44W95 | TMK906 | Star sensor B_RTS instruction count | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 94 | P44W96 | TMK907 | Star Sensor B all instruction count | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 95 | P44W97 | TMK908 | Star sensor B product temperature | %+8.5f | Func0002 | / | complement |  | (-127,127) | / |  |
| 96 | P44W98 | TMK909 | Star sensor AX Axis Accuracy Estimator | %+8.5f | Func0007 | 0.5 | unsigned int |  | (0,51) | / |  |
| 97 | P44W99 | TMK910 | Star sensor AY Axis Accuracy Estimator | %+8.5f | Func0007 | 0.5 | unsigned int |  | (0,51) | / |  |
| 98 | P44W100 | TMK911 | Star sensor AZ Axis Accuracy Estimator | %+8.5f | Func0007 | 0.5 | unsigned int |  | (0,51) | / |  |
| 99 | P44W101 | TMK912 | Star sensor BX Axis Accuracy Estimator | %+8.5f | Func0007 | 0.5 | unsigned int |  | (0,51) | / |  |
| 100 | P44W102 | TMK913 | Star sensor BY axis accuracy estimate | %+8.5f | Func0007 | 0.5 | unsigned int |  | (0,51) | / |  |
| 101 | P44W103 | TMK914 | Star sensor BZ Axis Accuracy Estimator | %+8.5f | Func0007 | 0.5 | unsigned int |  | (0,51) | / |  |
| 102 | P44W104~W105 | TMK915 | Star sensor A star map threshold | %+8.5f | Func0001 | / | unsigned int |  | (0,65535) | / |  |
| 103 | P44W106~W107 | TMK916 | Star sensor B star map threshold | %+8.5f | Func0001 | / | unsigned int |  | (0,65535) | / |  |
| 104 | P44W108~W109 | TMK917 | Star sensor A gets angular velocity omega1 | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 105 | P44W110~W111 | TMK918 | Star sensor A gets angular velocity omega2 | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 106 | P44W112~W113 | TMK919 | Star sensor A gets angular velocity omega3 | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 107 | P44W114~W115 | TMK920 | Star sensor B gets angular velocity omega1 | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 108 | P44W116~W117 | TMK921 | Star sensor B gets angular velocity omega2 | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 109 | P44W118~W119 | TMK922 | Star sensor B gets angular velocity omega3 | %+8.5f | Func0007 | 0.0005 | complement |  | (-16.38,16.38) | / |  |
| 110 | P44W120B7 | TMK923_1 | Injection program module 1 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 111 | P44W120B6 | TMK923_2 | Injection program module 2 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 112 | P44W120B5 | TMK923_3 | Injection program module 3 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 113 | P44W120B4 | TMK923_4 | Injection program module 4 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 114 | P44W120B3 | TMK923_5 | Injection program module 5 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 115 | P44W120B2 | TMK923_6 | Injection program module 6 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 116 | P44W120B1 | TMK923_7 | Injection program module 7 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 117 | P44W120B0 | TMK923_8 | Injection program module 8 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 118 | P44W121B7 | TMK923_9 | Injection program module 9 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 119 | P44W121B6 | TMK923_10 | Injection program module 10 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 120 | P44W121B5 | TMK923_11 | Injection program module 11 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 121 | P44W121B4 | TMK923_12 | Injection program module 12 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 122 | P44W121B3 | TMK923_13 | Injection program module 13 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 123 | P44W121B2 | TMK923_14 | Injection program module 14 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 124 | P44W121B1 | TMK923_15 | Injection program module 15 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 125 | P44W121B0 | TMK923_16 | Injection program module 16 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 126 | P44W122B7 | TMK923_17 | Injection program module 17 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 127 | P44W122B6 | TMK923_18 | Injection program module 18 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 128 | P44W122B5 | TMK923_19 | Injection program module 19 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 129 | P44W122B4 | TMK923_20 | Injection program module 20 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 130 | P44W122B3 | TMK923_21 | Injection program module 21 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 131 | P44W122B2 | TMK923_22 | Injection program module 22 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 132 | P44W122B1 | TMK923_23 | Injection program module 23 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 133 | P44W122B0 | TMK923_24 | Injection program module 24 running state | %s | Func0001 | / | unsigned int |  |  | 0x0: Not running; 0x1: Running |  |
| 134 | P44W123 | TMK924 | On-chip sun cumulative restarts | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 135 | P44W124 | TMK925 | On-chip solar integration time | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 136 | P44W125 | TMK926 | On-chip sun illegal instruction count | %+8.5f | Func0001 | / | unsigned int |  | (0,255) | / |  |
| 137 | P44W126~W127 | TMK927 | Software version number | %+8.5f | Func0001 | / | unsigned int |  |  | / |  |

BMU

BMU_fast_TM(28H)

Pack ID=28H；Pack length=13（Bytes=14）。

Table 8-23BMU_fast_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P28W0 | ND001 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 2 | P28W1 | ND002 | Batt_Cell-1 voltage | %6.4f | Func0010 | 0.0188,0.089 | unsigned int | V |  | / |  |
| 3 | P28W2 | ND003 | Batt_Cell-2 voltage | %6.4f | Func0010 | 0.0188,0.089 | unsigned int | V |  | / |  |
| 4 | P28W3 | ND004 | Batt_Cell-3 voltage | %6.4f | Func0010 | 0.0188,0.089 | unsigned int | V |  | / |  |
| 5 | P28W4 | ND005 | Batt_Cell-4 voltage | %6.4f | Func0010 | 0.0188,0.089 | unsigned int | V |  | / |  |
| 6 | P28W5 | ND006 | Batt_Cell-5 voltage | %6.4f | Func0010 | 0.0188,0.089 | unsigned int | V |  | / |  |
| 7 | P28W6 | ND007 | Batt_Cell-6 voltage | %6.4f | Func0010 | 0.0188,0.089 | unsigned int | V |  | / |  |
| 8 | P28W7 | ND008 | Batt_Cell-7 voltage | %6.4f | Func0010 | 0.0188,0.089 | unsigned int | V |  | / |  |
| 9 | P28W8B7 | ND009 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 10 | P28W8B6 | ND010 | FLASH erase enable | %s | Func0001 | / | unsigned int | / |  | 1:enabled;0:disenabled |  |
| 11 | P28W8B5 | ND011 | CAN usage status | %s | Func0001 | / | unsigned int | / |  | 0:CANA bus;1:CANB bus |  |
| 12 | P28W8B4 | ND012 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 13 | P28W8B3 | ND013 | Upload OC task status | %s | Func0001 | / | unsigned int | / |  | 1:started up;0:not started up |  |
| 14 | P28W8B2 | ND014 | Upload CAN task status | %s | Func0001 | / | unsigned int | / |  | 1:started up;0:not started up |  |
| 15 | P28W8B1 | ND015 | Upload the power task status | %s | Func0001 | / | unsigned int | / |  | 1:started up;0:not started up |  |
| 16 | P28W8B0 | ND016 | Upload task status | %s | Func0001 | / | unsigned int | / |  | 1:started up;0:not started up |  |
| 17 | P28W9B7~B4 | ND017 | Battery subcomputer hot start count | %u | Func0001 | / | unsigned int | / |  | / |  |
| 18 | P28W9B3~B0 | ND018 | Battery subcomputer indirect instruction count | %u | Func0001 | / | unsigned int | / |  | / |  |
| 19 | P28W10 | ND019 | CANA bus reset count | %u | Func0001 | / | unsigned int | / |  | / |  |
| 20 | P28W11B7 | ND020 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 21 | P28W11B6 | ND021 | Batt_Cell-7_ Balance_STATUS | %s | Func0001 | / | unsigned int | / |  | 0:balance off;1:balance on |  |
| 22 | P28W11B5 | ND022 | Batt_Cell-6_ Balance_STATUS | %s | Func0001 | / | unsigned int | / |  | 0:balance off;1:balance on |  |
| 23 | P28W11B4 | ND023 | Batt_Cell-5_ Balance_STATUS | %s | Func0001 | / | unsigned int | / |  | 0:balance off;1:balance on |  |
| 24 | P28W11B3 | ND024 | Batt_Cell-4_ Balance_STATUS | %s | Func0001 | / | unsigned int | / |  | 0:balance off;1:balance on |  |
| 25 | P28W11B2 | ND025 | Batt_Cell-3_ Balance_STATUS | %s | Func0001 | / | unsigned int | / |  | 0:balance off;1:balance on |  |
| 26 | P28W11B1 | ND026 | Batt_Cell-2_ Balance_STATUS | %s | Func0001 | / | unsigned int | / |  | 0:balance off;1:balance on |  |
| 27 | P28W11B0 | ND027 | Batt_Cell-1_ Balance_STATUS | %s | Func0001 | / | unsigned int | / |  | 0:balance off;1:balance on |  |
| 28 | P28W12B7 | ND028 | Batt_Balance__ENABLE | %s | Func0001 | / | unsigned int | / |  | 1:enabled;0:disenabled |  |
| 29 | P28W12B6~B0 | ND029 | Reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 30 | P28W13 | ND030 | CANB bus reset count | %u | Func0001 | / | unsigned int | / |  | / |  |

TCU

TCU_Anolog_fast_TM(27H)

Pack ID=27H；Pack length=11（Bytes=12）。

Table 8-24TCU_Anolog_fast_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P27W0~W2B23~B12 | RM001 | MC-VOLTAGE1 | %f | Func0010 | 0.0012207,0 | unsigned int | V |  | / |  |
| 2 | P27W0~W2B11~B0 | RM002 | MC-VOLTAGE2 | %f | Func0010 | 0.0012207,0 | unsigned int | V |  | / |  |
| 3 | P27W3~W5B23~B12 | RM003 | MC-VOLTAGE3 | %f | Func0010 | 0.0012207,0 | unsigned int | V |  | / |  |
| 4 | P27W3~W5B11~B0 | RM004 | MC-VOLTAGE4 | %f | Func0010 | 0.0012207,0 | unsigned int | V |  | / |  |
| 5 | P27W6~W8B23~B12 | RM005 | MC-VOLTAGE5 | %f | Func0010 | 0.0012207,0 | unsigned int | V |  | / |  |
| 6 | P27W6~W8B11~B0 | RM006 | Phased_array_antenna_Current | %f | Func0010 | 0.0012207,0 | unsigned int | V |  | / |  |
| 7 | P27W9~W11B23~B12 | RM007 | reserved | %f | Func0010 | 0.0012207,0 | unsigned int | V |  | / |  |
| 8 | P27W9~W11B11~B0 | RM008 | 28V_load_current | %f | Func0010 | 0.0109863,0 | unsigned int | V |  | / |  |

TCU_Temperature_slow_TM(47H)

Pack ID=47H；Pack length=63（Bytes=64）。

Table 8-25TCU_Temperature_slow_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P47W0 | RS001 | TCU_TC_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 2 | P47W1B7~B4 | RS002 | CANbus_A_STATUS | %u | Func0001 | / | unsigned int | / |  | 0:normal;1:error;2:offline |  |
| 3 | P47W1B3~B0 | RS003 | CANbus_B_STATUS | %s | Func0001 | / | unsigned int | / |  | 0:normal;1:error;2:offline |  |
| 4 | P47W2B7~B4 | RS004 | CANbus_A_Reset_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 5 | P47W2B3~B0 | RS005 | CANbus_B_Reset_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 6 | P47W3B7 | RS006 | TCU_WORKING | %s | Func0001 | / | unsigned int | / |  | 1:Main;0:Backup |  |
| 7 | P47W3B6 | RS007 | TCU_Boot_Status | %s | Func0001 | / | unsigned int | / |  | 1:Warm_Boot;0:Cold_Boot |  |
| 8 | P47W3B5 | RS008 | Thermal_Control_Enbale | %s | Func0001 | / | unsigned int | / |  | 1:Enabled;0:Disabled |  |
| 9 | P47W3B4 | RS009 | FLASH_Parameter_Load_Status | %s | Func0001 | / | unsigned int | / |  | 1:successful;0:failure |  |
| 10 | P47W3B3~B0 | RS010 | TCU_Reset_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 11 | P47W4B7 | RS013 | HL01_Battery(backup)_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 12 | P47W4B6 | RS014 | HL02_phased_array_antenna_(main)_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 13 | P47W4B5 | RS015 | HL03_phased_array_antenna_(backup)_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 14 | P47W4B4 | RS016 | HL04_reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 15 | P47W4B3 | RS017 | HL05_reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 16 | P47W4B2 | RS018 | HL06_reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 17 | P47W4B1 | RS019 | HL07_reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 18 | P47W4B0 | RS020 | HL08_reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 19 | P47W5B7 | RS021 | HL09_reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 20 | P47W5B6 | RS022 | HL10_reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 21 | P47W5B5 | RS023 | HL11_reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 22 | P47W5B4 | RS024 | HL12_reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 23 | P47W5B3 | RS025 | HL13_Sunshield_inside_outer_circumferential_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 24 | P47W5B2 | RS026 | HL14_+Y_Skin_of_+Z_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 25 | P47W5B1 | RS027 | HL15_+Y_Skin_of_-Z_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 26 | P47W5B0 | RS028 | HL16_-Y_Skin_of_-Z_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 27 | P47W6B7 | RS029 | HL17_-Y_Skin_of_+Z_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 28 | P47W6B6 | RS030 | HL18_Top_of_+Z_frame_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 29 | P47W6B5 | RS031 | HL19_Bottom_of_+Z_frame_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 30 | P47W6B4 | RS032 | HL20_Back_Skin_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 31 | P47W6B3 | RS033 | HL21_Upper_of_Back_Skin_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 32 | P47W6B2 | RS034 | HL22_Bottom_of_Back_Skin_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 33 | P47W6B1 | RS035 | HL23_Focal_plane_splice_lens_frame_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 34 | P47W6B0 | RS036 | HL24_Focal_circuit_Inside_frame_STATUS | %s | Func0001 | / | unsigned int | / |  | 1:Heating;0:Not_Heating |  |
| 35 | P47W7~W9B23~B12 | RS037 | 01_Momentum_wheel_Y_TEMPERATURE | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 36 | P47W7~W9B11~B0 | RS038 | 02_Momentum_wheel_Z_TEMPERATURE | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 37 | P47W10~W12B23~B12 | RS039 | 03_Stack_radiation_board_2(close_to_OBDH_board)_TEMPERATURE | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 38 | P47W10~W12B11~B0 | RS040 | 04_Integrated_data_transmission_equipment_TEMPERATURE | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 39 | P47W13~W15B23~B12 | RS041 | 05_phased_array_antenna_(main)_TEMPERATURE | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 40 | P47W13~W15B11~B0 | RS042 | 06_phased_array_antenna_(backup)_TEMPERATURE | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 41 | P47W16~W18B23~B12 | RS043 | 07_Battery(backup)_TEMPERATURE | %f | Func0015 | 10000,4095,-4.36216,4081.70194,-94033.78039 | unsigned int | ℃ |  | / |  |
| 42 | P47W16~W18B11~B0 | RS044 | 08_-Z_fixed_wing_TEMPERATURE | %f | Func0015 | 10000,4095,-2.36575,3805.6269,-119817.3758 | unsigned int | ℃ |  | / |  |
| 43 | P47W19~W21B23~B12 | RS045 | 09_-Y_ixed_wing_TEMPERATURE | %f | Func0015 | 10000,4095,-0.37044,2580.40222,-74905.72737 | unsigned int | ℃ |  | / |  |
| 44 | P47W19~W21B11~B0 | RS046 | 10_Sunshield_inside_outer_circumferential_TEMPERATURE | %f | FuncExpr | 1/(0.0013447791014172+0.000225132758579982*ln(10000/(4095/RS046-1))+1.26582588979491E-06*ln(10000/(4095/RS046-1))*ln(10000/(4095/RS046-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 45 | P47W22~W24B23~B12 | RS047 | 11_+Y_Skin_of_+Z_TEMPERATURE | %f | FuncExpr | 1/(0.00134596145954311+0.000224611612576773*ln(10000/(4095/RS047-1))+1.27886414776852E-06*ln(10000/(4095/RS047-1))*ln(10000/(4095/RS047-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 46 | P47W22~W24B11~B0 | RS048 | 12_+Y_Skin_of_-Z_TEMPERATURE | %f | FuncExpr | 1/(0.00134554546616874+0.00022435739701311*ln(10000/(4095/RS048-1))+1.30748154251016E-06*ln(10000/(4095/RS048-1))*ln(10000/(4095/RS048-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 47 | P47W25~W27B23~B12 | RS049 | 13_-Y_Skin_of_-Z_TEMPERATURE | %f | FuncExpr | 1/(0.00134912781893076+0.000224202934238328*ln(10000/(4095/RS049-1))+1.30464731049046E-06*ln(10000/(4095/RS049-1))*ln(10000/(4095/RS049-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 48 | P47W25~W27B11~B0 | RS050 | 14_-Y_Skin_of_+Z_TEMPERATURE | %f | FuncExpr | 1/(0.00134228719258436+0.000225456131480075*ln(10000/(4095/RS050-1))+1.26153785971454E-06*ln(10000/(4095/RS050-1))*ln(10000/(4095/RS050-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 49 | P47W28~W30B23~B12 | RS051 | 15_Top_of_+Z_frame_TEMPERATURE | %f | FuncExpr | 1/(0.00134710559301311+0.000225047895687175*ln(10000/(4095/RS051-1))+1.26401249612662E-06*ln(10000/(4095/RS051-1))*ln(10000/(4095/RS051-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 50 | P47W28~W30B11~B0 | RS052 | 16_Bottom_of_+Z_frame_TEMPERATURE | %f | FuncExpr | 1/(0.00134865314608177+0.000224571159542878*ln(10000/(4095/RS052-1))+1.27285779948928E-06*ln(10000/(4095/RS052-1))*ln(10000/(4095/RS052-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 51 | P47W31~W33B23~B12 | RS053 | 17_Back_Skin_TEMPERATURE | %f | FuncExpr | 1/(0.00135111287430053+0.000224250439218674*ln(10000/(4095/RS053-1))+1.2909489563754E-06*ln(10000/(4095/RS053-1))*ln(10000/(4095/RS053-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 52 | P47W31~W33B11~B0 | RS054 | 18_Upper_of_Back_Skin_TEMPERATURE | %f | FuncExpr | 1/(0.00134449593327334+0.000225287401945275*ln(10000/(4095/RS054-1))+1.26051597122979E-06*ln(10000/(4095/RS054-1))*ln(10000/(4095/RS054-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 53 | P47W34~W36B23~B12 | RS055 | 19_Bottom_of_Back_Skin_TEMPERATURE | %f | FuncExpr | 1/(0.00134198290673104+0.000225877857305288*ln(10000/(4095/RS055-1))+1.22451265919808E-06*ln(10000/(4095/RS055-1))*ln(10000/(4095/RS055-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 54 | P47W34~W36B11~B0 | RS056 | 20_Focal_plane_splice_lens_frame_TEMPERATURE | %f | FuncExpr | 1/(0.0013496927616621+0.000224378827360622*ln(10000/(4095/RS056-1))+1.29871967485543E-06*ln(10000/(4095/RS056-1))*ln(10000/(4095/RS056-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 55 | P47W37~W39B23~B12 | RS057 | 21_Focal_circuit_Inside_frame_TEMPERATURE | %f | FuncExpr | 1/(0.00134207092245985+0.00022505424461782*ln(10000/(4095/RS057-1))+1.30454510729852E-06*ln(10000/(4095/RS057-1))*ln(10000/(4095/RS057-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 56 | P47W37~W39B11~B0 | RS058 | 22-Main&Third_lens_cover_TEMPERATURE | %f | FuncExpr | 1/(0.00134565938121189+0.0002250809717496*ln(10000/(4095/RS058-1))+1.27927047998493E-06*ln(10000/(4095/RS058-1))*ln(10000/(4095/RS058-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 57 | P47W40~W42B23~B12 | RS059 | 23-Secondary_lens_cover_TEMPERATURE | %f | FuncExpr | 1/(0.00134730055582791+0.000224887281965641*ln(10000/(4095/RS059-1))+1.28181266029262E-06*ln(10000/(4095/RS059-1))*ln(10000/(4095/RS059-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 58 | P47W40~W42B11~B0 | RS060 | 24-CCD_TEMPERATURE | %f | FuncExpr | 1/(0.0013518830903529+0.000224131842360052*ln(10000/(4095/RS060-1))+1.30166838305161E-06*ln(10000/(4095/RS060-1))*ln(10000/(4095/RS060-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 59 | P47W43B7~B4 | RS061 | 01_Battery(backup)_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 60 | P47W43B3~B0 | RS062 | 02_phased_array_antenna_(main)_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 61 | P47W44B7~B4 | RS063 | 03_phased_array_antenna_(backup)_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 62 | P47W44B3~B0 | RS064 | 04_reserved | %u | Func0001 | / | unsigned int |  |  | / |  |
| 63 | P47W45B7~B4 | RS065 | 05_reserved | %u | Func0001 | / | unsigned int |  |  | / |  |
| 64 | P47W45B3~B0 | RS066 | 06_reserved | %u | Func0001 | / | unsigned int |  |  | / |  |
| 65 | P47W46B7~B4 | RS067 | 07_reserved | %u | Func0001 | / | unsigned int |  |  | / |  |
| 66 | P47W46B3~B0 | RS068 | 08_reserved | %u | Func0001 | / | unsigned int |  |  | / |  |
| 67 | P47W47B7~B4 | RS069 | 09_reserved | %u | Func0001 | / | unsigned int |  |  | / |  |
| 68 | P47W47B3~B0 | RS070 | 10_reserved | %u | Func0001 | / | unsigned int |  |  | / |  |
| 69 | P47W48B7~B4 | RS071 | 11_reserved | %u | Func0001 | / | unsigned int |  |  | / |  |
| 70 | P47W48B3~B0 | RS072 | 12_reserved | %u | Func0001 | / | unsigned int |  |  | / |  |
| 71 | P47W49B7~B4 | RS073 | 13_Sunshield_inside_outer_circumferential_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 72 | P47W49B3~B0 | RS074 | 14_+Y_Skin_of_+Z_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 73 | P47W50B7~B4 | RS075 | 15_+Y_Skin_of_-Z_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 74 | P47W50B3~B0 | RS076 | 16_-Y_Skin_of_-Z_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 75 | P47W51B7~B4 | RS077 | 17_-Y_Skin_of_+Z_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 76 | P47W51B3~B0 | RS078 | 18_Top_of_+Z_frame_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 77 | P47W52B7~B4 | RS079 | 19_Bottom_of_+Z_frame_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 78 | P47W52B3~B0 | RS080 | 20_Back_Skin_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 79 | P47W53B7~B4 | RS081 | 21_Upper_of_Back_Skin_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 80 | P47W53B3~B0 | RS082 | 22_Bottom_of_Back_Skin_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 81 | P47W54B7~B4 | RS083 | 23_Focal_plane_splice_lens_frame_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 82 | P47W54B3~B0 | RS084 | 24_Focal_circuit_Inside_frame_DUTY_RATIO | %u | Func0007 | 10 | unsigned int | % |  | / |  |
| 83 | P47W55B7 | RS085 | 01_Momentum_wheel_Y_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 84 | P47W55B6 | RS086 | 02_Momentum_wheel_Z | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 85 | P47W55B5 | RS087 | 03_Stack_radiation_board_2(close_to_OBDH_board)_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 86 | P47W55B4 | RS088 | 04_Integrated_data_transmission_equipment_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 87 | P47W55B3 | RS089 | 05_phased_array_antenna_(main)_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 88 | P47W55B2 | RS090 | 06_phased_array_antenna_(backup)_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 89 | P47W55B1 | RS091 | 07_Battery(backup)_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 90 | P47W55B0 | RS092 | 08_-Z_fixed_wing_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 91 | P47W56B7 | RS093 | 09_-Y_ixed_wing_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 92 | P47W56B6 | RS094 | 10_Sunshield_inside_outer_circumferential_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 93 | P47W56B5 | RS095 | 11_+Y_Skin_of_+Z_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 94 | P47W56B4 | RS096 | 12_+Y_Skin_of_-Z_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 95 | P47W56B3 | RS097 | 13_-Y_Skin_of_-Z_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 96 | P47W56B2 | RS098 | 14_-Y_Skin_of_+Z_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 97 | P47W56B1 | RS099 | 15_Top_of_+Z_frame_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 98 | P47W56B0 | RS100 | 16_Bottom_of_+Z_frame_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 99 | P47W57B7 | RS101 | 17_Back_Skin_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 100 | P47W57B6 | RS102 | 18_Upper_of_Back_Skin_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 101 | P47W57B5 | RS103 | 19_Bottom_of_Back_Skin_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 102 | P47W57B4 | RS104 | 20_Focal_plane_splice_lens_frame_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 103 | P47W57B3 | RS105 | 21_Focal_circuit_Inside_frame_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 104 | P47W57B2 | RS106 | 22_Main&Third_lens_cover_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 105 | P47W57B1 | RS107 | 23_Secondary_lens_cover_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 106 | P47W57B0 | RS108 | 24_CCD_temperature_HEALTH_STATUS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 107 | P47W58 | RS109 | FLASHOperationSector | %u | Func0001 | / | unsigned int |  |  | / |  |
| 108 | P47W59B7~B6 | RS110 | FLSHOperationMode | %s | Func0001 | / | unsigned int |  |  | 2:To_Write;1:To_Erasy;0:Idle |  |
| 109 | P47W59B5~B4 | RS111 | UsingTMR | %s | Func0001 | / | unsigned int |  |  | 2:used;0:unused |  |
| 110 | P47W59B3~B0 | RS112 | FLASHStatus | %s | Func0001 | / | unsigned int |  |  | 9:Failure;6:Successful;4:Writing;2:Erasing;0:Idle |  |
| 111 | P47W60B7 | RS113 | 01_Battery(backup)_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 112 | P47W60B6 | RS114 | 02_phased_array_antenna_(main)_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 113 | P47W60B5 | RS115 | 03_phased_array_antenna_(backup)_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 114 | P47W60B4 | RS116 | 04_reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 115 | P47W60B3 | RS117 | 05_reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 116 | P47W60B2 | RS118 | 06_reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 117 | P47W60B1 | RS119 | 07_reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 118 | P47W60B0 | RS120 | 08_reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 119 | P47W61B7 | RS121 | 09_reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 120 | P47W61B6 | RS122 | 10_reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 121 | P47W61B5 | RS123 | 11_reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 122 | P47W61B4 | RS124 | 12_reserved | %X | Func0001 | / | unsigned int |  |  | / |  |
| 123 | P47W61B3 | RS125 | 13_Sunshield_inside_outer_circumferential_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 124 | P47W61B2 | RS126 | 14_+Y_Skin_of_+Z_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 125 | P47W61B1 | RS127 | 15_+Y_Skin_of_-Z_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 126 | P47W61B0 | RS128 | 16_-Y_Skin_of_-Z_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 127 | P47W62B7 | RS129 | 17_-Y_Skin_of_+Z_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 128 | P47W62B6 | RS130 | 18_Top_of_+Z_frame_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 129 | P47W62B5 | RS131 | 19_Bottom_of_+Z_frame_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 130 | P47W62B4 | RS132 | 20_Back_Skin_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 131 | P47W62B3 | RS133 | 21_Upper_of_Back_Skin_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 132 | P47W62B2 | RS134 | 22_Bottom_of_Back_Skin_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 133 | P47W62B1 | RS135 | 23_Focal_plane_splice_lens_frame_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 134 | P47W62B0 | RS136 | 24_Focal_circuit_Inside_frame_HL_HEALTH_STAUTS | %s | Func0001 | / | unsigned int |  |  | 1:Error;0:Normal |  |
| 135 | P47W63B7 | RS137 | Up_task_1_running_status | %s | Func0001 | / | unsigned int |  |  | 1:Running;0:Suspending |  |
| 136 | P47W63B6 | RS138 | Up_task_2_running_status | %s | Func0001 | / | unsigned int |  |  | 1:Running;0:Suspending |  |
| 137 | P47W63B5 | RS139 | Up_task_3_running_status | %s | Func0001 | / | unsigned int |  |  | 1:Running;0:Suspending |  |
| 138 | P47W63B4 | RS140 | Up_task_4_running_status | %s | Func0001 | / | unsigned int |  |  | 1:Running;0:Suspending |  |
| 139 | P47W63B3 | RS141 | Up_task_1_check_status | %s | Func0001 | / | unsigned int |  |  | 1:Right;0:Error |  |
| 140 | P47W63B2 | RS142 | Up_task_2_check_status | %s | Func0001 | / | unsigned int |  |  | 1:Right;0:Error |  |
| 141 | P47W63B1 | RS143 | Up_task_3_check_status | %s | Func0001 | / | unsigned int |  |  | 1:Right;0:Error |  |
| 142 | P47W63B0 | RS144 | Up_task_4_check_status | %s | Func0001 | / | unsigned int |  |  | 1:Right;0:Error |  |

TCU_Configuration_slow_TM(67H)

Pack ID=67H；Pack length=107（Bytes=108）。

Table 8-26TCU_Configuration_slow_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P67W0 | RH001 | TCU_Packet_Type | %u | Func0001 | / | unsigned int | / |  | / |  |
| 2 | P67W1 | RH002 | OBC_require_Backup_Data_COUNTER | %u | Func0001 | / | unsigned int | / |  | / |  |
| 3 | P67W2~W5 | RH003 | OBC_require_Backup_Data_Time | %u | Func0001 | / | unsigned int | / |  | / |  |
| 4 | P67W6~W7 | RH004 | TTC/D_backup_Number | %u | Func0001 | / | unsigned int | / |  | / |  |
| 5 | P67W8~W10B23~B12 | RH005 | HL01_Battery(backup)_High_THRESHOLD | %f | Func0015 | 10000,4095,-4.36216,4081.70194,-94033.78039 | unsigned int | ℃ |  | / |  |
| 6 | P67W8~W10B11~B0 | RH006 | HL01_Battery(backup)_Low_THRESHOLD | %f | Func0015 | 10000,4095,-4.36216,4081.70194,-94033.78039 | unsigned int | ℃ |  | / |  |
| 7 | P67W11~W13B23~B12 | RH007 | HL02_phased_array_antenna_(main) | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 8 | P67W11~W13B11~B0 | RH008 | HL02_phased_array_antenna_(main) | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 9 | P67W14~W16B23~B12 | RH009 | HL03_phased_array_antenna_(backup) | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 10 | P67W14~W16B11~B0 | RH010 | HL03_phased_array_antenna_(backup) | %f | Func0015 | 10000,4095,-6.01188,4622.53337,-86421.72414 | unsigned int | ℃ |  | / |  |
| 11 | P67W17~W19B23~B12 | RH011 | HL04_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 12 | P67W17~W19B11~B0 | RH012 | HL04_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 13 | P67W20~W22B23~B12 | RH013 | HL05_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 14 | P67W20~W22B11~B0 | RH014 | HL05_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 15 | P67W23~W25B23~B12 | RH015 | HL06_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 16 | P67W23~W25B11~B0 | RH016 | HL06_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 17 | P67W26~W28B23~B12 | RH017 | HL07_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 18 | P67W26~W28B11~B0 | RH018 | HL07_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 19 | P67W29~W31B23~B12 | RH019 | HL08_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 20 | P67W29~W31B11~B0 | RH020 | HL08_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 21 | P67W32~W34B23~B12 | RH021 | HL09_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 22 | P67W32~W34B11~B0 | RH022 | HL09_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 23 | P67W35~W37B23~B12 | RH023 | HL10_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 24 | P67W35~W37B11~B0 | RH024 | HL10_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 25 | P67W38~W40B23~B12 | RH025 | HL11_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 26 | P67W38~W40B11~B0 | RH026 | HL11_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 27 | P67W41~W43B23~B12 | RH027 | HL12_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 28 | P67W41~W43B11~B0 | RH028 | HL12_reserved | %u | Func0001 | / | unsigned int | ℃ |  | / |  |
| 29 | P67W44~W46B23~B12 | RH029 | HL13_Sunshield_inside_outer_circumferential_High_THRESHOLD | %f | FuncExpr | 1/(0.0013447791014172+0.000225132758579982*ln(10000/(4095/RS046-1))+1.26582588979491E-06*ln(10000/(4095/RS046-1))*ln(10000/(4095/RS046-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 30 | P67W44~W46B11~B0 | RH030 | HL13_Sunshield_inside_outer_circumferential_Low_THRESHOLD | %f | FuncExpr | 1/(0.0013447791014172+0.000225132758579982*ln(10000/(4095/RS046-1))+1.26582588979491E-06*ln(10000/(4095/RS046-1))*ln(10000/(4095/RS046-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 31 | P67W47~W49B23~B12 | RH031 | HL14_+Y_Skin_of_+Z_High_THRESHOLD | %f | FuncExpr | 1/(0.00134596145954311+0.000224611612576773*ln(10000/(4095/RS047-1))+1.27886414776852E-06*ln(10000/(4095/RS047-1))*ln(10000/(4095/RS047-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 32 | P67W47~W49B11~B0 | RH032 | HL14_+Y_Skin_of_+Z_Low_THRESHOLD | %f | FuncExpr | 1/(0.00134596145954311+0.000224611612576773*ln(10000/(4095/RS047-1))+1.27886414776852E-06*ln(10000/(4095/RS047-1))*ln(10000/(4095/RS047-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 33 | P67W50~W52B23~B12 | RH033 | HL15_+Y_Skin_of_-Z_High_THRESHOLD | %f | FuncExpr | 1/(0.00134554546616874+0.00022435739701311*ln(10000/(4095/RS048-1))+1.30748154251016E-06*ln(10000/(4095/RS048-1))*ln(10000/(4095/RS048-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 34 | P67W50~W52B11~B0 | RH034 | HL15_+Y_Skin_of_-Z_Low_THRESHOLD | %f | FuncExpr | 1/(0.00134554546616874+0.00022435739701311*ln(10000/(4095/RS048-1))+1.30748154251016E-06*ln(10000/(4095/RS048-1))*ln(10000/(4095/RS048-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 35 | P67W53~W55B23~B12 | RH035 | HL16_-Y_Skin_of_-Z_High_THRESHOLD | %f | FuncExpr | 1/(0.00134912781893076+0.000224202934238328*ln(10000/(4095/RS049-1))+1.30464731049046E-06*ln(10000/(4095/RS049-1))*ln(10000/(4095/RS049-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 36 | P67W53~W55B11~B0 | RH036 | HL16_-Y_Skin_of_-Z_Low_THRESHOLD | %f | FuncExpr | 1/(0.00134912781893076+0.000224202934238328*ln(10000/(4095/RS049-1))+1.30464731049046E-06*ln(10000/(4095/RS049-1))*ln(10000/(4095/RS049-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 37 | P67W56~W58B23~B12 | RH037 | HL17_-Y_Skin_of_+Z_High_THRESHOLD | %f | FuncExpr | 1/(0.00134228719258436+0.000225456131480075*ln(10000/(4095/RS050-1))+1.26153785971454E-06*ln(10000/(4095/RS050-1))*ln(10000/(4095/RS050-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 38 | P67W56~W58B11~B0 | RH038 | HL17_-Y_Skin_of_+Z_Low_THRESHOLD | %f | FuncExpr | 1/(0.00134228719258436+0.000225456131480075*ln(10000/(4095/RS050-1))+1.26153785971454E-06*ln(10000/(4095/RS050-1))*ln(10000/(4095/RS050-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 39 | P67W59~W61B23~B12 | RH039 | HL18_Top_of_+Z_frame_High_THRESHOLD | %f | FuncExpr | 1/(0.00134710559301311+0.000225047895687175*ln(10000/(4095/RS051-1))+1.26401249612662E-06*ln(10000/(4095/RS051-1))*ln(10000/(4095/RS051-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 40 | P67W59~W61B11~B0 | RH040 | HL18_Top_of_+Z_frame_Low_THRESHOLD | %f | FuncExpr | 1/(0.00134710559301311+0.000225047895687175*ln(10000/(4095/RS051-1))+1.26401249612662E-06*ln(10000/(4095/RS051-1))*ln(10000/(4095/RS051-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 41 | P67W62~W64B23~B12 | RH041 | HL19_Bottom_of_+Z_frame_High_THRESHOLD | %f | FuncExpr | 1/(0.00134865314608177+0.000224571159542878*ln(10000/(4095/RS052-1))+1.27285779948928E-06*ln(10000/(4095/RS052-1))*ln(10000/(4095/RS052-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 42 | P67W62~W64B11~B0 | RH042 | HL19_Bottom_of_+Z_frame_Low_THRESHOLD | %f | FuncExpr | 1/(0.00134865314608177+0.000224571159542878*ln(10000/(4095/RS052-1))+1.27285779948928E-06*ln(10000/(4095/RS052-1))*ln(10000/(4095/RS052-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 43 | P67W65~W67B23~B12 | RH043 | HL20_Back_Skin_High_THRESHOLD | %f | FuncExpr | 1/(0.00135111287430053+0.000224250439218674*ln(10000/(4095/RS053-1))+1.2909489563754E-06*ln(10000/(4095/RS053-1))*ln(10000/(4095/RS053-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 44 | P67W65~W67B11~B0 | RH044 | HL20_Back_Skin_Low_THRESHOLD | %f | FuncExpr | 1/(0.00135111287430053+0.000224250439218674*ln(10000/(4095/RS053-1))+1.2909489563754E-06*ln(10000/(4095/RS053-1))*ln(10000/(4095/RS053-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 45 | P67W68~W70B23~B12 | RH045 | HL21_Upper_of_Back_Skin_High_THRESHOLD | %f | FuncExpr | 1/(0.00134449593327334+0.000225287401945275*ln(10000/(4095/RS054-1))+1.26051597122979E-06*ln(10000/(4095/RS054-1))*ln(10000/(4095/RS054-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 46 | P67W68~W70B11~B0 | RH046 | HL21_Upper_of_Back_Skin_Low_THRESHOLD | %f | FuncExpr | 1/(0.00134449593327334+0.000225287401945275*ln(10000/(4095/RS054-1))+1.26051597122979E-06*ln(10000/(4095/RS054-1))*ln(10000/(4095/RS054-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 47 | P67W71~W73B23~B12 | RH047 | HL22_Bottom_of_Back_Skin_High_THRESHOLD | %f | FuncExpr | 1/(0.00134198290673104+0.000225877857305288*ln(10000/(4095/RS055-1))+1.22451265919808E-06*ln(10000/(4095/RS055-1))*ln(10000/(4095/RS055-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 48 | P67W71~W73B11~B0 | RH048 | HL22_Bottom_of_Back_Skin_Low_THRESHOLD | %f | FuncExpr | 1/(0.00134198290673104+0.000225877857305288*ln(10000/(4095/RS055-1))+1.22451265919808E-06*ln(10000/(4095/RS055-1))*ln(10000/(4095/RS055-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 49 | P67W74~W76B23~B12 | RH049 | HL23_Focal_plane_splice_lens_frame_High_THRESHOLD | %f | FuncExpr | 1/(0.0013496927616621+0.000224378827360622*ln(10000/(4095/RS056-1))+1.29871967485543E-06*ln(10000/(4095/RS056-1))*ln(10000/(4095/RS056-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 50 | P67W74~W76B11~B0 | RH050 | HL23_Focal_plane_splice_lens_frame_Low_THRESHOLD | %f | FuncExpr | 1/(0.0013496927616621+0.000224378827360622*ln(10000/(4095/RS056-1))+1.29871967485543E-06*ln(10000/(4095/RS056-1))*ln(10000/(4095/RS056-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 51 | P67W77~W79B23~B12 | RH051 | HL24_Focal_circuit_Inside_frame_High_THRESHOLD | %f | FuncExpr | 1/(0.00134207092245985+0.00022505424461782*ln(10000/(4095/RS057-1))+1.30454510729852E-06*ln(10000/(4095/RS057-1))*ln(10000/(4095/RS057-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 52 | P67W77~W79B11~B0 | RH052 | HL24_Focal_circuit_Inside_frame_Low_THRESHOLD | %f | FuncExpr | 1/(0.00134207092245985+0.00022505424461782*ln(10000/(4095/RS057-1))+1.30454510729852E-06*ln(10000/(4095/RS057-1))*ln(10000/(4095/RS057-1)))-273.15 | unsigned int | ℃ |  | / |  |
| 53 | P67W80 | RH053 | HL01_Battery(backup)_CONTROL_MODE | %s | Func0001 | / | unsigned int | / | / | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 54 | P67W81 | RH054 | HL02_phased_array_antenna_(main)_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 55 | P67W82 | RH055 | HL03_phased_array_antenna_(backup)_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 56 | P67W83 | RH056 | HL04_reserved | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 57 | P67W84 | RH057 | HL05_reserved | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 58 | P67W85 | RH058 | HL06_reserved | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 59 | P67W86 | RH059 | HL07_reserved | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 60 | P67W87 | RH060 | HL08_reserved | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 61 | P67W88 | RH061 | HL09_reserved | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 62 | P67W89 | RH062 | HL10_reserved | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 63 | P67W90 | RH063 | HL11_reserved | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 64 | P67W91 | RH064 | HL12_reserved | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 65 | P67W92 | RH065 | HL13_Sunshield_inside_outer_circumferential_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 66 | P67W93 | RH066 | HL14_+Y_Skin_of_+Z_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 67 | P67W94 | RH067 | HL15_+Y_Skin_of_-Z_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 68 | P67W95 | RH068 | HL16_-Y_Skin_of_-Z_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 69 | P67W96 | RH069 | HL17_-Y_Skin_of_+Z_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 70 | P67W97 | RH071 | HL18_Top_of_+Z_frame_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 71 | P67W98 | RH072 | HL19_Bottom_of_+Z_frame_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 72 | P67W99 | RH073 | HL20_Back_Skin_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 73 | P67W100 | RH074 | HL21_Upper_of_Back_Skin_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 74 | P67W101 | RH075 | HL22_Bottom_of_Back_Skin_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 75 | P67W102 | RH076 | HL23_Focal_plane_splice_lens_frame_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 76 | P67W103 | RH077 | HL24_Focal_circuit_Inside_frame_CONTROL_MODE | %s | Func0001 | / | unsigned int | / |  | 0x60:Open_Loop-On;0xA0:Open_Loop-Off;0x90:Close_Loop-Proportional_control-Min;0x91:Close_Loop-Proportional_control-R1;0x92:Close_Loop-Proportional_control-R2;0x93:Close_Loop-Proportional_control-R3;0x9F:Close_Loop-Proportional_control-Average;0x50:Close_Loop-Threshold_Control-Min;0x51:Close_Loop-Threshold_Control-R1;0x52:Close_Loop-Threshold_Control-R2;0x53:Close_Loop-Threshold_Control-R3;0x5F:Close_Loop-Threshold_Control-Average |  |
| 77 | P67W104 | RH078 | reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 78 | P67W105 | RH079 | reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 79 | P67W106 | RH080 | reserved | %u | Func0001 | / | unsigned int | / |  | / |  |
| 80 | P67W107 | RH081 | reserved | %u | Func0001 | / | unsigned int | / |  | / |  |

DCU

DTS_fast_TM(2BH)

Pack ID=2BH；Pack length=7（Bytes=8）。

Table 8-27DTS_fast_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P2BW0 | TMYA001 | Reserved | %X | Func0001 | / | unsigned int | / |  | / | / |
| 2 | P2BW1B7 | TMYA002 | Integrate Transmission Machine Work State | %s | Func0001 | / | unsigned int | / |  | 1:loaded successfully | 0~1B |
| 3 | P2BW1B6 | TMYA003 | RF On/Off State | %s | Func0001 | / | unsigned int | / |  | 0:RF&Antenna TURN OFF;1:RF&Antenna TURN ON | 0~1B |
| 4 | P2BW1B5 | TMYA004 | Real-time Transmission State | %s | Func0001 | / | unsigned int | / |  | 1:Real-time Transmission Stop; 0:Real-time Transmission Strat | 0~1B |
| 5 | P2BW1B4 | TMYA005 | Modulation Spectrum State | %s | Func0001 | / | unsigned int | / |  | 0:Modulation Mode;1:Carrier Mode | 0~1B |
| 6 | P2BW1B3 | TMYA006 | Reserved | %s | Func0001 | / | unsigned int | / |  | / |  |
| 7 | P2BW1B2 | TMYA007 | Reserved | %s | Func0001 | / | unsigned int | / |  | / |  |
| 8 | P2BW1B1 | TMYA008 | Compression Ratio | %s | Func0001 | / | unsigned int | / |  | 0: 2:1CompressMode; 1:Lossless CompressMode | 0~1B |
| 9 | P2BW1B0 | TMYA009 | Reserved | %s | Func0001 | / | unsigned int | / |  | / |  |
| 10 | P2BW2B7~B4 | TMYA010 | SSM MAIN/REDN State | %s | Func0001 | / | unsigned int | / |  | 0000:SSM Main Work; 1111:SSM Backup Work | 0000~1111B |
| 11 | P2BW2B3~B0 | TMYA011 | SSM Work State | %s | Func0001 | / | unsigned int | / |  | 0000:SSM standyby; 0001:SSM self-Detecting 0010:SSM erasing; 0011:SSM erasing over ; 0100:SSM resetting; 0101:SSM resetting over; 0110:SSM recording by sequence; 0111:SSM recording over; 1000:SSM replaying by sequence; 1001:SSM recording by address; 1010:SSM replaying by sequence; 1011:SSM replaying over ;1100:SSM self-Detectingover | 0000~1111B |
| 12 | P2BW3B7~B4 | TMYA012 | CAN A Reset Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 13 | P2BW3B3~B0 | TMYA013 | CAN B Reset Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 14 | P2BW4B7~B4 | TMYA014 | CAN A Command Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 15 | P2BW4B3~B0 | TMYA015 | CAN B Command Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 16 | P2BW5B7~B4 | TMYA016 | SSM Capacity State | %s | Func0001 | / | unsigned int | / |  | 0000:SMM no full; 0011:SSM empty; 1100:SSM FULL | 0000~1111B |
| 17 | P2BW5B3~B2 | TMYA017 | Pointer Save Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~3 |
| 18 | P2BW5B1~B0 | TMYA018 | Pointer Recovery Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~3 |
| 19 | P2BW6B7~B4 | TMYA019 | Receiving GPS Broadcast Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 20 | P2BW6B3~B0 | TMYA020 | Forwarding GPS Broadcast Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 21 | P2BW7B7~B4 | TMYA021 | Receiving Satellite Time Broadcast Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 22 | P2BW7B3~B0 | TMYA022 | Forwarding Satellite Time Broadcast Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |

DTS_slow_TM(4BH)

Pack ID=4BH；Pack length=15（Bytes=16）。

Table 8-28DTS_slow_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P4BW0~W1 | TMYA023 | SSM Recording Pointer | %X | Func0001 | / | unsigned int | / |  | / |  |
| 2 | P4BW02B7~B4 | TMYA024 | Receiving Attitude Broadcast Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 3 | P4BW02B3~B0 | TMYA025 | Forwarding Attitude Broadcast Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 4 | P4BW03B7 | TMYA026 | GPS time availability sign | %s | Func0001 | / | unsigned int | / |  | 1:effective value; 0:invalid value | 0/1B |
| 5 | P4BW03B6~B4 | TMYA027 | GPS positioning or orbit determination data | %s | Func0001 | / | unsigned int | / |  | 101:Absoulte positioning data; 011:Orbit determination date; 110:Orbit extrapolating data | 000~111B |
| 6 | P4BW03B3~B0 | TMYA028 | GPS data error counter | %u | Func0001 | / | unsigned int | / |  | / | 0~15 |
| 7 | P4BW4~W5 | TMYA029 | SSM Playback Pointer | %X | Func0001 | / | unsigned int | / |  | / |  |
| 8 | P4BW6~W7 | TMYA030 | SSM Remainder Space | %u | Func0001 | / | unsigned int | / |  | / | 0~16383 |
| 9 | P4BW8~W9 | TMYA031 | Antenna Central Angle | %d | Func0001 | / | complement | 0.1° |  | / | 0°~+360° |
| 10 | P4BW10~W11 | TMYA032 | Antenna Azimuth | %d | Func0001 | / | complement | 0.1° |  | / | -360.0°~+360.0° |
| 11 | P4BW12 | TMYA033 | Antenna Receiving Command Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~255 |
| 12 | P4BW13 | TMYA034 | Antenna Receiving Command Successful Counter | %u | Func0001 | / | unsigned int | / |  | / | 0~255 |
| 13 | P4BW14B7 | TMYA035 | Antenna Time Synchonization State | %s | Func0001 | / | unsigned int | / |  | 0:Time Synchonizationfailure; 1:Time Synchonizationsuccesssful; | 0~1B |
| 14 | P4BW14B6 | TMYA036 | Antenna Upload Data Package State | %s | Func0001 | / | unsigned int | / |  | 0:Data write failure; 1:Data write successsful; | 0~1B |
| 15 | P4BW14B5~B4 | TMYA037 | Antenna Working mode | %s | Func0001 | / | unsigned int | / |  | 01:Autonomic Computing mode; 00:Standby mode; 10:Upload diretection Angles mode | 00~11B |
| 16 | P4BW14B3~B0 | TMYA038 | Ground Station marker | %s | Func0001 | / | unsigned int | / |  | 0001:Ground Station1;0010:Ground Station2;0011:Ground Station 3;0100:Ground Station 4;0101:Ground Station 5;0110:Ground Station 6;0111:Ground Station 7;1000:Ground Station 8 | 0000~1111B |
| 17 | P4BW15B7~B4 | TMYA039 | Antenna Internal Time | %s | Func0001 | / | unsigned int | / |  | / | 0000~1111B |
| 18 | P4BW15B3~B0 | TMYA040 | Antenna Receive GPS Broadcast Counter | %s | Func0001 | / | unsigned int | / |  | / | 0000~1111B |

GPS

GPS_fast_TM(2AH)

Pack ID=2AH；Pack length=19（Bytes=20）。

Table 8-29GPS_fast_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P2AW0~W3 | GS001 | UTC cumulative seconds | %d | Func0001 | / | unsigned int | s | [0,4294967295] | / |  |
| 2 | P2AW4B7~B4 | GS002 | Number of captured stars | %d | Func0001 | / | unsigned int |  | [0,12] | / |  |
| 3 | P2AW4B3~B0 | GS003 | Number of Available stars | %d | Func0001 | / | unsigned int |  | [0,12] | / |  |
| 4 | P2AW5B7 | GS004 | GPS A/B mark | %s | Func0001 | / | unsigned int |  |  | 0:GPS A on duty;1:GPS B on duty |  |
| 5 | P2AW5B6 | GS005 | Positioning status | %s | Func0001 | / | unsigned int |  |  | 1:positioning state;0:non-positioning state |  |
| 6 | P2AW5B5~B4 | GS006 | Orbit determination state | %s | Func0001 | / | unsigned int |  |  | 0:Invalidation;1:Invalidation;2:Extrapolating;3:Orbit determination |  |
| 7 | P2AW5B3~B2 | GS007 | Positioning solution state | %s | Func0001 | / | unsigned int |  |  | 0:not resolving;1:GPS resolving;2:invalidity;3:invalidity |  |
| 8 | P2AW5B1~B0 | GS008 | Orbit determination solution state | %s | Func0001 | / | unsigned int |  |  | 0:not resolving;1:GPS resolving;2:invalidity;3:invalidity |  |
| 9 | P2AW6~W9B31~B30 | GS009 | Main core and slave core communication status | %s | Func0001 | / | unsigned int |  |  | 0:normal;1:fault of frame head or check sum;2:communication overtime;3:invalidity |  |
| 10 | P2AW6~W9B29 | GS010 | Tmark status | %s | Func0001 | / | unsigned int |  |  | 0:normal;1:overtime fault |  |
| 11 | P2AW6~W9B28~B26 | GS011 | Positioning module non-positioning reason | %s | Func0001 | / | unsigned int |  |  | 0:initial state;1:LS return value abnormity;2:GDOP exceed a quota;3:GPS time resolving exceed a quota;4:user single axis position exceed a quota;5:user location and speed resolving exceed a quota;6:user time data exceed a quota;7:ture form |  |
| 12 | P2AW6~W9B25~B23 | GS012 | Highest carrier-to-noise ratio flag | %s | Func0001 | / | unsigned int |  |  | 0:<30dB;1:30~40dB;2:40~50dB;3:>50dB |  |
| 13 | P2AW6~W9B22~B0 | GS013 | Current Orbit determination health word | %d | Func0001 | / | unsigned int |  |  | / |  |
| 14 | P2AW10~W18 | GS014 | Channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 15 | P2AW19 | GS015 | Reserved | %s | Func0001 | / | unsigned int |  |  | / |  |

GPS_status_slow_TM(4AH)

Pack ID=4AH；Pack length=127（Bytes=128）。

Table 8-30GPS_status_slow_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P4AW0~W3 | GH001 | TIC count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 2 | P4AW4~W5B15~B14 | GH002 | Reserved | %d | Func0001 | / | unsigned int |  |  | / |  |
| 3 | P4AW4~W5B13~B12 | GH003 | Positioning status | %s | Func0001 | / | unsigned int |  |  | 3:positioning;0:non-positioning |  |
| 4 | P4AW4~W5B11~B10 | GH004 | Orbit determination state | %s | Func0001 | / | unsigned int |  |  | 3:orbit determination;2:extrapolating;0:invalidation |  |
| 5 | P4AW4~W5B9~B8 | GH005 | Operating mode | %s | Func0001 | / | unsigned int |  |  | 1:GPS mode |  |
| 6 | P4AW4~W5B7~B6 | GH006 | Positioning solution state | %s | Func0001 | / | unsigned int |  |  | 0:not resolving;1:GPS resolving |  |
| 7 | P4AW4~W5B5~B4 | GH007 | Orbit determination solution state | %s | Func0001 | / | unsigned int |  |  | 0:not resolving;1:GPS resolving |  |
| 8 | P4AW4~W5B3~B2 | GH008 | Orbit determination output enable flag | %s | Func0001 | / | unsigned int |  |  | 3:permission;2:forbidden |  |
| 9 | P4AW4~W5B1~B0 | GH009 | Second pulse output enable flag | %d | Func0001 | / | unsigned int |  |  | 3:permission;2:forbidden;0:initialization,permission |  |
| 10 | P4AW6B7~B4 | GH010 | Number of captured stars | %d | Func0001 | / | unsigned int |  | [0,12] | / |  |
| 11 | P4AW6B3~B0 | GH011 | Number of available stars | %d | Func0001 | / | unsigned int |  | [0,12] | / |  |
| 12 | P4AW7~W8 | GH012 | Sampling moment GPS week | %d | Func0001 | / | unsigned int | week | [0,65536] | / |  |
| 13 | P4AW9~W12 | GH013 | Sampling moment GPS seconds | %d | Func0001 | / | unsigned int | s | [0,604799] | / |  |
| 14 | P4AW13~W16B31~B8 | GH014 | Sampling moment GPS microsecond | %d | Func0001 | / | unsigned int | us | [0,999999] | / |  |
| 15 | P4AW13~W16B7~B0 | GH015 | GPS current leap second | %d | Func0001 | / | unsigned int | us | [0,255] | / |  |
| 16 | P4AW17~W20 | GH016 | UTC cumulative seconds | %d | Func0001 | / | unsigned int | s | [0,4294967295] | / |  |
| 17 | P4AW21~W22 | GH017 | GDOP value | %d | Func0001 | / | unsigned int |  | [0,655.35] | / |  |
| 18 | P4AW23~W27B39~B34 | GH018 | GPS 1st tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 19 | P4AW23~W27B33~B30 | GH019 | GPS 1st tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 20 | P4AW23~W27B29~B24 | GH020 | GPS 1st tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 21 | P4AW23~W27B23~B18 | GH021 | GPS 1st tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 22 | P4AW23~W27B17~B12 | GH022 | GPS 1st tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 23 | P4AW23~W27B11~B10 | GH023 | GPS 1st tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 24 | P4AW23~W27B9 | GH024 | GPS 1st tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 25 | P4AW23~W27B8 | GH025 | GPS 1st tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 26 | P4AW23~W27B7~B0 | GH026 | GPS 1st tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 27 | P4AW28~W32B39~B34 | GH027 | GPS 2nd tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 28 | P4AW28~W32B33~B30 | GH028 | GPS 2nd tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 29 | P4AW28~W32B29~B24 | GH029 | GPS 2nd tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 30 | P4AW28~W32B23~B18 | GH030 | GPS 2nd tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 31 | P4AW28~W32B17~B12 | GH031 | GPS 2nd tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 32 | P4AW28~W32B11~B10 | GH032 | GPS 2nd tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 33 | P4AW28~W32B9 | GH033 | GPS 2nd tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 34 | P4AW28~W32B8 | GH034 | GPS 2ndt tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 35 | P4AW28~W32B7~B0 | GH035 | GPS 2nd tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 36 | P4AW33~W37B39~B34 | GH036 | GPS 3rd tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 37 | P4AW33~W37B33~B30 | GH037 | GPS 3rd tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 38 | P4AW33~W37B29~B24 | GH038 | GPS 3rd tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 39 | P4AW33~W37B23~B18 | GH039 | GPS 3rd tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 40 | P4AW33~W37B17~B12 | GH040 | GPS 3rd tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 41 | P4AW33~W37B11~B10 | GH041 | GPS 3rd tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 42 | P4AW33~W37B9 | GH042 | GPS 3rd tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 43 | P4AW33~W37B8 | GH043 | GPS 3rd tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 44 | P4AW33~W37B7~B0 | GH044 | GPS 3rd tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 45 | P4AW38~W42B39~B34 | GH045 | GPS 4th tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 46 | P4AW38~W42B33~B30 | GH046 | GPS 4th tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 47 | P4AW38~W42B29~B24 | GH047 | GPS 4th tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 48 | P4AW38~W42B23~B18 | GH048 | GPS 4th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 49 | P4AW38~W42B17~B12 | GH049 | GPS 4th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 50 | P4AW38~W42B11~B10 | GH050 | GPS 4th tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 51 | P4AW38~W42B9 | GH051 | GPS 4th tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 52 | P4AW38~W42B8 | GH052 | GPS 4th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 53 | P4AW38~W42B7~B0 | GH053 | GPS 4th tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 54 | P4AW43~W47B39~B34 | GH054 | GPS 5th tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 55 | P4AW43~W47B33~B30 | GH055 | GPS 5th tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 56 | P4AW43~W47B29~B24 | GH056 | GPS 5th tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 57 | P4AW43~W47B23~B18 | GH057 | GPS 5th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 58 | P4AW43~W47B17~B12 | GH058 | GPS 5th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 59 | P4AW43~W47B11~B10 | GH059 | GPS 5th tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 60 | P4AW43~W47B9 | GH060 | GPS 5th tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 61 | P4AW43~W47B8 | GH061 | GPS 5th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 62 | P4AW43~W47B7~B0 | GH062 | GPS 5th tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 63 | P4AW48~W52B39~B34 | GH063 | GPS 6th tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 64 | P4AW48~W52B33~B30 | GH064 | GPS 6th tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 65 | P4AW48~W52B29~B24 | GH065 | GPS 6th tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 66 | P4AW48~W52B23~B18 | GH066 | GPS 6th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 67 | P4AW48~W52B17~B12 | GH067 | GPS 6th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 68 | P4AW48~W52B11~B10 | GH068 | GPS 6th tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 69 | P4AW48~W52B9 | GH069 | GPS 6th tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 70 | P4AW48~W52B8 | GH070 | GPS 6th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 71 | P4AW48~W52B7~B0 | GH071 | GPS 6th tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 72 | P4AW53~W57B39~B34 | GH072 | GPS 7th tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 73 | P4AW53~W57B33~B30 | GH073 | GPS 7th tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 74 | P4AW53~W57B29~B24 | GH074 | GPS 7th tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 75 | P4AW53~W57B23~B18 | GH075 | GPS 7th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 76 | P4AW53~W57B17~B12 | GH076 | GPS 7th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 77 | P4AW53~W57B11~B10 | GH077 | GPS 7th tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 78 | P4AW53~W57B9 | GH078 | GPS 7th tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 79 | P4AW53~W57B8 | GH079 | GPS 7th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 80 | P4AW53~W57B7~B0 | GH080 | GPS 7th tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 81 | P4AW58~W62B39~B34 | GH081 | GPS 8th tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 82 | P4AW58~W62B33~B30 | GH082 | GPS 8th tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 83 | P4AW58~W62B29~B24 | GH083 | GPS 8th tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 84 | P4AW58~W62B23~B18 | GH084 | GPS 8th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 85 | P4AW58~W62B17~B12 | GH085 | GPS 8th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 86 | P4AW58~W62B11~B10 | GH086 | GPS 8th tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 87 | P4AW58~W62B9 | GH087 | GPS 8th tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 88 | P4AW58~W62B8 | GH088 | GPS 8th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 89 | P4AW58~W62B7~B0 | GH089 | GPS 8th tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 90 | P4AW63~W67B39~B34 | GH090 | GPS 9th tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 91 | P4AW63~W67B33~B30 | GH091 | GPS 9th tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 92 | P4AW63~W67B29~B24 | GH092 | GPS 9th tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 93 | P4AW63~W67B23~B18 | GH093 | GPS 9th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 94 | P4AW63~W67B17~B12 | GH094 | GPS 9th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 95 | P4AW63~W67B11~B10 | GH095 | GPS 9th tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 96 | P4AW63~W67B9 | GH096 | GPS 9th tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 97 | P4AW63~W67B8 | GH097 | GPS 9th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 98 | P4AW63~W67B7~B0 | GH098 | GPS 9th tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 99 | P4AW68~W72B39~B34 | GH099 | GPS 10th tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 100 | P4AW68~W72B33~B30 | GH100 | GPS 10th tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 101 | P4AW68~W72B29~B24 | GH101 | GPS 10th tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 102 | P4AW68~W72B23~B18 | GH102 | GPS 10th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 103 | P4AW68~W72B17~B12 | GH103 | GPS 10th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 104 | P4AW68~W72B11~B10 | GH104 | GPS 10th tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 105 | P4AW68~W72B9 | GH105 | GPS 10th tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 106 | P4AW68~W72B8 | GH106 | GPS 10th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 107 | P4AW68~W72B7~B0 | GH107 | GPS 10th tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 108 | P4AW73~W77B39~B34 | GH108 | GPS 11th tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 109 | P4AW73~W77B33~B30 | GH109 | GPS 11th tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 110 | P4AW73~W77B29~B24 | GH110 | GPS 11th tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 111 | P4AW73~W77B23~B18 | GH111 | GPS 11th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 112 | P4AW73~W77B17~B12 | GH112 | GPS 11th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 113 | P4AW73~W77B11~B10 | GH113 | GPS 11th tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 114 | P4AW73~W77B9 | GH114 | GPS 11th tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 115 | P4AW73~W77B8 | GH115 | GPS 11th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 116 | P4AW73~W77B7~B0 | GH116 | GPS 11th tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 117 | P4AW78~W82B39~B34 | GH117 | GPS 12th tracking channel tracking asterisk | %d | Func0001 | / | unsigned int |  |  | / |  |
| 118 | P4AW78~W82B33~B30 | GH118 | GPS 12th tracking channel L1CA code tracking parameters | %d | Func0001 | / | unsigned int |  |  | / |  |
| 119 | P4AW78~W82B29~B24 | GH119 | GPS 12th tracking channel L1CA code CNR | %d | Func0001 | / | unsigned int |  |  | / |  |
| 120 | P4AW78~W82B23~B18 | GH120 | GPS 12th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 121 | P4AW78~W82B17~B12 | GH121 | GPS 12th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 122 | P4AW78~W82B11~B10 | GH122 | GPS 12th tracking channel L1CA tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 123 | P4AW78~W82B9 | GH123 | GPS 12th tracking channel L1P tracking status | %d | Func0001 | / | unsigned int |  |  | / |  |
| 124 | P4AW78~W82B8 | GH124 | GPS 12th tracking channel reservation | %d | Func0001 | / | unsigned int |  |  | / |  |
| 125 | P4AW78~W82B7~B0 | GH125 | GPS 12th tracking channel L1CA code channel rejection reason | %d | Func0001 | / | unsigned int |  |  | / |  |
| 126 | P4AW83~W86 | GH126 | Calculating clock correction | %d | Func0002 | / | complement | m | [-2147483648,2147483647] | / |  |
| 127 | P4AW87~W90 | GH127 | Calculating clock drift | %d | Func0002 | / | complement | m | [-2147483648,2147483647] | / |  |
| 128 | P4AW91 | GH128 | Reserved | %d | Func0001 | / | unsigned int |  |  | / |  |
| 129 | P4AW92B7~B4 | GH129 | A bus last initialization reason | %d | Func0001 | / | unsigned int |  |  | 0:initialization;1:not receiving bus data for 4s;2:data overflow;3:register error in CAN bus interrupt;4:not receiving data accord with protocol for 60 seconds;0x5:resetting by command;0x6:bus A state false for 200 seconds |  |
| 130 | P4AW92B3~B0 | GH130 | A bus initialization count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 131 | P4AW93B7~B4 | GH131 | B bus last initialization reason | %d | Func0001 | / | unsigned int |  |  | 0:initialization;1:not receiving bus data for 4s;2:data overflow;3:register error in CAN bus interrupt;4:not receiving data accord with protocol for 60 seconds;0x5:resetting by command;0x6:bus B state false for 200 seconds |  |
| 132 | P4AW93B3~B0 | GH132 | B bus initialization count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 133 | P4AW94~W95B15~B8 | GH133 | A bus status register | %d | Func0001 | / | unsigned int |  |  | / |  |
| 134 | P4AW94~W95B7~B0 | GH134 | B bus status register | %d | Func0001 | / | unsigned int |  |  | / |  |
| 135 | P4AW96B7~B4 | GH135 | Slave core resetting count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 136 | P4AW96B3~B0 | GH136 | Slave core resetting reason | %d | Func0001 | / | unsigned int |  |  | 1:resetting by command;3:mode switch;5:non-positioning for 10 minutes |  |
| 137 | P4AW97B7~B4 | GH137 | Latest receive indirect command status | %d | Func0001 | / | unsigned int |  |  | 0:initialization;1:upload permission or positioning software resetting;2:upload over;3:upload startup;4:sending upload data to salve core |  |
| 138 | P4AW97B3~B0 | GH138 | Receive indirect instruction count | %d | Func0001 | / | unsigned int |  |  | / |  |
| 139 | P4AW98~W101B31~B17 | GH139 | Program upload - incremental data length | %d | Func0001 | / | unsigned int |  |  | / |  |
| 140 | P4AW98~W101B16~B9 | GH140 | Program version number / check word | %d | Func0001 | / | unsigned int |  |  | / |  |
| 141 | P4AW98~W101B8~B7 | GH141 | Program upload command | %s | Func0001 | / | unsigned int |  |  | 0:initialization;1:program upload permission;2:program upload over;3:program upload startup |  |
| 142 | P4AW98~W101B6~B4 | GH142 | Program upload - check return value | %s | Func0001 | / | unsigned int |  |  | / |  |
| 143 | P4AW98~W101B3~B1 | GH143 | Program upload annotation | %s | Func0001 | / | unsigned int |  |  | / |  |
| 144 | P4AW98~W101B0 | GH144 | Boot guidance area | %s | Func0001 | / | unsigned int |  |  | / |  |
| 145 | P4AW102B7~B6 | GH145 | Upload parameter type | %s | Func0001 | / | unsigned int |  |  | 0:initialization;1:alterable parameter;2:EOP parameter;3:type false |  |
| 146 | P4AW102B5~B4 | GH146 | Upload parameter writes the FLASH identifier | %s | Func0001 | / | unsigned int |  |  | 0:initialization;1:write-in successful;2:write-in failed;3:check false |  |
| 147 | P4AW102B3~B2 | GH147 | Orbital parameter source | %s | Func0001 | / | unsigned int |  |  | 0:initialization;1:flash;2:default |  |
| 148 | P4AW102B1~B0 | GH148 | EOP parameter source | %s | Func0001 | / | unsigned int |  |  | 0:initialization;1:flash;2:default |  |
| 149 | P4AW103 | GH149 | Slave core SRAM monitoring error count | %d | Func0001 | / | unsigned int |  | [0,255] | / |  |
| 150 | P4AW104 | GH150 | Primary core SRAM monitoring error count | %d | Func0001 | / | unsigned int |  | [0,255] | / |  |
| 151 | P4AW105~W108 | GH151 | Orbit determation UTC Accumulated Seconds | %d | Func0001 | / | unsigned int |  |  | / |  |
| 152 | P4AW109B7~B4 | GH152 | Single point positioning using stars | %d | Func0001 | / | unsigned int |  |  | / |  |
| 153 | P4AW109B3~B0 | GH153 | Filtering using stars | %d | Func0001 | / | unsigned int |  |  | / |  |
| 154 | P4AW110 | GH154 | Orbit determation parameter PDOP | %d | Func0002 | / | complement |  |  | / |  |
| 155 | P4AW111 | GH155 | Orbit determation parameter position residual | %d | Func0002 | / | complement |  |  | / |  |
| 156 | P4AW112 | GH156 | Orbit determation parameter speed residual | %d | Func0002 | / | complement |  |  | / |  |
| 157 | P4AW113 | GH157 | Orbit determation convergence value | %d | Func0002 | / | complement |  |  | / |  |
| 158 | P4AW114 | GH158 | Standard deviation of orbital prior information | %d | Func0002 | / | complement |  |  | / |  |
| 159 | P4AW115 | GH159 | Measure the updated standard deviation | %d | Func0002 | / | complement |  |  | / |  |
| 160 | P4AW116~W127 | GH160 | Orbital removal reason | %d | Func0001 | / | unsigned int |  |  | / |  |

GPS_Orbit_slow_TM(6AH)

Pack ID=6AH；Pack length=86（Bytes=87）。

Table 8-31GPS_Orbit_slow_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P6AW0~W3 | GA001 | Absolute positioning time | %d | Func0001 | / | unsigned int | s | [0,4294967295] | / | UTC time |
| 2 | P6AW4~W7 | GA002 | Satellite position X1 | %d | Func0007 | 0.1 | complement | m | [-8000,8000] | / | WGS-84 coordinates system |
| 3 | P6AW8~W11 | GA003 | Satellite position Y1 | %d | Func0007 | 0.1 | complement | m | [-8000,8000] | / | WGS-84 coordinates system |
| 4 | P6AW12~W15 | GA004 | Satellite position Z1 | %d | Func0007 | 0.1 | complement | m | [-8000,8000] | / | WGS-84 coordinates system |
| 5 | P6AW16~W19 | GA005 | Satellite speed VX1 | %d | Func0007 | 0.01 | complement | m/s | [-8,8] | / | WGS-84 coordinates system |
| 6 | P6AW20~W23 | GA006 | Satellite speed VY1 | %d | Func0007 | 0.01 | complement | m/s | [-8,8] | / | WGS-84 coordinates system |
| 7 | P6AW24~W27 | GA007 | Satellite speed VZ1 | %d | Func0007 | 0.01 | complement | m/s | [-8,8] | / | WGS-84 coordinates system |
| 8 | P6AW28 | GA008 | Absolute positioning data availability | %X | Func0001 | / | unsigned int |  |  | 0xAA:available;0x55:unavailable |  |
| 9 | P6AW29~W30B15 | GA009 | Reserved | %s | Func0001 | / | unsigned int |  |  | / |  |
| 10 | P6AW29~W30B14~B12 | GA010 | Orbit determation filtering state | %s | Func0001 | / | unsigned int |  |  | 3:orbit determination filter data;6:extrapolating data |  |
| 11 | P6AW29~W30B11~B8 | GA011 | Positioning available stars | %d | Func0001 | / | unsigned int |  |  | / |  |
| 12 | P6AW29~W30B7~B6 | GA012 | Operating mode | %s | Func0001 | / | unsigned int |  |  | 1:GPS mode |  |
| 13 | P6AW29~W30B5~B4 | GA013 | Orbit determationl output enable flag | %s | Func0001 | / | unsigned int |  |  | 3:output permission;2:output forbidden |  |
| 14 | P6AW29~W30B3~B2 | GA014 | Positioning solution state | %s | Func0001 | / | unsigned int |  |  | 0:not resolving;1:GPS resolving |  |
| 15 | P6AW29~W30B1~B0 | GA015 | Orbit determination solution state | %s | Func0001 | / | unsigned int |  |  | 0:not resolving;1:GPS resolving |  |
| 16 | P6AW31~W34 | GA016 | Orbit determation time | %d | Func0001 | / | unsigned int | s | [0,4294967295] | / | UTC time |
| 17 | P6AW35~W38 | GA017 | Satellite position X2 | %d | Func0007 | 0.1 | complement | m | [-8000,8000] | / | WGS-84 coordinates system |
| 18 | P6AW39~W42 | GA018 | Satellite position Y2 | %d | Func0007 | 0.1 | complement | m | [-8000,8000] | / | WGS-84 coordinates system |
| 19 | P6AW43~W46 | GA019 | Satellite position Z2 | %d | Func0007 | 0.1 | complement | m | [-8000,8000] | / | WGS-84 coordinates system |
| 20 | P6AW47~W50 | GA020 | Satellite speed VX2 | %d | Func0007 | 0.01 | complement | m/s | [-8,8] | / | WGS-84 coordinates system |
| 21 | P6AW51~W54 | GA021 | Satellite speed VY2 | %d | Func0007 | 0.01 | complement | m/s | [-8,8] | / | WGS-84 coordinates system |
| 22 | P6AW55~W58 | GA022 | Satellite speed VZ2 | %d | Func0007 | 0.01 | complement | m/s | [-8,8] | / | WGS-84 coordinates system |
| 23 | P6AW59~W62 | GA023 | Orbital semi-major axis (a) | %d | Func0007 | 0.1 | unsigned int | m | [6000000,8000000] | / | WGS-84 coordinates system |
| 24 | P6AW63~W66 | GA024 | Eccentricity (e) | %d | Func0007 | 2.24E-11 | unsigned int |  | [0,1] | / | WGS-84 coordinates system |
| 25 | P6AW67~W70 | GA025 | Inclination (i) | %d | Func0007 | 2.93E-09 | unsigned int | rad | [0,3.1416] | / | WGS-84 coordinates system |
| 26 | P6AW71~W74 | GA026 | Right ascension of ascending node (Ω) | %d | Func0007 | 2.93E-09 | unsigned int | rad | [0,6.2832] | / | WGS-84 coordinates system |
| 27 | P6AW75~W78 | GA027 | Argument of perigee (ω) | %d | Func0007 | 2.93E-09 | unsigned int | rad | [0,6.2832] | / | WGS-84 coordinates system |
| 28 | P6AW79~W82 | GA028 | Mean anomaly (M) | %d | Func0007 | 2.93E-09 | unsigned int | rad | [0,6.2832] | / | WGS-84 coordinates system |
| 29 | P6AW83~W86 | GA029 | Orbit determination module running mark | %d | Func0001 | / | unsigned int |  |  | / |  |

MC

MC_fast_TM(2CH)

Pack ID=2CH；Pack length=28（Bytes=29）。

Table 8-32MC_fast_TMTable

| No | Data Bytes/Bits | Parameter Number | Parameter Name | Display Mode | Processing Function | Parameter Coefficient | Telemetry Type | Unit | Work Range | Processing | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P2CW0~W3 | AS001 | MC Received_Latest_TC | %u | Func0001 | / | unsigned int | / |  | / |  |
| 2 | P2CW4 | AS002 | MC Clock status | %u | Func0001 | / | unsigned int | / | [0,1] | / |  |
| 3 | P2CW5 | AS003 | MC Parameter1（right counter） | %u | Func0001 | / | unsigned int | / | [0,255] | / |  |
| 4 | P2CW6 | AS004 | MC Parameter2（wrong counter） | %u | Func0001 | / | unsigned int | / | [0,255] | / |  |
| 5 | P2CW7 | AS005 | MC Parameter3（soft check） | %u | Func0001 | / | unsigned int | / |  | / |  |
| 6 | P2CW7B7 | AS024 | MC Can bus | %s | Func0001 | / | unsigned int | / |  | 1:CANB;0:CANA |  |
| 7 | P2CW7B6~B0 | AS025 | Reserved TM Parameter | %u | Func0001 | / | unsigned int | / |  | / |  |
| 8 | P2CW8 | AS006 | Reserved TM Parameter | %u | Func0001 | / | unsigned int | / |  | / |  |
| 9 | P2CW9 | AS007 | MC image-output status | %u | Func0001 | / | unsigned int | / | [0,255] | / |  |
| 10 | P2CW11~W12 | AS009 | B1 Gain_Code_SET | %X | Func0001 | / | unsigned int | / |  | / |  |
| 11 | P2CW10 | AS008 | Received_Wrong_TC_COUNTER | %u | Func0001 | / | unsigned int | / | [0,255] | / |  |
| 12 | P2CW13~W14 | AS010 | B2 Gain_Code_SET | %X | Func0001 | / | unsigned int | / |  | / |  |
| 13 | P2CW15~W16 | AS011 | B3 Gain_Code_SET | %X | Func0001 | / | unsigned int | / |  | / |  |
| 14 | P2CW17~W18 | AS012 | B4 Gain_Code_SET | %X | Func0001 | / | unsigned int | / |  | / |  |
| 15 | P2CW20 | AS014 | MC Integration_Time_STATUS | %X | Func0001 | / | unsigned int | / | [1,10] | / |  |
| 16 | P2CW19 | AS013 | Algorithm_ENABLE | %s | Func0001 | / | unsigned int | / |  | / |  |
| 17 | P2CW21 | AS015 | MC B1 Exposure time _STATUS | %X | Func0001 | / | unsigned int | / | [1,11] | / |  |
| 18 | P2CW22 | AS016 | MC B2 Exposure time _STATUS | %X | Func0001 | / | unsigned int | / | [1,11] | / |  |
| 19 | P2CW23 | AS017 | MC B3 Exposure time _STATUS | %X | Func0001 | / | unsigned int | / | [1,11] | / |  |
| 20 | P2CW24 | AS018 | MC B4 Exposure time _STATUS | %X | Func0001 | / | unsigned int | / | [1,11] | / |  |
| 21 | P2CW25 | AS019 | MC reset CANA counter | %u | Func0001 | / | unsigned int | / | [0,255] | / |  |
| 22 | P2CW26 | AS020 | MC reset CANB counter | %u | Func0001 | / | unsigned int | / | [0,255] | / |  |
| 23 | P2CW27~W28 | AS021 | Reserved TM Parameter | %u | Func0001 | / | unsigned int | / |  | / |  |

- Appendix Formula Description

  - Direct reading

Unsigned Integer：Func0001；

Complement Integer：Func0002；

Single-precision floating-point number：Func0003；

Double-precision floating-point number：Func0004；

  - Additive

Code Name：Func0005

  - Subtraction

Code Name：Func0006

  - Multiplication

Code Name：Func0007

  - Division

Code Name：Func0008

  - Remainder

Code Name：Func0009

  - Linear processing

Function：

tm = k1 * raw data + b

Code Name：

Func0010

  - Temperature Processing

    - Temperature Processing I

Function：

R=Rt,

Code Name：

Func0014

    - Temperature Processing II

Function：

R=Rt,

Code Name：

Func0015

  - Custom Expression

Code Name：

FuncExpr