Key TMs

| Sn | Subsystem | TM Name | TM Number | Expected Value |
| --- | --- | --- | --- | --- |
|  | TT&C | Transponder 1 transmitter on/off status | TA001 | 1:On |
|  |  | Transponder 2 transmitter on/off status | TB001p | 0:OFF |
|  |  | Transponder 1 transponder power | TA016 | 3.1~3.3 |
|  |  | Transponder 1 carrier lock status | TA002 | 1:Lock |
|  |  | Transponder 2 carrier lock status | TB002 | 1:Lock |
|  |  | Transponder 1 +3.5V power | TA020 | 3~4V |
|  |  | Transponder 2 +3.5V power | TB020 | 3~4V |
|  |  | Transponder 1 temperature | TA021 | -15 ~ +55°C |
|  |  | Transponder 2 temperature | TB021 | -15 ~ +55°C |
|  | PSS | 28V bus voltage | NS001 | 26~29V |
|  |  | +12V voltage | NS002 | 11.7~12.5V |
|  |  | +5V voltage | NS003 | 4.7~5.7V |
|  |  | -12V voltage | NS004 | -13~-11V |
|  |  | Batt_voltage | NS005 | 26~29V |
|  |  | Bus Load current | NS006 | 1.5～7A ( only platform working 1.5~3A; In record mode add about 1A ; In replay mode add about2A ; In real-time mode add about 3A) |
|  |  | Solar array current | NS007 | 0~7A In lighting time 5.9~6.6A( when payload is not working), In umbra ~0A |
|  |  | Battery Charging current | NS008 | 0～6A |
|  |  | Battery Discharge current | NS009 | 0～7A |
|  |  | Batt_temperature 1 | NS010 | 17℃~20℃ |
|  |  | Overdischarge flag | NS019 | 0: not over-discharged |
|  |  | Energy security status | NH007 | 00：normal |
|  |  | Batt_Current_CAPACITY | NH004 | 6~10Ah |
|  |  | PCU_Reset_COUNTER | NH001 | 0 |
|  |  | Battery automatically switch on FLAG | NH014 | 0:not reconnected |
|  |  | 08_-Z_fixed_wing_TEMPERATURE | RS044 | -80℃~+90℃ |
|  |  | 09_-Y_wing_TEMPERATURE | RS045 | -80℃~+90℃ |
|  | OBDH | TC_COUNTER | XS001 | 0-0xFFFF(Add 1 when an Indirect TC or a Data block is received) |
|  |  | TTC_Check_Error_Safety_Mode_COUNTER | XH164 | Don’t Increase |
|  |  | TTD_Check_Error_Safety_Mode_COUNTER | XH165 | Don’t Increase |
|  |  | Energy_protect_Safety_Mode(_triggered_by_PCU)_COUNTER | XH166 | Don’t Increase |
|  |  | AOCS_Global_Attitude_Acquisition_Safety_Mode_COUNTER | XH167 | Don’t Increase |
|  |  | AOCS_Stop_Control_Safety_ModeCOUNTER | XH168 | Don’t Increase |
|  |  | Energy_warning_Safety_Mode_1COUNTER | XH169 | Don’t Increase |
|  |  | Energy_warning_Safety_Mode_2_COUNTER | XH170 | Don’t Increase |
|  |  | Energy_protect_Safety_Mode(_triggered_by_TCU)_COUNTER | XH171 | Don’t Increase |
|  |  | CCD_temperature_Safety_Mode_COUNTER | XH172 | Don’t Increase |
|  |  | Phased_array_antenna_temperature_Safety_Mode_COUNTER | XH173 | Don’t Increase |
|  |  | Phased_array_antenna_working_timeout_Safety_Mode_COUNTER | XH174 | Don’t Increase |
|  |  | OBC_Data_Recover_Failure | XS151 | 0: Not_happened |
|  |  | TTC_Task_Permission_Status | XS027 | 1: Enabled |
|  |  | TTC_Insert_Status | XS038 | 102: Normal;153: Init_Finished |
|  |  | TTD_Insert_Status | XS042 | 102: Normal;153: Init_Finished |
|  |  | On-board_TIME_(s) | XW00001 | Add 1 per second |
|  |  | Thermal_Control_Enable | RS008 | 1: Enabled; |
|  | AOCS | Satellite time | TMK102 | Increase normally |
|  |  | Work mode word | TMK103 | Depends on the solar/shadow zone and missions carried on: 0x30: SPM(if there is no mission in the solar zone) 0x43: MAM-towards to SPM(if just into the solar zone, or the mission is just finished in the solar zone) 0x45: MAM-towards to EPM(if just into the shadow zone, or just received the Run-EPM-command in the data transmission mission when in the solar zone) 0x46: MAM-towards to AMM(if just received the Bias-flight-command in the imaging mission) 0x50: EPM(if there is no mission in the shadow zone, or in the data transmission mission) 0x60: AMM(if the imaging mission is taking place) |
|  |  | Roll angle estimate | TMK104 | -0.1° ~ +0.1°(when in EPM); Expected angle-0.1° ~ Expected angle+0.1°(when in AMM); -180° ~ +180°(when in other work modes); |
|  |  | Pitch angle estimate | TMK105 | -0.1° ~ +0.1°(when in EPM and AMM); -180° ~ +180°(when in other work modes); |
|  |  | Yaw angle estimate | TMK106 | -4° ~ +4°(when in EPM and AMM); -180° ~ +180°(when in other work modes); |
|  |  | Roll angular velocity estimate | TMK107 | -0.01°/s ~ +0.01°/s(when in EPM and AMM); -1°/s ~ +1°/s(when in MAM); No need to be noticed(when in SPM); |
|  |  | Pitch angular velocity estimate | TMK108 | -0.01°/s ~ +0.01°/s(when in EPM and AMM); -1°/s ~ +1°/s(when in MAM); No need to be noticed(when in SPM); |
|  |  | Yaw angular velocity estimate | TMK109 | -0.1°/s ~ +0.1°/s(when in EPM and AMM); -1°/s ~ +1°/s(when in MAM); No need to be noticed(when in SPM); |
|  |  | ASS-alpha angle data | TMK120_3 | -5° ~ +5°(when in SPM); No need to be noticed(when in other work modes); |
|  |  | ASS-beta angle data | TMK121_3 | -5° ~ +5°(when in SPM); No need to be noticed(when in other work modes); |
|  |  | DSS-alpha angle data | TMK122_3 | -5° ~ +5°(when in SPM); No need to be noticed(when in other work modes); |
|  |  | DSS-beta angle data | TMK123_3 | -5° ~ +5°(when in SPM); No need to be noticed(when in other work modes); |
|  |  | FOG X/Y/Z/S health state | TMK268_1~ TMK268_4 | 0x01: healthy |
|  |  | Sun sensor health state | TMK269_1 | 0x01: Only ASS is healthy |
|  |  | MW X/Y/Z/S introduction state | TMK271_1~ TMK271_4 | 0x01: Introduced |
|  |  | Attitude determination flag | TMK280_6 | 0x01: Star sensor(STS) and Gyro |
|  |  | SunZoneFlag | TMK280_3 | 0x01:Solar Zone; 0x00:Shadow Zone |
|  |  | System-reset count | TMK170_12 | 0 |