ERSS-1 spacecraft system mission analysis and design integration overview

# Abstract

In April 2018, the Ministry of Innovation and Technology (MInT) formerly Ministry of Science and Technology (MOST) signed an agreement with the national development and reform commission of china to design and built the 1st Ethiopian earth observation satellite. The Ethiopian Remote Sensing Satellite (ETRSS-1) of 65kg was designed, developed and manufactured by DFH Satellite Company Ltd one of the subsidiaries of China Academy of Space Technology (CAST). The satellite was launched into space in 2019 from Taiyuan launch center with a mission of collecting an earth imagery data to address the Ethiopian climate changes.

Currently, the satellite is under operation by Ethiopian Space Science and Technology Institute (ESSTI) from sun synchronous orbit at an altitude of 628km and providing the necessary data to satisfy the application requirements of the fields such as agriculture monitoring, environmental and disaster monitoring, territory investigation, urban planning, mapping, forest and vegetation observation, etc. through ground control station facility located on Entoto. This study is aimed to introduce an overview of synthetic ETRSS-1 spacecraft system design, mission status, payload complement and its ground segment facility. Also, the major subsystems involved in the development of the satellite and its integration system will be discussed.

# ETRSS-1 spacecraft system overview

The ETRSS-1 (Ethiopian Remote Sensing satellite) is Ethiopia’s first satellite launched on the December 2019 from Taiyuan launch center with a mission of collecting multispectral optical remote sensing data in the fields of agriculture, forestry, water conservancy, disaster prevention and mitigation in order to address the Ethiopian climate changes and weather-related phenomena [1]. The satellite was placed on sun-synchronous orbit at an altitude of 628.61km and collects imagery data at a ground sampling distance of 13.45meters within a revisit period of 4 days [2].

A 65kg of multi-spectral Ethiopian remote sensing satellite was developed and built at the DFH Satellite Company Ltd, CAST consisting of two distinct modules: the spacecraft base module (platform) and the payload (instrument module). The platform module of ETRSS-1 microsatellite comprehend the attitude and orbit control, onboard data handling, power supply, telemetry and telecommand, thermal control and structural subsystem to support the payload. The payload module includes a multi-spectral camera and data transmission system providing an imagery data.

Figure 1: Illustration of ETRSS-1 configuration and layout (image credit: CAST)

General ETRSS-1 technical parameter

Table 1: ETRSS-1 technical parameter

| Item | Technical parameters |
| --- | --- |
| Orbit | SSO Altitude: 628km |
| Mass | 65kg |
| AOCS | 3-axis stabilized, zero-momentum control Pointing Accuracy：0.1° (3σ, 3-axis in normal mode) Pointing Stability：0.01°/s (3σ, 3-axis) Attitude Determination Accuracy：0.05° (3σ, 3-axis) Attitude Maneuver： ±35° (along roll axis) Maneuver Capacity： 30/70s |
| TT and C | USB + GPS |
| PSS | Output power of Solar Array more than 150W (EOL) Li-ion Battery Capacity：20Ah |
| MSC | Type push-broom imaging Spectral: B1/Blue：0.45m ～ 0.52m B2/Green：0.52m ～ 0.59m B3/Red：0.63m ～ 0.69m B4/NIR：0.77m ～ 0.89m GSD: ≤15m@628km Swath: ≥60km@628km |
| DTS | Frequency Band: X band Storage Capacity: 128Gbits Downlink Data Rate: 100Mbps |

Subsystem details

# ETRSS-1 Launch and its mission status

Orbit design

# ETRSS-1 Payload complements

# ETRRS-1 Ground system

# Conclusion

# Reference

[1]	Gunter’s space page, “ETRSS 1,” https://space.skyrocket.de/doc_sdat/etrss-1.htm, 2019.

[2]	S. B. Tessema, “First Ethiopian Remote Sensing Satellite (ETRSS-1): Mission information and overview,” pp. 1–15, 2020, doi: https://doi.org/10.1002/essoar.10502532.1.