KARIOS CUBESAT Verification & MBSE

3.1 Mission Verification process

This section presents a comprehensive verification process for the KARIOS CubeSat mission, from initial conceptual design to in-orbit commissioning, using life-cycle standards (ECSS-E-ST-10) and a structured system engineering approach tailored to this mission. The main purpose is to demonstrate that the deliverable KARIOS spacecraft achieves all of its mission objectives, hence validating the design and verifying that the overall system can meet the mission requirements. The governing standards include ESA space engineering, environmental testing, in-orbit demonstration (IOD), CubeSat design parameters for structural/dimensional compliance, along with the verification methods (see Table 3) to support KARIOS CubeSat missions.

| Approach/method | Description | Primary use in KARIOS |
| --- | --- | --- |
| Testing (T) | Physical measurement on hardware | Structure, thermal, and EMC |
| Analysis (A) | Analytical/Computational methods | Orbital mechanics, stress, thermal model |
| Review of design (RoD) | Inspection of design documentation | Interface control, heritage component reuse |
| Inspection (I) | Visual/dimensional check | Dimensional compliance |

The KARIOS CubeSat employs a two-model strategy adapted from the ECSS prototype flight model (PFM) approach for small satellite mission verifications limited budgets. The engineering model (EM) is an electrically and mechanically representative model that is used for early functional testing and interface verification using flight-representative PCBs and structural panels but does not necessarily include flight grade components. The EM testing involves SW functional testing, electrical interface compatibility, and preliminary FMEA validation. The flight model (FM) is an actual flight unit subjected to acceptance-level environmental testing (shorter duration than qualification levels).

3.2. Model-based system engineering

Bus verification process

KARIOS CubeSat Orbital Dynamics and Coverage Analysis

Model based system engineering

Preliminary Design Optimization Strategy

- Analysis of Interdependencies Among System Components

  - Utilization of SysML within Model-Based Systems Engineering (MBSE)

    - Design and Analysis of Interdependencies

    - Across System Architecture, Mechanical Structures, and Electronics

  - Developed by the International Council on Systems Engineering (INCOSE)

    - Space Systems Working Group (SSWG)

Pre-launch orbital simulation

| Orbit regime | SSO, approximately 550 km altitude |
| --- | --- |
| Inclination | 97.7444° |
| RAAN | 108.1576° |
| Eccentricity | 0.0006955 |
| Argument of Perigee | 79.2781° |
| Mean Anomaly | 280.9219° |
| Orbital Period | 96.75 minutes |
| Mean Motion | 14.88 revolutions/day |